3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240638

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3566 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 11.4.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In step 29a there can be multiple PDU sessions triggered, depending on the UE configuration, but there will be at least two of them regardless. The function f_NR5GC_PDUSessionEstablishment by default waits 10s for an eventual IMS signalling for every session. If UE sends all requests at once there will be at least 20s delay before sending Accept for the 2nd session which exceeds T3580 and so UE may repeat the request not accepted within T3580 which will lead to test failure. In order to prevent T3580 expiry it is necessary to disable the 10s waiting timer for every PDU session in this step. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “false” as an input parameter into f_NR5GC_PDUSessionEstablishment that controls the “WaitIfIMS” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case especially when multiple PDU/PDNs are configured on the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_4_11_NR5GC_TestBody |
| --- | --- |
| Reason for change | In step 29a there can be multiple PDU sessions triggered, depending on the UE configuration, but there will be at least two of them regardless. The function f_NR5GC_PDUSessionEstablishment by default waits 10s for an eventual IMS signalling for every session. If UE sends all requests at once there will be at least 20s delay before sending Accept for the 2nd session which exceeds T3580 and so UE may repeat the request not accepted within T3580 which will lead to test failure. In order to prevent T3580 expiry it is necessary to disable the 10s waiting timer for every PDU session in this step. |
| Summary of change | Added “false” as an input parameter into f_NR5GC_PDUSessionEstablishment that controls the “WaitIfIMS” |
| TTCN module | Emergency_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | Rejected. Alternative solution implemented under R5s240641 |

Before Change

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26, -, cr_PDU_SessionStatusAny ifpresent, // @sic R5-231417 sic@ SUPPORTED); // @sic R5-237320 sic@ // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, -, v_PduSessionInfoList); // @sic R5s210149, R5s220883, R5s221342 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); } } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@

After Change

function fl_TC_11_4_11_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 18-28" siclog@ f_NR5GC_IRAT_MobilityRegistration (nr_Cell1, v_NAS_SecurityParams, omit, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), f_LastVisited_TAI(nr_Cell1), -, -, noRrcConnectionRelease, -, cds_NG_NetworkFeatureSupport_NoN26, -, cr_PDU_SessionStatusAny ifpresent, // @sic R5-231417 sic@ SUPPORTED); // @sic R5-237320 sic@ // Tell IMS PTCs the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // This function stores all received PDN Connectivity msgs, so just expect one more... //@siclog "Step 29a1" siclog@ f_NR5GC_PDUSessionEstablishment (nr_Cell1, (pc_noOf_PDUsSameConnection+1), tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, -, -, -, v_PduSessionInfoList, false); // @sic R5s210149, R5s220883, R5s221342 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { if (match (v_PduSessionInfoList[i].PdnIndex, f_GetMultiPdnIndex(Emergency_PDN))) { // Keep note of this for later checks v_EmergencySessionId := int2oct(v_PduSessionInfoList[i].PDU_SessionId, 1); } } // ...then check one of received msgs is for the emergency bearer // @siclog "Step 1 Parallel Behaviour" siclog@ for (i := 1; i <= f_NR5GC_MsgInDefault_GetNumOfNASMessageReq(); i := i+1){ // @sic R5s220174 sic@ v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i); if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('100'B), omit, omit)) { // Emergency bearer v_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId; if (match (v_EmergencySessionId, v_SessionId.sessionId)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1"); } // transfer of existing bearer } else if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -,-, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('010'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Request Type neither existing nor emergency PDU session"); } } // @siclog "Step 30" siclog@