3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240648

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3574 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC MAC UL SPS test case 7.1.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case is configured for MAC test mode with DL/UL transparent mode enabled. MAC doesn't decode the subheader. RLC is configured in transparent mode so RLC layer will not do any header manipulation. Now in preamble during radio bearer configuration, SS is configuring PDCP in RB terminating mode which configures the PDCP in non-transparent mode. This leads PDCP to decode the RLC TMD data received. Now since the very first bit of PDCP PDU is D/C (data / control flag) which is decoded as control PDU (D/C = 0) due to this PDCP discards this PDU considering it as PSR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SS needs to configure the PDCP in transparent mode so the header manipulation by PDCP should not be performed and uplink data is directly received at RLC SAP port. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_7_1_1_6_2_ENDC_NR () |
| --- | --- |
| Reason for change | The test case is configured for MAC test mode with DL/UL transparent mode enabled. MAC doesn't decode the subheader. RLC is configured in transparent mode so RLC layer will not do any header manipulation. Now in preamble during radio bearer configuration, SS is configuring PDCP in RB terminating mode which configures the PDCP in non-transparent mode. This leads PDCP to decode the RLC TMD data received. Now since the very first bit of PDCP PDU is D/C (data / control flag) which is decoded as control PDU (D/C = 0) due to this PDCP discards this PDU considering it as PSR. |
| Summary of change | SS needs to configure the PDCP in transparent mode so the header manipulation by PDCP should not be performed and uplink data is directly received at RLC SAP port. |
| TTCN module | IWD_24wk37/IWD_24wk37_5G/ENDC_IWD_24wk37/ENDC/7_1_1/MAC_SPS_ENDC_NR.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently similar to other 7.1.1.6.X test cases. |

Before Change:

function f_TC_7_1_1_6_2_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of UL grant / configured grant Type 1 const integer const_UL_TBS1 := 848; //@sic R5s230285 sic@ var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template (value) SpCellConfig v_SpCellConfig ; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var template (value) ConfiguredGrantConfig v_ConfiguredGrantConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation1 ; var template (value) BWP.locationAndBandwidth v_LocationAndBandwidth; var integer v_BwpSize ; var integer v_RIV; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s230285 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var default v_MyDefaultVar := null; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId)}; //@sic R5s230285 sic@ v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, -, -, f_NR_DL_UM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)),-,true_); f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList, v_RLC_BearerConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 1-2 siclog@

After Change:

function f_TC_7_1_1_6_2_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of UL grant / configured grant Type 1 const integer const_UL_TBS1 := 848; //@sic R5s230285 sic@ var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList ; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; var template (value) SpCellConfig v_SpCellConfig ; var NR_DMRS_UL_AdditionalPosition_Type v_DMRS_UL_AdditionalPosition; var SubcarrierSpacing v_SubcarrierSpacing ; var ConfigGrant_Period_Type v_ConfigGrant_Period; var template (value) ConfiguredGrantConfig v_ConfiguredGrantConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation1 ; var template (value) BWP.locationAndBandwidth v_LocationAndBandwidth; var integer v_BwpSize ; var integer v_RIV; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s230285 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var default v_MyDefaultVar := null; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId, -, cs_NR_PDCP_RbConfig_TransparentMode)}; //@sic R5s230285 sic@ v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, -, -, f_NR_DL_UM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)),-,true_); f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList, v_RLC_BearerConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 1-2 siclog@

MCC TF160 Implementation

function f_TC_7_1_1_6_2_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of UL grant / configured grant Type 1 const integer const_UL_TBS1 := 848; //@sic R5s230285 sic@ var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ;//@sic R5s240648 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; … //v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId)}; … }