3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240660

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3583 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC DL SPS test case 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Step 16 is voided via R5-244723, however its implementation is still present in TTCN. - Since the time for sending CRC corrupted data at step 17 is fixed, so we expect NACK according to TTI of CRC corrupted data. - The current logic for allowing upto 3 NACKS (step 18) and one ACK is incorrect, it fails to meet the condition that only one ack is required at step 21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the step 16 to allocate DL SPS Grant. - TTI for receiving NACK is defined with ‘v_TimingUL_7’ instead of ‘Any Time’ - Modifed handling to accept 3 NACKs and only 1 ACK for step 18,21 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC, 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | - Step 16 is voided via R5-244723, however its implementation is still present in TTCN. - Since the time for sending CRC corrupted data at step 17 is fixed, so we expect NACK according to TTI of CRC corrupted data. - The current logic for allowing upto 3 NACKS (step 18) and one ACK is incorrect, it fails to meet the condition that only one ack is required at step 21. |
| Summary of change | - Removed the step 16 to allocate DL SPS Grant. - TTI for receiving NACK is defined with ‘v_TimingUL_7’ instead of ‘Any Time’ - Modifed handling to accept 3 NACKs and only 1 ACK for step 18,21 |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_2; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; //var SubFrameTiming_Type v_TimingUL_7; //var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingDL_Step11b; //@sic R5s240268 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_0_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing, -, cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('111'B)), cs_NR_RedundancyVersionList_SingleRV(0)); //@sic R5s237339 sic@ //@sic R5s240155 sic@ //@sic R5s240563 sic@ var integer v_RetransSchInt := 8; //suitable for FDD var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; timer t_Watchdog := 10.0; //Start default UL Grant WA# f_NR_ULGrantConfiguration_Start(nr_Cell1); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 //@sic R5s230300 sic@ if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230088 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // @siclog "Steps 1 - 2" siclog@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); t_Watchdog.start; // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 6-7" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 R5-240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); //v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); /* //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); */ //@siclog "Step 19-20" siclog@ //@sic R5s240563 sic@ for (i:=1; i< 4; i:=i+1) //Scheduled 3 retransmissions assuming worst case. { v_TimingDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, i * v_RetransSchInt); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL_2),v_N_PRB,v_I_MCS,true,-,-,v_RVList[i]); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_2), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); } //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,2*const_SPS_Period-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 R5s240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,50))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 10 11a3 or 11b2" siclog@ //@sic R5s230088 sic@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_4) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_5_CRNTI) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11a3 or 11b2"); } } // @siclog "Step 15" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_6) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); /* // @siclog "Step 18" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_NR( v_TimingUL_7) )); //f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); // @siclog "Step 21" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_8) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); */ //@sic R5s240563 sic@ // @siclog "Step 18 & 21" siclog@ alt { []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_Any )) { v_Ack_Count := v_Ack_Count +1; if ((v_Ack_Count + v_Nack_Count) < 3) { repeat; } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_Any )) { v_Nack_Count := v_Nack_Count + 1; if ((v_Ack_Count + v_Nack_Count) < 3) { repeat; } else if (v_Ack_Count == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21: UE sent HARQ ACK "); } } } // @siclog "Step 23" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_9) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // @siclog "Step 25 and 28" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_10) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_11) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); t_Watchdog.stop; //Start default UL Grant WA# f_NR_ULGrantConfiguration_Start(nr_Cell1); return (v_TimingDL); }

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_2; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; //var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var SubFrameTiming_Type v_TimingDL_Step11b; //@sic R5s240268 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cds_NR_DciFormat_1_0_Params_SetPDSCHarqIndicator(v_NR_PhysicalParameters.SubcarrierSpacing, -, cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('111'B)), cs_NR_RedundancyVersionList_SingleRV(0)); //@sic R5s237339 sic@ //@sic R5s240155 sic@ //@sic R5s240563 sic@ var integer v_RetransSchInt := 8; //suitable for FDD var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; var integer v_Ack_Count := 0; timer t_Watchdog := 10.0; //Start default UL Grant WA# f_NR_ULGrantConfiguration_Start(nr_Cell1); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 //@sic R5s230300 sic@ if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 480); v_TimingUL_1:= v_TimingDL; v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); //@sic R5s230088 sic@ select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // @siclog "Steps 1 - 2" siclog@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); t_Watchdog.start; // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (0, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 4" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 6-7" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 9" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 R5-240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; //f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); /* //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); */ //@siclog "Step 19-20" siclog@ //@sic R5s240563 sic@ for (i:=1; i< 4; i:=i+1) //Scheduled 3 retransmissions assuming worst case. { v_TimingDL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, i * v_RetransSchInt); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL_2),v_N_PRB,v_I_MCS,true,-,-,v_RVList[i]); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_2), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); } //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,2*const_SPS_Period-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 R5s240268 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,50))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 10 11a3 or 11b2" siclog@ //@sic R5s230088 sic@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_4) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_5_CRNTI) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11a3 or 11b2"); } } // @siclog "Step 15" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_6) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); /* // @siclog "Step 18" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_NR( v_TimingUL_7) )); //f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); // @siclog "Step 21" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_8) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); */ //@sic R5s240563 sic@ // @siclog "Step 18 & 21" siclog@ alt { [v_Nack_Count==0] SYSIND.receive( car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, nack, cr_TimingInfo_NR( v_TimingUL_7))) { v_Nack_Count := 1; repeat; } [v_Nack_Count<3] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)) { v_Nack_Count := v_Nack_Count +1; repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21: UE did not send HARQ ACK"); } } // @siclog "Step 23" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_9) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); // @siclog "Step 25 and 28" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_10) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); repeat; } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_11) )) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); t_Watchdog.stop; //Start default UL Grant WA# f_NR_ULGrantConfiguration_Start(nr_Cell1); }