3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240661

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0958 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS Re-Registration test case 6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE is provided with a second P-CSCF address by SS and subsequently IMS server for new P-CSCF address is started after step 11 and IP sockets for TCP listen requests are configured. Now UE needs to perform re-registration and once the process is done, SS release connection. However during release SS does not close the TCP connections used for SIP. This leads to non clearing of contexts at IPSEC engine and test case fails. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional argument waitAndClose is passed in function f_IMS_CC_ReleaseConnection() which shall eventually close the TCP connections if any for IMS SIP service during re-register events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | function fl_TC_6_3_ReRegistration () |
| --- | --- |
| Reason for change | UE is provided with a second P-CSCF address by SS and subsequently IMS server for new P-CSCF address is started after step 11 and IP sockets for TCP listen requests are configured. Now UE needs to perform re-registration and once the process is done, SS release connection. However during release SS does not close the TCP connections used for SIP. This leads to non clearing of contexts at IPSEC engine and test case fails. |
| Summary of change | Additional argument waitAndClose is passed in function f_IMS_CC_ReleaseConnection() which shall eventually close the TCP connections if any for IMS SIP service during re-register events. |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment | Accepted in principle: The issue seems to be that f_IMS_GlobalRelease has p_WaitForCNF := false per default which causes at the IP_PTC fl_IMS_Server_RemoveSecurityContexts not to wait for confirmation of TCP connections being closed and therefore fl_IMS_Server_CheckPendingSecurityContext_ReleaseIPsec does not call fl_IPsec_Release what is done later on in a_IMS_TCP_Close_CNF_Handler. f_IMS_GlobalRelease shall be called with p_WaitForCNF := true to ensure that IPsec is released before calling f_IMS_RestartPCSCF (see below) |

Before Change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin, timer p_TRegExpireMax, integer p_NonceCount, integer p_StatusCode, template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit, template (omit) charstring p_RegisterExpire := omit) runs on IMS_PTC { /* @sic R5s201611 Change 3: p_RegisterExpire sic@ */ var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var charstring v_Expires; var float v_TimerVal; var SipUrl v_ContactUrl; var template (value) RegInfo_RegistrationList_Type v_RegistrationList; var charstring v_PublicUserId; f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); v_IMS_REGISTER_REQ := f_IMS_PTC_ReRegistration_WaitForRegistration(p_TRegExpireMin, p_TRegExpireMax, p_NonceCount, p_RegisterExpire); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); if (isvalue(p_PrevSecurityClientParams)) { f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams)); } select (p_StatusCode) { case (500) { // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader)))); } case (200) { // @siclog "Step 21" siclog@ v_Expires := "1600"; v_TimerVal := 1000.0; p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal)); p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal)); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires)))); } case (423) { // @siclog "Step 23" siclog@ v_Expires := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires)))); // @siclog "Step 24" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 25" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires)))); // @siclog "Step 26 - 27" siclog@ v_Expires := "60"; v_TimerVal := 30.0; p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal)); p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal)); v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader); v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId); v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) }; f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList)); } case (401) { // @siclog "Step 29" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, not4b tsc_IMS_AuthRAND); // => 2nd security context IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 30" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, p_RegisterExpire); // nonce-count starts from 1 again v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 31" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); } } f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); }

After Change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin, timer p_TRegExpireMax, integer p_NonceCount, integer p_StatusCode, template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit, template (omit) charstring p_RegisterExpire := omit) runs on IMS_PTC { /* @sic R5s201611 Change 3: p_RegisterExpire sic@ */ var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var charstring v_Expires; var float v_TimerVal; var SipUrl v_ContactUrl; var template (value) RegInfo_RegistrationList_Type v_RegistrationList; var charstring v_PublicUserId; f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); v_IMS_REGISTER_REQ := f_IMS_PTC_ReRegistration_WaitForRegistration(p_TRegExpireMin, p_TRegExpireMax, p_NonceCount, p_RegisterExpire); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); if (isvalue(p_PrevSecurityClientParams)) { f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams)); } select (p_StatusCode) { case (500) { // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader)))); } case (200) { // @siclog "Step 21" siclog@ v_Expires := "1600"; v_TimerVal := 1000.0; p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal)); p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal)); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires)))); } case (423) { // @siclog "Step 23" siclog@ v_Expires := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires)))); // @siclog "Step 24" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 25" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires)))); // @siclog "Step 26 - 27" siclog@ v_Expires := "60"; v_TimerVal := 30.0; p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal)); p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal)); v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader); v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId); v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) }; f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList)); } case (401) { // @siclog "Step 29" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, not4b tsc_IMS_AuthRAND); // => 2nd security context IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 30" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, p_RegisterExpire); // nonce-count starts from 1 again v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 31" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); } } f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling,waitAndClose); }

MCC160 Implementation

function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC { var SecurityClientParams_Type v_SecurityClientParams; var charstring v_Expires; var float v_TimerVal; var integer v_NonceCount; var integer v_StatusCode; var integer v_NumOfIPAddressesToAllocate := 2; /* @sic R5-244794/R5s240470: NG session establishment with 2nd P-CSCF sic@ */ var boolean v_WaitForCNF; timer t_TRegExpireMin; timer t_TRegExpireMax; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); /* @sic R5-244794/R5s240470: NG session establishment with 2nd P-CSCF sic@ */ f_IMS_TestBody_Set(true); // @siclog "Step 1 - 9" siclog@ v_Expires := "120"; v_TimerVal := 60.0; fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @siclog "Step 10 - 11" siclog@ v_NonceCount := 2; v_StatusCode := 500; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_WaitForCNF := true; f_IMS_GlobalRelease(-, v_WaitForCNF); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); /* @sic R5-244794/R5s240470: 2nd P-CSCF - px_IPv4_Address_Other, px_IPv6_Address_Other as used in f_NR5GC_PDUSessionEstablishment_InnerLoop sic@ */ // @siclog "Step 12 - 19" siclog@ v_Expires := "360"; v_TimerVal := 180.0; fl_TC_6_3_InitialRegistration(IPCAN_MO_IMS_Signalling, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @sic R5s201611 Change 2: IPCAN_MO_IMS_Signalling sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @siclog "Step 20 - 21" siclog@ v_NonceCount := 2; v_StatusCode := 200; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 22 - 27" siclog@ v_NonceCount := 3; v_StatusCode := 423; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Step 28 - 31" siclog@ v_NonceCount := 5; v_StatusCode := 401; v_Expires := "800000"; fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, -, v_Expires); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(); // @sic R5s201611: no test procedure as connection is already released in fl_TC_6_3_ReRegistration sic@ }