3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240674

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3585 | rev | - | Current version: | 18.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_maxNumberMIMO_LayersPDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS38.101 7.2 A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a RedCap UE. Redcap UE will fail the maxNumberMIMO_LayersPDSCH check in this function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add code for redcap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## Change 1

| Functionname | fl_Check_maxNumberMIMO_LayersPDSCH |
| --- | --- |
| Reason for change | According to TS38.101 7.2 A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a RedCap UE. Redcap UE will fail the maxNumberMIMO_LayersPDSCH check in this function. |
| Summary of change | Add code for redcap UE. |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted but implemented with slight differences to simplify. |

Before Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Current_MAX_MIMO_LayersDL := 0; // 0, 2, 4 or 8 var integer v_Found_MIMO_LayersDL := 0; // if signalled, the actual Max MIMO LayersPDSCH found is 2, 4 or 8 var integer v_MIMO_LayersDLPICS_Set; var integer v_Index := 0; var boolean v_Exit := false; v_Received_NRCapabilityMsg := p_NRCapability; v_MIMO_LayersDLPICS_Set := f_MIMO_LayersDLParameterSet(); // Retrieves the maxNumberMIMO_LayersPDSCH as integer set in PICS or raises error if more than one option is chosen if (ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) //OPTIONAL parameter { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) and (not v_Exit)) { // search for the maxNumberMIMO-LayersPDSCH content in featureSetsDownlinkPerCC if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH)) { v_Found_MIMO_LayersDL := fl_Convert_NumberMIMO_LayersPDSCH(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH); if (v_Found_MIMO_LayersDL >= v_Current_MAX_MIMO_LayersDL){ v_Current_MAX_MIMO_LayersDL := v_Found_MIMO_LayersDL; } if (v_Current_MAX_MIMO_LayersDL==8) { v_Exit:= true; // It won't find a bigger value than eightLayers } } v_Index := v_Index + 1; } if (not (v_MIMO_LayersDLPICS_Set == v_Current_MAX_MIMO_LayersDL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS set for maxNumberMIMO-LayersPDSCH not matching the maxNumberMIMO-LayersPDSCH signalled by the UE for DL reception"); } // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if ((v_Current_MAX_MIMO_LayersDL == 0) and (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR > 256)) { // It is standalone NR, simple check for FR2 Band f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

After Change:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Current_MAX_MIMO_LayersDL := 0; // 0, 2, 4 or 8 var integer v_Found_MIMO_LayersDL := 0; // if signalled, the actual Max MIMO LayersPDSCH found is 2, 4 or 8 var integer v_MIMO_LayersDLPICS_Set; var integer v_Index := 0; var boolean v_Exit := false; v_Received_NRCapabilityMsg := p_NRCapability; v_MIMO_LayersDLPICS_Set := f_MIMO_LayersDLParameterSet(); // Retrieves the maxNumberMIMO_LayersPDSCH as integer set in PICS or raises error if more than one option is chosen if (ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) //OPTIONAL parameter { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) and (not v_Exit)) { // search for the maxNumberMIMO-LayersPDSCH content in featureSetsDownlinkPerCC if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH)) { v_Found_MIMO_LayersDL := fl_Convert_NumberMIMO_LayersPDSCH(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH); if (v_Found_MIMO_LayersDL >= v_Current_MAX_MIMO_LayersDL){ v_Current_MAX_MIMO_LayersDL := v_Found_MIMO_LayersDL; } if (v_Current_MAX_MIMO_LayersDL==8) { v_Exit:= true; // It won't find a bigger value than eightLayers } } v_Index := v_Index + 1; } if (not (v_MIMO_LayersDLPICS_Set == v_Current_MAX_MIMO_LayersDL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS set for maxNumberMIMO-LayersPDSCH not matching the maxNumberMIMO-LayersPDSCH signalled by the UE for DL reception"); } // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. // For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if(pc_supportOfRedCap_r17) { if (v_Current_MAX_MIMO_LayersDL < 2) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in Redcap UE"); } } else { if ((v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } } if ((v_Current_MAX_MIMO_LayersDL == 0) and (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR > 256)) { // It is standalone NR, simple check for FR2 Band f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }

MCC160 implementation:

function fl_Check_maxNumberMIMO_LayersPDSCH (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 R5s240674 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Current_MAX_MIMO_LayersDL := 0; // 0, 2, 4 or 8 var integer v_Found_MIMO_LayersDL := 0; // if signalled, the actual Max MIMO LayersPDSCH found is 2, 4 or 8 var integer v_MIMO_LayersDLPICS_Set; var integer v_Index := 0; var boolean v_Exit := false; v_Received_NRCapabilityMsg := p_NRCapability; v_MIMO_LayersDLPICS_Set := f_MIMO_LayersDLParameterSet(); // Retrieves the maxNumberMIMO_LayersPDSCH as integer set in PICS or raises error if more than one option is chosen if (ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) //OPTIONAL parameter { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC)) and (not v_Exit)) { // search for the maxNumberMIMO-LayersPDSCH content in featureSetsDownlinkPerCC if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH)) { v_Found_MIMO_LayersDL := fl_Convert_NumberMIMO_LayersPDSCH(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlinkPerCC[v_Index].maxNumberMIMO_LayersPDSCH); if (v_Found_MIMO_LayersDL >= v_Current_MAX_MIMO_LayersDL){ v_Current_MAX_MIMO_LayersDL := v_Found_MIMO_LayersDL; } if (v_Current_MAX_MIMO_LayersDL==8) { v_Exit:= true; // It won't find a bigger value than eightLayers } } v_Index := v_Index + 1; } if (not (v_MIMO_LayersDLPICS_Set == v_Current_MAX_MIMO_LayersDL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "PICS set for maxNumberMIMO-LayersPDSCH not matching the maxNumberMIMO-LayersPDSCH signalled by the UE for DL reception"); } // TS 38.306, clause 4.2.7.6 FeatureSetDownlinkPerCC parameters. if ((p_Configuration_Type == NR_SA) and (lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR) == 1)) { if ((not pc_supportOfRedCap_r17) and (v_Current_MAX_MIMO_LayersDL < 4) and (f_NR_Is5GCoreBand(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR))) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory"); } if (pc_supportOfRedCap_r17 and (v_Current_MAX_MIMO_LayersDL < 2)) { f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.101, clause 7.2: A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. "); } // Conditions for RedCap UEs if ((v_Current_MAX_MIMO_LayersDL == 0) and (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0].bandNR > 256)) { // It is standalone NR, simple check for FR2 Band f_SetVerdict(fail,__FILE__, __LINE__, "TS 38.306, clause 4.2.7.6: For single CC standalone NR, it is mandatory with capability signalling to support at least 2 MIMO layers in FR2"); } } } else if (v_MIMO_LayersDLPICS_Set > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "A maxNumberMIMO_LayersPDSCH PICS is wrongly set as this feature is not supported by the UE"); } }