3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240706

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4873 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1 in TTCN CR R5s240593 has not been implemented correctly. The delay is required between Step 7A and Step 8 to to ensure the step7A RLC STATUS PDU is received by UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | For NB-IoT NTN, enlarge the time delay according to Koffset between Step 7A and Step8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the f_TC_22_3_1_4_NBIOT	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_4_NBIOT

| Function name | f_TC_22_3_1_4_NBIOT () |
| --- | --- |
| Reason for change | Change 1 in TTCN CR R5s240593 has not been implemented correctly. The delay is required between Step 7A and Step 8 to to ensure the step7A RLC STATUS PDU is received by UE |
| Summary of change | For NB-IoT NTN, enlarge the time delay according to Koffset between Step 7A and Step8. |
| TTCN module | ttcn\develop\NBIOT\22_3\NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_22_3_1_4_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; … … … … //@siclog "Step 6" siclog@ // Msg4 scheduled at the SS: C-RNTI based contention resolution with UL grant of 176 //@siclog "Step 7" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList_Step7) }; /* @sic R5-176882, R5s170821: RACH procedure sic@ */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, noBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 7A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821: RLC STATUS PDU sic@ */ //@siclog "Step 8" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5-176882, R5s170821: timing sic@ */ v_RlcSduList := { crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 9 - 10" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* 3 NPDCCH cycles after sending of the additional RLC SDU at step 8 @sic R5-172835/R5s170238 change 2.3: 2 cycles instead of 1 sic@ @sic R5-176882, R5s170821: 3 cycles instead of 2 sic@ * @sic R5-241564 3 bundles for TN, 1 bundle for NTN sic@ */ v_BSR_Index_Min := 1; /* @sic R5-176882, R5s170821: 1 instead of 8 sic@ */ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10");

After Change:

function f_TC_22_3_1_4_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; … … … … //@siclog "Step 6" siclog@ // Msg4 scheduled at the SS: C-RNTI based contention resolution with UL grant of 176 //@siclog "Step 7" siclog@ v_MAC_SDUList_RX := { f_NBIOT_MacSdu_RlcAmdPdu_RX(v_RlcSduList_Step7) }; /* @sic R5-176882, R5s170821: RACH procedure sic@ */ v_MAC_PDU_RX := f_NBIOT_MacPdu_BSR_MacSdus_Padding_RX(-, noPadding, noBSR, v_MAC_SDUList_RX); f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, v_CellId, v_MAC_PDU_RX); //moved code //@siclog "Step 7A" siclog@ f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, v_CellId); /* @sic R5-176882, R5s170821: RLC STATUS PDU sic@ */ //WA#WI Delay shall be after Step 7A if(f_NBIOT_MobileInfo_IsNTN()) { //@sic R5s240593 sic@ f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(v_CellId))/500.0); // 2 * NTN message delay (550ms * 2) } //@siclog "Step 8" siclog@ v_SubFrameTiming := f_NBIOT_GetNextSearchSpace(v_CellId, tsc_NBIOT_CPModeSendRlcSdus_SchedulingDelay); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_SubFrameTiming); /* @sic R5-176882, R5s170821: timing sic@ */ v_RlcSduList := { crs_PDCP_SDU_10B }; f_NBIOT_MAC_CPMode_SendRlcSdus(v_RLC_CountsInfoListByRef, v_CellId, v_TestLoopMode, v_TimingInfo, v_RlcSduList); //@siclog "Step 9 - 10" siclog@ v_SubFrameTiming := f_NBIOT_IncrementSearchSpace(v_SubFrameTiming, v_SchedulingOffset); /* 3 NPDCCH cycles after sending of the additional RLC SDU at step 8 @sic R5-172835/R5s170238 change 2.3: 2 cycles instead of 1 sic@ @sic R5-176882, R5s170821: 3 cycles instead of 2 sic@ * @sic R5-241564 3 bundles for TN, 1 bundle for NTN sic@ */ v_BSR_Index_Min := 1; /* @sic R5-176882, R5s170821: 1 instead of 8 sic@ */ fl_TC_22_3_1_4_TriggerBSR(v_RLC_CountsInfoListByRef, v_CellId, v_SubFrameTiming, v_BSR_Index_Min); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 10");