3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250041

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0963 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS-IRAT testcase 19.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of TTCN, at Step 9b4 in the RAU Accept message, the IE PDP Context Status is populated based on what is received in the RAU REQ. However, according to 34.229-1 specific message contents for Step 9b4, the IE PDP Context Status should set the NSAPI(0)- NSAPI(15) to 0, which means that the SM state of all PDP Contexts is PDP-INACTIVE. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN to set PDP Context Status according to the specific message content. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not in line with Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Parikshit Bhise
	Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_UTRAN_GMM_RAU |
| --- | --- |
| Reason for change | In the current implementation of TTCN, at Step 9b4 in the RAU Accept message, the IE PDP Context Status is populated based on what is received in the RAU REQ. However, according to 34.229-1 specific message contents for Step 9b4, the IE PDP Context Status should set the NSAPI(0)- NSAPI(15) to 0, which means that the SM state of all PDP Contexts is PDP-INACTIVE. This needs to be corrected. |
| Summary of change | The function f_TC_19_1_3_UTRAN f_IMS_CSFB_UTRAN_Body f_UTRAN_GMM_RAU is called in which the variable v_PDP_ContextStatus is assigned with the value received in the RAU REQ, which is then passed to function f_UTRAN_GMM_RAU_Without_RAUReq . Instead of this, passed a new parameter p_PDP_ContextStatus, which is assigned to parameter v_PDP_ContextStatus, and then this is passed to f_UTRAN_GMM_RAU_Without_RAUReq. In the call to function f_UTRAN_GMM_RAU, the parameter p_PDP_ContextStatus is initialized with cs_PDP_ContextStatusInactive. Please see changes below as well. |
| TTCN module | UTRAN_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function f_UTRAN_GMM_RAU(UTRAN_CellId_Type p_CellId, boolean p_DoAuth := true, boolean p_NoSecurityAfterHO := false) runs on UTRAN_PTC { var START_Value v_START_Value := '00000000000000000000'B; var B3_Type v_UpdateType := '000'B; var RRC_DATA_IND v_RRC_DataInd; var PDP_ContextStatus v_PDP_ContextStatus; var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get(); var template (present) RRCDataIndType v_RAU := cr_U_RA_UpdReqAny(cr_GMM_UpdateType(?, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI))); // @sic R5s120776 sic@ alt { [] U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} [] U_Dc.receive(car_PS_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} } v_SecurityInfo.authKeys.AuthPS.KeySeq := v_RRC_DataInd.msg.routingAreaUpdateRequest.gprsCiphKeySeqNo.keySeq; // @sic R5s140858 sic@ f_UTRAN_Security_Set(v_SecurityInfo); // @sic R5s140858 sic@ if (p_DoAuth) { // @sic R5s120141 sic@ v_START_Value := '00000000000000000000'B; } else if (ispresent(v_RRC_DataInd.ttcn_start)) { v_START_Value := v_RRC_DataInd.ttcn_start; } else { v_START_Value := v_SecurityInfo.start_PS; // current value. If not previously set, default value is all zeros } if (not p_NoSecurityAfterHO) { // @sic R5s110216 sic@ f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); } v_UpdateType := f_UG_RauReq_GetUpdateValueForRauAcc(v_RRC_DataInd.msg.routingAreaUpdateRequest); /* @sic R5s120019 Additional Changes sic@ */ v_PDP_ContextStatus := v_RRC_DataInd.msg.routingAreaUpdateRequest.pDP_ContextStatus; f_UTRAN_GMM_RAU_Without_RAUReq(p_CellId, v_START_Value, v_UpdateType, v_PDP_ContextStatus, p_DoAuth, p_NoSecurityAfterHO); // @sic R5-133584 sic@ }

After Change

function f_UTRAN_GMM_RAU(UTRAN_CellId_Type p_CellId, boolean p_DoAuth := true, boolean p_NoSecurityAfterHO := false, template (omit) PDP_ContextStatus p_PDP_ContextStatus := omit) runs on UTRAN_PTC //WA#WI1372000 WA#19_1_3 { var START_Value v_START_Value := '00000000000000000000'B; var B3_Type v_UpdateType := '000'B; var RRC_DATA_IND v_RRC_DataInd; //var PDP_ContextStatus v_PDP_ContextStatus; var template (omit) PDP_ContextStatus v_PDP_ContextStatus; //WA#WI1372000 WA#19_1_3 var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get(); var template (present) RRCDataIndType v_RAU := cr_U_RA_UpdReqAny(cr_GMM_UpdateType(?, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI))); // @sic R5s120776 sic@ alt { [] U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} [] U_Dc.receive(car_PS_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, v_RAU)) // @sic R5s110216 sic@ -> value v_RRC_DataInd {} } v_SecurityInfo.authKeys.AuthPS.KeySeq := v_RRC_DataInd.msg.routingAreaUpdateRequest.gprsCiphKeySeqNo.keySeq; // @sic R5s140858 sic@ f_UTRAN_Security_Set(v_SecurityInfo); // @sic R5s140858 sic@ if (p_DoAuth) { // @sic R5s120141 sic@ v_START_Value := '00000000000000000000'B; } else if (ispresent(v_RRC_DataInd.ttcn_start)) { v_START_Value := v_RRC_DataInd.ttcn_start; } else { v_START_Value := v_SecurityInfo.start_PS; // current value. If not previously set, default value is all zeros } if (not p_NoSecurityAfterHO) { // @sic R5s110216 sic@ f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); } v_UpdateType := f_UG_RauReq_GetUpdateValueForRauAcc(v_RRC_DataInd.msg.routingAreaUpdateRequest); /* @sic R5s120019 Additional Changes sic@ */ v_PDP_ContextStatus := v_RRC_DataInd.msg.routingAreaUpdateRequest.pDP_ContextStatus; if(isvalue(p_PDP_ContextStatus)) { //WA#WI1372000 WA#19_1_3 v_PDP_ContextStatus := p_PDP_ContextStatus; } f_UTRAN_GMM_RAU_Without_RAUReq(p_CellId, v_START_Value, v_UpdateType, v_PDP_ContextStatus, p_DoAuth, p_NoSecurityAfterHO); // @sic R5-133584 sic@ }

## Change 2

| Function name | f_IMS_CSFB_UTRAN_Body |
| --- | --- |
| Reason for change | Please see change 1 above. |
| Summary of change | Added p_PDP_ContextStatus as additional argument to function , and passed it to function f_UTRAN_GMM_RAU |
| TTCN module | IMS_IRAT_CSFB_UTRAN.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_CSFB_UTRAN_Body (EMERGENCY_CALL_Type p_CallType := normalEmergencyCall, template ServCategory p_ServCategory := cr_ServCategory ifpresent) runs on UTRAN_IRAT_PTC { // 19.1.3, 19.3.2 (and 19.3.2c) now all have the same procedure in UTRAN // 21.x as well //Initialise test case f_IMS_CSFB_UTRAN_Init (); // @sic R5s160070 sic@ f_UTRAN_TestBody_Set(true); //@siclog "Step 9a1" siclog@ // UE performs cell reselection to a cell supporting CS domain (UTRAN or GERAN cell) and performs emergency call in CS domain together with MM/GMM registration f_UTRAN_EmgCallSetup_RAU(utran_Cell5, p_ServCategory); // @sic R5-184704, R5s140788, R5-145792 sic@ //@siclog "Step 9a2" siclog@ if (p_CallType == eCall) { f_Delay(5.0); } //The SS releases the UE connection & receives release complete message(s). f_UTRAN_CC_Disconnect(cs_TI_MO); // @sic R5s140788 sic@ @sic R5-145792 sic@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch, true); f_UTRAN_RRC_ConnEst(utran_Cell5); f_UTRAN_GMM_RAU(utran_Cell5, false); f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-182560, R5-183106 sic@ // Switch off/power off and release cells // f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@ f_UTRAN_Postamble(utran_Cell5, U1_IDLE); f_UTRAN_TestBody_Set(false); // send coordination message for end of test case f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); }

After Change

function f_IMS_CSFB_UTRAN_Body (EMERGENCY_CALL_Type p_CallType := normalEmergencyCall, template ServCategory p_ServCategory := cr_ServCategory ifpresent, template (omit) PDP_ContextStatus p_PDP_ContextStatus := omit) runs on UTRAN_IRAT_PTC //WA#WI1372000 WA#19_1_3 { // 19.1.3, 19.3.2 (and 19.3.2c) now all have the same procedure in UTRAN // 21.x as well //Initialise test case f_IMS_CSFB_UTRAN_Init (); // @sic R5s160070 sic@ f_UTRAN_TestBody_Set(true); //@siclog "Step 9a1" siclog@ // UE performs cell reselection to a cell supporting CS domain (UTRAN or GERAN cell) and performs emergency call in CS domain together with MM/GMM registration f_UTRAN_EmgCallSetup_RAU(utran_Cell5, p_ServCategory); // @sic R5-184704, R5s140788, R5-145792 sic@ //@siclog "Step 9a2" siclog@ if (p_CallType == eCall) { f_Delay(5.0); } //The SS releases the UE connection & receives release complete message(s). f_UTRAN_CC_Disconnect(cs_TI_MO); // @sic R5s140788 sic@ @sic R5-145792 sic@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch, true); f_UTRAN_RRC_ConnEst(utran_Cell5); f_UTRAN_GMM_RAU(utran_Cell5, false, -, p_PDP_ContextStatus); //WA#WI1372000 WA#19_1_3 f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); f_UTRAN_OptionalPDPEstabForIMSReRegistration(utran_Cell5); // @sic R5-182560, R5-183106 sic@ // Switch off/power off and release cells // f_UTRAN_IP_Handling_Change_Configuration(utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140788, R5s150736 now done in postamble sic@ f_UTRAN_Postamble(utran_Cell5, U1_IDLE); f_UTRAN_TestBody_Set(false); // send coordination message for end of test case f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); }

## Change 3

| Function name | f_TC_19_1_3_UTRAN |
| --- | --- |
| Reason for change | Please see change 1 and 2 above |
| Summary of change | Updated the call to function f_IMS_CSFB_UTRAN_Body and passed cs_PDP_ContextStatusInactive as argument to initialize the IE PDP Context Status. |
| TTCN module | IMS_IRAT_19_UTRAN.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_19_1_3_UTRAN() runs on UTRAN_IRAT_PTC { /* Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain */ var template ServCategory v_ServCategory := cr_ServCategory(cr_EmergServCat_non_eCall) ifpresent; f_IMS_CSFB_UTRAN_Body(-, v_ServCategory); // @sic R5-1500700, R5-185087 sic@ }

After Change

function f_TC_19_1_3_UTRAN() runs on UTRAN_IRAT_PTC { /* Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain */ var template ServCategory v_ServCategory := cr_ServCategory(cr_EmergServCat_non_eCall) ifpresent; f_IMS_CSFB_UTRAN_Body(-, v_ServCategory, cs_PDP_ContextStatusInactive); // @sic R5-1500700, R5-185087 sic@ //WA#WI1372000 WA#19_1_3 }