3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250046

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4874 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The updated implementation of R5s240579 results in a runtime error when f_NBIOT_MobileInfo_IsNTN is called without f_NBIOT_Init is being called. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the test case sequence as f_NBIOT_Init and update SIB31. Confgiure the cell and call f_NBIOT_L2_Enter_State2B_NB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 22.3.1.6	2

2.1	Change to f_TC_22_3_1_6_NBIOT	2

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 22.3.1.6

## Change to f_TC_22_3_1_6_NBIOT

| Function name | f_TC_22_3_1_6_NBIOT |
| --- | --- |
| Reason for change | The updated implementation of R5s240579 results in a runtime error when f_NBIOT_MobileInfo_IsNTN is called without f_NBIOT_Init is being called. |
| Summary of change | Updated the test case sequence as f_NBIOT_Init and update SIB31. Confgiure the cell and call f_NBIOT_L2_Enter_State2B_NB |
| TTCN module | NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var SubFrameTiming_Type v_SubFrameTiming; var template (value) TimingInfo_Type v_TimingInfo; var Table_22_3_1_6_3_2__2_Entry_Type v_TestData; var RLC_SDU_Type v_RlcSdu; var RLC_SDUList_Type v_RlcSduList; var integer v_RlcSduSize; var integer v_Start := 0; var integer i; var integer v_NpdcchBundleOffset; var float v_SchedulingOffset; var template (omit) NB_NTN_SpecificSysInfoParams_Type v_NB_NTN_SpecificSysInfoParams := omit; if(f_NBIOT_MobileInfo_IsNTN()){ //@sic R5s240579, R5-246574 sic@ v_NB_NTN_SpecificSysInfoParams := cds_SpecificSyncValidityDuration(s900); /*@sic R5-247617 sic@*/ } f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, -,-,-,-, v_NB_NTN_SpecificSysInfoParams); //@sic R5s240579 sic@ v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@

After Change:

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var SubFrameTiming_Type v_SubFrameTiming; var template (value) TimingInfo_Type v_TimingInfo; var Table_22_3_1_6_3_2__2_Entry_Type v_TestData; var RLC_SDU_Type v_RlcSdu; var RLC_SDUList_Type v_RlcSduList; var integer v_RlcSduSize; var integer v_Start := 0; var integer i; var integer v_NpdcchBundleOffset; var float v_SchedulingOffset; var template (omit) NB_NTN_SpecificSysInfoParams_Type v_NB_NTN_SpecificSysInfoParams := omit; //WA#WI=1368159 f_NBIOT_Init(c1); if(f_NBIOT_MobileInfo_IsNTN()){ f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(v_CellId,s900); } //Create and configure all cells f_NBIOT_CellConfig_Def(v_CellId, CONTROL_PLANE); //f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, -,-,-,-, v_NB_NTN_SpecificSysInfoParams); //@sic R5s240579 sic@ f_NBIOT_L2_Enter_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode); v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@