3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250011

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0961 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-12-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect placement of passing parameter p_OriginatingIdentificationRestriction while calling function f_IMS_A_2_1_Invite_CommonChecks causes TC to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the parameter p_OriginatingIdentificationRestriction at correct place |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 34.229-1 Annex C.21 (context A.2.1.A4) Testcases 15.2a, J.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |
| --- | --- |
| Reason for change | Incorrect placement of passing parameter p_OriginatingIdentificationRestriction while calling function f_IMS_A_2_1_Invite_CommonChecks causes TC to fail. |
| Summary of change | Pass the parameter p_OriginatingIdentificationRestriction at correct place |
| TTCN module | IMS_Procedures_CallControl4G5G |
| MCC160 Comment | Accepted but implemented differently by moving p_SrvccAlertingSupported behind p_OriginatingIdentificationRestriction in f_IMS_A_2_1_Invite_CommonChecks |

Before Change

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_MimeWithPIDF := ispresent(p_MessageHeader_Invite.geolocation); /* @sic R5s230480 sic@ NOTE: there is no Geolocation without PIDF body part and no PIDF body part without Geolocation and the Geolcation addresses the PIDF body part within the MIME body */ var boolean v_MimeWithUSSD := ispresent(p_MessageHeader_Invite.recvInfo) and match(valueof(cs_RecvInfo_Def("g.3gpp.ussd")), p_MessageHeader_Invite.recvInfo); /* @sic R5s230734 sic@ */ var boolean v_MimeWithXML := v_MimeWithPIDF or v_MimeWithUSSD; var boolean v_MimeWithBinaryData := ispresent(p_MessageHeader_Invite.callInfo); /* @sic R5s230480 sic@ NOTE: there is no Call-Info without MSD and no MSD without Call-Info and Call-Info addresses the MSD body part within the MIME body */ var template (present) MessageBody v_MessageBody := f_SIP_MessageBodyWithSDP_RX(v_MimeWithXML, v_MimeWithBinaryData); /* @sic R5w140112 sic@ @sic R5s210009: v_UseMIME instead of p_A8 sic@ @sic R5s230480: v_MimeWithXML, v_MimeWithBinaryData instead of v_UseMIME sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; }

After Change

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A11 := false, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_MimeWithPIDF := ispresent(p_MessageHeader_Invite.geolocation); /* @sic R5s230480 sic@ NOTE: there is no Geolocation without PIDF body part and no PIDF body part without Geolocation and the Geolcation addresses the PIDF body part within the MIME body */ var boolean v_MimeWithUSSD := ispresent(p_MessageHeader_Invite.recvInfo) and match(valueof(cs_RecvInfo_Def("g.3gpp.ussd")), p_MessageHeader_Invite.recvInfo); /* @sic R5s230734 sic@ */ var boolean v_MimeWithXML := v_MimeWithPIDF or v_MimeWithUSSD; var boolean v_MimeWithBinaryData := ispresent(p_MessageHeader_Invite.callInfo); /* @sic R5s230480 sic@ NOTE: there is no Call-Info without MSD and no MSD without Call-Info and Call-Info addresses the MSD body part within the MIME body */ var template (present) MessageBody v_MessageBody := f_SIP_MessageBodyWithSDP_RX(v_MimeWithXML, v_MimeWithBinaryData); /* @sic R5w140112 sic@ @sic R5s210009: v_UseMIME instead of p_A8 sic@ @sic R5s230480: v_MimeWithXML, v_MimeWithBinaryData instead of v_UseMIME sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A11, -, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; }

MCC160 Implementation

function f_IMS_A_2_1_Invite_CommonChecks(IMS_DATA_REQ p_IMS_INVITE_REQ, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := false, boolean p_A11 := false, boolean p_OriginatingIdentificationRestriction := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_A18 := pc_BSRVCC) runs on IMS_PTC { … }