3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250065

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3625 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Step 19 of the TC, the prose states that “The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Z’, NDI=1; Where (P+3X < Z <P+4X); The DL HARQ process is same as in step 18.” The current implementation uses the variable v_RetransSchInt to control the re-transmissions. However, the value of this variable is incorrect as with 8ms, the transmission cannot happen on CS-RNTI as it does not match the SPS pattern and also, in the case of TDD with absoluteK1 = 8 slots, UE cannot send an HARQ ACK at the specified slot as this would be a DL Slot For e.g.: With v_RetransSchInt = 8 In TDD (30Khz SCS): Corrupted DL transmission using CS-RNTI at SFN/SF/Slot = 491/0/0, HARQ NACK is to be received at 491/4/0 (an UL Slot) 1st Normal DL transmission at 491/8/0, not matching SPS pattern, with absoluteK1 = 8, HARQ ACK/NACK not possible at 492/2/0 as this is DL slot. 2nd Normal DL transmission at 492/6/0, not matching SPS pattern, with absoluteK1 = 8, HARQ ACK/NACK not possible at 493/0/0 as this is DL slot. 3rd Normal DL transmission at 493/4/0, not matching SPS pattern also this is a UL Slot. Hence, transmission is not possible. Hence, in order to send the re-transmissions on CS-RNTI, it is necessary to match the SPS pattern and set the variable v_RetransSchInt = 10 ms (1 SFN) for the case of TDD. This would result in re-transmissions at 492/0/0, 493/0/0, 494/0/0 in the above example matching the SPS pattern and also aligning with the absoluteK1 = 8 for reception of HARQ ACK/NACK. For the case of FDD (15 Khz) as well, the calculated timings,with v_RetransSchInt = 8, do not match the SPS pattern and cannot be transmitted using the CS-RNTI However, the variable v_RetransSchInt = 20 ms (2 SFNs) because here 1 SF = 1 ms and with absoluteK1 = 8, it would leave only 1 ms to transmit the next re-transmission. For e.g. in FDD, if Corrupted DL transmission using CS-RNTI is at 491/1/0, HARQ NACK would be received at 491/9/0, and if v_RetransSchInt = 10ms , the first re-transmission would be at 492/1/0 leaving only 1 ms gap for the SS implementation to schedule the re-transmission on the same HARQ process. This is too tight for the SS implementation. Hence, with v_RetransSchInt = 20ms, it would be 493/1/0 for the first re-transmission, with the same HARQ process now being available for the re-transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the variable v_RetransSchInt = 10 (TDD) and v_RetransSchInt = 20 (FDD) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC, 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Overview	5

2	Corrections required	5

2.1	Change 1	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | At Step 19 of the TC, the prose states that “The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Z’, NDI=1; Where (P+3X < Z <P+4X); The DL HARQ process is same as in step 18.” The current implementation uses the variable v_RetransSchInt to control the re-transmissions. However, the value of this variable is incorrect, as with 8ms, the transmission cannot happen on CS-RNTI as it does not match the SPS pattern and also, in the case of TDD, with absoluteK1 = 8 slots, UE cannot send an HARQ ACK at the specified slot as this would be a DL Slot. For e.g.: With v_RetransSchInt = 8 In TDD (30Khz SCS): Corrupted DL transmission using CS-RNTI at SFN/SF/Slot = 491/0/0, HARQ NACK is to be received at 491/4/0 (an UL Slot) 1st Normal DL transmission at 491/8/0, not matching SPS pattern, with absoluteK1 = 8, HARQ ACK/NACK not possible at 492/2/0 as this is DL slot. 2nd Normal DL transmission at 492/6/0, not matching SPS pattern, with absoluteK1 = 8, HARQ ACK/NACK not possible at 493/0/0 as this is DL slot. 3rd Normal DL transmission at 493/4/0, not matching SPS pattern also this is a UL Slot. Hence, transmission is not possible. Hence, in order to send the re-transmissions on CS-RNTI, it is necessary to match the SPS pattern and set the variable v_RetransSchInt = 10 ms (1 SFN) for the case of TDD. This would result in re-transmissions at 492/0/0, 493/0/0, 494/0/0 in the above example matching the SPS pattern and also aligning with the absoluteK1 = 8 for reception of HARQ ACK/NACK For the case of FDD (15 Khz) as well, the calculated timings, with v_RetransSchInt = 8,do not match the SPS pattern and cannot be transmitted using the CS-RNTI However, the variable v_RetransSchInt = 20 ms (2 SFNs) because here 1 SF = 1 ms and with absoluteK1 = 8, it would leave only 1 ms to transmit the next re-transmission. For e.g. in FDD, if Corrupted DL transmission using CS-RNTI is at 491/1/0, HARQ NACK would be received at 491/9/0, and if v_RetransSchInt = 10ms , the first re-transmission would be at 492/1/0 leaving only 1 ms gap for the SS implementation to schedule the re-transmission on the same HARQ process. This is too tight for the SS implementation. Hence, with v_RetransSchInt = 20ms, it would be 493/1/0 for the first re-transmission, with the same HARQ process now being available for the re-transmission. |
| Summary of change | Modified the variable v_RetransSchInt = 10 (TDD) and v_RetransSchInt = 20 (FDD) |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comments | Accepted as improvement and uniform value of 20 ms used for all |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> var integer v_RetransSchInt := 8; //suitable for FDD var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> var integer v_RetransSchInt; //WA# var integer i; var NR_RedundancyVersionList_Type v_RVList := {0, 2, 3, 1, 0}; var integer v_Nack_Count := 0; <<SKIPPED CODE>> select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; v_RetransSchInt := 20; //WA# } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; v_RetransSchInt := 10; //WA# } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } <<SKIPPED CODE>>