3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250087

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3635 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_CheckDataPath_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TTCN CR R5s240675 proposed changes for function f_NR5GC_CheckDataPath_CA. A similar situation exists in the function f_NR_CheckDataPath_CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented the MCC160 proposed implementation of TTCN CR R5s240675 in the function f_NR_CheckDataPath_CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confirmant UE may fail the case due to timing issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the ENDC CA Test cases involving Scell Activation MAC CE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change to the function f_NR_CheckDataPath_CA	3

2.2	Change to the function f_NR_CheckDataPath	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the function f_NR_CheckDataPath_CA

| Function name | f_NR_CheckDataPath_CA |
| --- | --- |
| Reason for change | The TTCN CR R5s240675 proposed changes for function f_NR5GC_CheckDataPath_CA. A similar situation exists in the function f_NR_CheckDataPath_CA |
| Summary of change | Implemented the MCC160 proposed implementation of TTCN CR R5s240675 in the function f_NR_CheckDataPath_CA |
| TTCN module | NR_CommonFunctions_ENDC_CA.ttcn |
| MCC160 | Accepted |

Before Change:

function f_NR_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_2, boolean p_UL_CA := false ) runs on NR_BASE_PTC { //@sic R5-213516 sic@ var template (value) MacBearerRouting_Type v_MacBearerRoutingDL := crs_MacBearerRouting(p_SCell); //Send DL data on SCell var template (value) MacBearerRouting_Type v_MacBearerRoutingUL := crs_MacBearerRouting(p_PCell); // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex); } if (p_UL_CA) { v_MacBearerRoutingUL := crs_MacBearerRouting(p_SCell); } // Steps 2 f_NR_CheckDataPath(p_PCell, // for DL @sic R5s210681 sic@ PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(p_PCell), cs_RlcBearerRouting_NR(p_PCell), p_StepNo, -, v_MacBearerRoutingDL, v_MacBearerRoutingUL ); }

After Change:

module NR_CommonFunctions_ENDC_CA { import from CommonDefs all; import from CommonAspDefs all; //WA# <<SKIPPED CODE>> import from Common4G5G_Timing all; //WA# import from NR_Timing all; //WA# <<SKIPPED CODE>> function f_NR_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_2, boolean p_UL_CA := false ) runs on NR_BASE_PTC { //@sic R5-213516 sic@ var template (value) MacBearerRouting_Type v_MacBearerRoutingDL := crs_MacBearerRouting(p_SCell); //Send DL data on SCell var template (value) MacBearerRouting_Type v_MacBearerRoutingUL := crs_MacBearerRouting(p_PCell); var SubFrameTiming_Type v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell); //WA# // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex, cs_TimingInfo_NR(v_TimingInfo)); //WA# } if (p_UL_CA) { v_MacBearerRoutingUL := crs_MacBearerRouting(p_SCell); } //WA# Add 20ms before sending data (longest k1=7slots=7ms in SCS15kHz allowing a retransmission ) v_TimingInfo := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 20); // Steps 2 f_NR_CheckDataPath(p_PCell, // for DL @sic R5s210681 sic@ PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(p_PCell), cs_RlcBearerRouting_NR(p_PCell), p_StepNo, -, v_MacBearerRoutingDL, v_MacBearerRoutingUL, cs_TimingInfo_NR(v_TimingInfo) //WA# ); }

## Change to the function f_NR_CheckDataPath

| Function name | f_NR_CheckDataPath |
| --- | --- |
| Reason for change | The TTCN CR R5s240675 proposed changes for function f_NR5GC_CheckDataPath_CA. A similar situation exists in the function f_NR_CheckDataPath_CA |
| Summary of change | Implemented the MCC160 proposed implementation of TTCN CR R5s240675 in the function f_NR_CheckDataPath_CA |
| TTCN module | NR_CommonFunctions_ENDC.ttcn |
| MCC160 | Accepted |

Before Change:

function f_NR_CheckDataPath(NR_CellId_Type p_NR_CellId, PDN_Index_Type p_PdnIndex, DataPathCheck_BearerType_Type p_BearerType, DRB_Identity p_DRBId, template (value) RlcBearerRouting_Type p_RlcRoutingDL, template (omit) RlcBearerRouting_Type p_RlcRoutingUL := omit, charstring p_StepNo := "", integer p_SequenceNumber := 1, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA_DL := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA_UL := omit ) runs on NR_BASE_PTC { /* implementation as according to @sic R5s190117 R5s190161 sic@ */ /* @sic R5-193981, R5-193983: p_PdnIndex, p_BearerType sic@ */ /* Rs190936: split into 2 functions */ /* @sic R5-203666: added p_UL_Cell for ENDC CA test cases sic@ */ /* @sic R5-213516: added p_MacBearerRoutingCA_DL and p_MacBearerRoutingCA_UL deleted p_UL_Cell sic@ */ var octetstring v_SduTX := f_LoopbackModeBorPing_GetDataTX(p_PdnIndex, p_BearerType, p_SequenceNumber); var NR_PDCP_SDUList_Type v_PDCP_Data := { v_SduTX }; var template (value) NR_DRB_COMMON_REQ v_DRB_DL; var template (present) NR_DRB_COMMON_IND v_DRB_UL; var template (value) RlcBearerRouting_Type v_RlcRoutingUL; if (isvalue(p_RlcRoutingUL)) { v_RlcRoutingUL := valueof(p_RlcRoutingUL); } else { v_RlcRoutingUL := p_RlcRoutingDL; } v_DRB_DL := cas_NR_DRB_COMMON_REQ_PDCP_SDUList_DC(p_NR_CellId, p_DRBId, p_RlcRoutingDL, -, v_PDCP_Data, p_MacBearerRoutingCA_DL); v_DRB_UL := car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(p_NR_CellId, p_DRBId, v_RlcRoutingUL, -, -, p_MacBearerRoutingCA_UL); f_NR_CheckDataPathCommon(v_SduTX, v_DRB_DL, v_DRB_UL, p_StepNo); }

After Change:

function f_NR_CheckDataPath(NR_CellId_Type p_NR_CellId, PDN_Index_Type p_PdnIndex, DataPathCheck_BearerType_Type p_BearerType, DRB_Identity p_DRBId, template (value) RlcBearerRouting_Type p_RlcRoutingDL, template (omit) RlcBearerRouting_Type p_RlcRoutingUL := omit, charstring p_StepNo := "", integer p_SequenceNumber := 1, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA_DL := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA_UL := omit, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now //WA# ) runs on NR_BASE_PTC { /* implementation as according to @sic R5s190117 R5s190161 sic@ */ /* @sic R5-193981, R5-193983: p_PdnIndex, p_BearerType sic@ */ /* Rs190936: split into 2 functions */ /* @sic R5-203666: added p_UL_Cell for ENDC CA test cases sic@ */ /* @sic R5-213516: added p_MacBearerRoutingCA_DL and p_MacBearerRoutingCA_UL deleted p_UL_Cell sic@ */ var octetstring v_SduTX := f_LoopbackModeBorPing_GetDataTX(p_PdnIndex, p_BearerType, p_SequenceNumber); var NR_PDCP_SDUList_Type v_PDCP_Data := { v_SduTX }; var template (value) NR_DRB_COMMON_REQ v_DRB_DL; var template (present) NR_DRB_COMMON_IND v_DRB_UL; var template (value) RlcBearerRouting_Type v_RlcRoutingUL; if (isvalue(p_RlcRoutingUL)) { v_RlcRoutingUL := valueof(p_RlcRoutingUL); } else { v_RlcRoutingUL := p_RlcRoutingDL; } v_DRB_DL := cas_NR_DRB_COMMON_REQ_PDCP_SDUList_DC(p_NR_CellId, p_DRBId, p_RlcRoutingDL, p_TimingInfo, v_PDCP_Data, p_MacBearerRoutingCA_DL); //WA# v_DRB_UL := car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(p_NR_CellId, p_DRBId, v_RlcRoutingUL, -, -, p_MacBearerRoutingCA_UL); f_NR_CheckDataPathCommon(v_SduTX, v_DRB_DL, v_DRB_UL, p_StepNo); }