3GPP RAN5 - TTCN Workshop #45	R5w190108r1

# Sophia Antipolis, France, 16th April 2019

Agenda Item:	10 (5G)

Source:	MCC TF160

Title:	NR TBS matters (continued)

Document for:	Endorsement

# Introduction

At TTCN Workshop #44, a couple of contributions were presented on NR TBS matters in R5w190003 and R5w190013. Consequently, a prose CR R5-192858 was agreed at RAN5#82, with one editor’s note added at the beginning of Annex B.2 to cover one point not concluded yet.

The purpose of this contribution is to discuss and conclude on the open point.

# Discussion

The following editor’s note is present in TS 38.523-3 version 15.3.0 at the beginning of Annex B.2:

# B.2	Uplink TBS (informative)

Editor’s Note: For creation of the TBS tables in the present Annex B.2, the same criteria, as specified for the DL in subclause 7.1.2.2.4.2, have been applied for selection of the LRBs / IMCS pair. Discussions are ongoing in RAN5 on this matter.

The discussion point is on the second rule (RuleB) of the LRBs / IMCS pair determination of subclause 7.1.2.2.4.2:

-------------------------------------------------------------------------

LRBs / IMCS pair determination

Using the selected TBS, the SS shall apply the following rules:

-	[RuleA] When the TBS can be achieved with more than one LRBs / IMCS pairs the SS shall choose the LRBs / IMCS pairs with maximum LRBs.

-	[RuleB] When there is more than one LRBs / IMCS pair with maximum LRBs the pair with maximum IMCS shall be chosen.

-------------------------------------------------------------------------

There is consideration on whether the maximum IMCS or the minimum IMCS should be used for RuleB.

First, to put the issue at stake into perspective, let us see how often we have practical cases where we still have more than 1 pairs to select from after RuleA (maximum LRBs) has been applied.

In section 4 you will find TS 38.523-3 version 15.3.0 Annex B, with the TBS tables updated to show the pairs still available after RuleA has been applied:

-	Yellow highlighted are the TBSs with more than 1 pairs still available,

-	In red is the 2nd IMCS available.

The following observations can be drawn:

Observation1: In each TBS table, only a small number of TBSs still have more than 1 pairs still available after RuleA has been applied.

Observation2: For the current configurations, there are not more than 2 IMCS values per LRBs, not more than 2 pairs per TBS.

Observation3: If there are 2 IMCS values for a particular LRBs, then the IMCS values belong to different modulation orders (see TS 38.214 Table 5.1.3.1-1), typically:

-	{IMCS10; Qm4 = 16QAM} vs. {IMCS9; Qm2 = QPSK}, or

-	{IMCS17; Qm6 = 64QAM} vs. {IMCS16; Qm4 = 16QAM}.

Observation4: According to TS 38.214 Table 5.1.3.1-1:

-	IMCS10 provide a lower target code rate compared to IMCS9,

-	IMCS17 provide a lower target code rate compared to IMCS16.

TS 38.214-f50 Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

In conclusion, for the cases where more than 1 pairs are still available after RuleA has been applied:

-	When using ‘minimum’: we get higher code rate but lower modulation order,

-	When using ‘maximum’: we get higher modulation order but lower code rate.

In TF160, we have selected the ‘lower code rate’ option because:

-	It provides more redundancy of the data bits onto the physical channel symbols, which in principle makes it easier for the receiver to be able to recover from potential transmission errors, which may have occurred on the physical medium (RF cable or OTA).

-	And also because of the particular case of TBS = 24 (see R5w190003 section 2a).

# Conclusion

SS Vendors are asked to state their preference, with technical justification, on whether they prefer as RuleB:

-	maximum IMCS, or

-	minimum IMCS.

Note: As second attachment to the contribution zip file is a draft prose CR proposing some prose clarifications on the TBS subclauses, assuming maximum IMCS is kept. In case minimum IMCS is chosen, the draft prose CR will need to be updated.

# TBS tables in 38.523-3 annex B after RuleA has been applied

Annex B: NR TBS tables

# B.1	Downlink TBS (normative)

The tables in this clause are depending on parameters provided by RRC signalling as described in subclause 7.1.2.2.4.1. LRBs is limited according to the DL scheduling scheme in subclause 7.1.2.2.3.

## B.1.1	Downlink TBS using MCS index table 5.1.3.1-1

### B.1.1.1	Downlink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 0, number of CDM groups = 1

Table B.1.1.1-1: Void

Table B.1.1.1-2: Void

Table B.1.1.1-3: TBS for PDSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 0, number of CDM groups = 1, PDSCH-duration = 12

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 1 | 0 | 576 | 11 | 2 | 2216 | 12 | 10 ; 9 | 5760 | 10 | 23 |
| 40 | 1 | 1 | 608 | 14 | 1 | 2280 | 16 | 7 | 5888 | 14 | 19 |
| 48 | 1 | 2 | 640 | 15 | 1 | 2408 | 17 | 7 | 6016 | 17 | 17 ; 16 |
| 64 | 2 | 0 | 672 | 16 | 1 | 2472 | 15 | 8 | 6144 | 8 | 28 |
| 80 | 2 | 1 | 704 | 5 | 7 | 2536 | 6 | 19 | 6272 | 15 | 19 |
| 96 | 3 | 0 | 736 | 17 | 1 | 2600 | 16 | 8 | 6400 | 17 | 18 |
| 104 | 2 | 2 | 768 | 11 | 3 | 2664 | 13 | 11 | 6656 | 16 | 19 |
| 120 | 3 | 1 | 808 | 15 | 2 | 2728 | 15 | 10 ; 9 | 6784 | 11 | 24 |
| 128 | 4 | 0 | 848 | 16 | 2 | 2792 | 17 | 8 | 6912 | 15 | 20 |
| 136 | 1 | 7 | 888 | 17 | 2 | 2856 | 14 | 11 | 7040 | 17 | 19 |
| 152 | 3 | 2 | 928 | 11 | 4 | 2976 | 16 | 10 ; 9 | 7296 | 16 | 20 |
| 160 | 5 | 0 | 984 | 14 | 3 | 3104 | 17 | 10 ; 9 | 7424 | 15 | 21 |
| 168 | 4 | 1 | 1032 | 15 | 3 | 3240 | 16 | 11 | 7552 | 14 | 22 |
| 176 | 1 | 10 ; 9 | 1064 | 4 | 13 | 3368 | 14 | 12 | 7680 | 11 | 26 |
| 192 | 6 | 0 | 1128 | 16 | 3 | 3496 | 17 | 11 | 7808 | 17 | 20 |
| 208 | 5 | 1 | 1160 | 17 | 3 | 3624 | 12 | 14 | 7936 | 16 | 21 |
| 224 | 7 | 0 | 1192 | 5 | 12 | 3752 | 16 | 12 | 8064 | 15 | 22 |
| 240 | 2 | 6 | 1224 | 12 | 5 | 3840 | 13 | 14 | 8456 | 17 | 21 |
| 256 | 8 | 0 | 1256 | 15 | 4 | 3904 | 11 | 16 | 8712 | 16 | 22 |
| 272 | 4 | 3 | 1288 | 9 | 7 | 3968 | 17 | 12 | 9224 | 17 | 22 |
| 288 | 9 | 0 | 1320 | 13 | 5 | 4032 | 7 | 23 | 9480 | 13 | 27 |
| 304 | 7 | 1 | 1352 | 16 | 4 | 4096 | 14 | 14 | 9992 | 17 | 23 |
| 320 | 10 | 0 | 1416 | 17 | 4 | 4224 | 16 | 13 | 10248 | 14 | 27 |
| 336 | 8 | 1 | 1480 | 14 | 5 | 4352 | 13 | 15 | 10504 | 17 | 24 |
| 352 | 11 | 0 | 1544 | 15 | 5 | 4480 | 17 | 13 | 10760 | 14 | 28 |
| 368 | 7 | 2 | 1608 | 13 | 6 | 4608 | 14 | 15 | 11016 | 15 | 27 |
| 384 | 12 | 0 | 1672 | 16 | 5 | 4736 | 16 | 14 | 11272 | 17 | 25 |
| 408 | 6 | 3 | 1736 | 17 | 5 | 4864 | 13 | 18 | 11528 | 15 | 28 |
| 432 | 13 | 0 | 1800 | 11 | 8 | 4992 | 17 | 14 | 11784 | 16 | 27 |
| 456 | 14 | 0 | 1864 | 15 | 6 | 5120 | 7 | 27 | 12040 | 17 | 26 |
| 480 | 15 | 0 | 1928 | 5 | 18 | 5248 | 16 | 15 | 12296 | 16 | 28 |
| 504 | 12 | 1 | 2024 | 16 | 6 | 5376 | 13 | 19 | 12552 | 17 | 27 |
| 528 | 16 | 0 | 2088 | 17 | 6 | 5504 | 12 | 20 | 13064 | 17 | 28 |
| 552 | 17 | 0 | 2152 | 15 | 7 | 5632 | 17 | 15 |  |  |  |

### B.1.1.2	Downlink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 1, number of CDM groups = 1

Table B.1.1.2-1: Void

Table B.1.1.2-2: Void

Table B.1.1.2-3: TBS for PDSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 1, number of CDM groups = 1, PDSCH-duration = 12

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 1 | 0 | 552 | 11 | 2 | 2280 | 13 | 10 ; 9 | 6016 | 15 | 19 |
| 40 | 1 | 1 | 576 | 14 | 1 | 2408 | 17 | 7 | 6144 | 17 | 18 |
| 48 | 1 | 2 | 608 | 15 | 1 | 2472 | 16 | 8 | 6272 | 10 | 25 |
| 56 | 2 | 0 | 640 | 16 | 1 | 2536 | 13 | 11 | 6400 | 16 | 19 |
| 64 | 1 | 3 | 672 | 5 | 7 | 2600 | 9 | 14 | 6528 | 15 | 20 |
| 72 | 1 | 4 | 704 | 17 | 1 | 2664 | 17 | 8 | 6656 | 14 | 21 |
| 80 | 2 | 1 | 736 | 9 | 4 | 2728 | 14 | 11 | 6784 | 17 | 19 |
| 88 | 3 | 0 | 768 | 15 | 2 | 2792 | 16 | 10 ; 9 | 6912 | 11 | 25 |
| 96 | 2 | 2 | 808 | 16 | 2 | 2856 | 10 | 14 | 7040 | 16 | 20 |
| 112 | 1 | 6 | 848 | 17 | 2 | 2976 | 17 | 10 ; 9 | 7168 | 15 | 21 |
| 120 | 4 | 0 | 888 | 11 | 4 | 3104 | 16 | 11 | 7296 | 10 | 28 |
| 128 | 2 | 3 | 928 | 14 | 3 | 3240 | 11 | 14 | 7424 | 17 | 20 |
| 144 | 3 | 2 | 984 | 15 | 3 | 3368 | 17 | 11 | 7552 | 16 | 21 |
| 152 | 5 | 0 | 1032 | 16 | 3 | 3496 | 12 | 14 | 7680 | 15 | 22 |
| 160 | 4 | 1 | 1064 | 9 | 6 | 3624 | 16 | 12 | 7808 | 14 | 23 |
| 168 | 1 | 10 ; 9 | 1128 | 17 | 3 | 3752 | 13 | 14 | 8064 | 17 | 21 |
| 184 | 6 | 0 | 1160 | 10 | 6 | 3824 | 17 | 12 | 8192 | 16 | 22 |
| 192 | 4 | 2 | 1192 | 15 | 4 | 3840 | 7 | 23 | 8456 | 14 | 24 |
| 208 | 5 | 1 | 1224 | 9 | 7 | 3904 | 9 | 20 | 8712 | 17 | 22 |
| 224 | 7 | 0 | 1256 | 8 | 8 | 3968 | 14 | 14 | 8968 | 16 | 23 |
| 240 | 8 | 0 | 1288 | 16 | 4 | 4032 | 16 | 13 | 9224 | 13 | 27 |
| 256 | 5 | 2 | 1320 | 3 | 20 | 4096 | 13 | 15 | 9480 | 17 | 23 |
| 272 | 9 | 0 | 1352 | 17 | 4 | 4224 | 17 | 13 | 9736 | 14 | 27 |
| 288 | 7 | 1 | 1416 | 14 | 5 | 4352 | 13 | 17 ; 16 | 9992 | 15 | 26 |
| 304 | 10 | 0 | 1480 | 15 | 5 | 4480 | 16 | 14 | 10248 | 17 | 24 |
| 320 | 8 | 1 | 1544 | 13 | 6 | 4608 | 13 | 18 | 10504 | 15 | 27 |
| 336 | 11 | 0 | 1608 | 16 | 5 | 4736 | 15 | 15 | 10760 | 17 | 25 |
| 352 | 7 | 2 | 1672 | 17 | 5 | 4864 | 17 | 14 | 11016 | 15 | 28 |
| 368 | 12 | 0 | 1736 | 15 | 6 | 4992 | 14 | 18 | 11272 | 16 | 27 |
| 384 | 6 | 3 | 1800 | 13 | 7 | 5120 | 16 | 15 | 11528 | 17 | 26 |
| 408 | 13 | 0 | 1864 | 16 | 6 | 5248 | 12 | 20 | 11784 | 16 | 28 |
| 432 | 14 | 0 | 1928 | 14 | 7 | 5376 | 17 | 15 | 12040 | 17 | 27 |
| 456 | 11 | 1 | 2024 | 17 | 6 | 5504 | 10 | 23 | 12552 | 17 | 28 |
| 480 | 15 | 0 | 2088 | 15 | 7 | 5632 | 14 | 19 |  |  |  |
| 504 | 16 | 0 | 2152 | 16 | 7 | 5760 | 17 | 17 ; 16 |  |  |  |
| 528 | 17 | 0 | 2216 | 10 | 12 | 5888 | 10 | 24 |  |  |  |

### B.1.1.3	Downlink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 2, number of CDM groups = 2,
modulation order <= 2

NOTE:	The major purpose of the tables in this clause is to cope with PDSCH transmissions being restricted to Qm <= 2 (QPSK) like PDSCH transmissions being scheduled with P-RNTI, RA-RNTI, SI-RNTI (see TS 38.214 clause 5.1.3.1 [22]).

Table B.1.1.3-1: Void

Table B.1.1.3-2: Void

Table B.1.1.3-3: TBS for PDSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 2, number of CDM groups = 2, modulation order <= 2, PDSCH-duration = 12

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 1 | 0 | 128 | 4 | 1 | 256 | 4 | 4 | 504 | 4 | 8 |
| 32 | 1 | 1 | 136 | 1 | 9 | 272 | 5 | 3 | 552 | 7 | 5 |
| 40 | 1 | 2 | 144 | 6 | 0 | 288 | 7 | 2 | 576 | 6 | 6 |
| 48 | 2 | 0 | 152 | 3 | 3 | 320 | 6 | 3 | 640 | 5 | 8 |
| 64 | 2 | 1 | 160 | 5 | 1 | 336 | 3 | 7 | 672 | 7 | 6 |
| 72 | 3 | 0 | 176 | 7 | 0 | 368 | 7 | 3 | 736 | 5 | 9 |
| 80 | 2 | 2 | 184 | 2 | 6 | 384 | 6 | 4 | 768 | 6 | 8 |
| 88 | 1 | 6 | 192 | 6 | 1 | 408 | 5 | 5 | 808 | 7 | 7 |
| 96 | 4 | 0 | 208 | 5 | 2 | 432 | 3 | 9 | 888 | 7 | 8 |
| 104 | 2 | 3 | 224 | 7 | 1 | 456 | 7 | 4 | 1032 | 7 | 9 |
| 120 | 5 | 0 | 240 | 6 | 2 | 480 | 6 | 5 |  |  |  |

### B.1.1.4	Downlink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 2, number of CDM groups = 2

Table B.1.1.4-1: Void

Table B.1.1.4-2: Void

Table B.1.1.4-3: TBS for PDSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 2, number of CDM groups = 2, PDSCH-duration = 12

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 1 | 0 | 456 | 14 | 1 | 1800 | 16 | 7 | 4736 | 17 | 17 ; 16 |
| 32 | 1 | 1 | 480 | 9 | 3 | 1864 | 13 | 10 ; 9 | 4864 | 15 | 19 |
| 40 | 1 | 2 | 504 | 15 | 1 | 1928 | 17 | 7 | 4992 | 17 | 18 |
| 48 | 2 | 0 | 528 | 16 | 1 | 2024 | 16 | 8 | 5120 | 10 | 25 |
| 64 | 2 | 1 | 552 | 7 | 5 | 2088 | 13 | 11 | 5248 | 16 | 19 |
| 72 | 3 | 0 | 576 | 17 | 1 | 2152 | 17 | 8 | 5376 | 15 | 20 |
| 80 | 2 | 2 | 608 | 15 | 2 | 2216 | 14 | 11 | 5504 | 17 | 19 |
| 88 | 1 | 6 | 640 | 12 | 3 | 2280 | 16 | 10 ; 9 | 5760 | 16 | 20 |
| 96 | 4 | 0 | 672 | 16 | 2 | 2408 | 15 | 11 | 5888 | 14 | 22 |
| 104 | 2 | 3 | 704 | 17 | 2 | 2472 | 17 | 10 ; 9 | 6016 | 17 | 20 |
| 120 | 5 | 0 | 736 | 14 | 3 | 2536 | 16 | 11 | 6272 | 16 | 21 |
| 128 | 4 | 1 | 768 | 8 | 6 | 2600 | 14 | 12 | 6400 | 14 | 23 |
| 136 | 1 | 10 ; 9 | 808 | 15 | 3 | 2664 | 13 | 13 | 6528 | 11 | 28 |
| 144 | 6 | 0 | 848 | 16 | 3 | 2728 | 17 | 11 | 6656 | 17 | 21 |
| 152 | 3 | 3 | 888 | 11 | 5 | 2792 | 12 | 14 | 6784 | 15 | 23 |
| 160 | 5 | 1 | 928 | 17 | 3 | 2856 | 11 | 15 | 6912 | 12 | 27 |
| 176 | 7 | 0 | 984 | 15 | 4 | 2976 | 16 | 12 | 7168 | 17 | 22 |
| 184 | 2 | 6 | 1032 | 16 | 4 | 3104 | 17 | 12 | 7296 | 16 | 23 |
| 192 | 6 | 1 | 1064 | 11 | 6 | 3240 | 11 | 18 | 7424 | 13 | 27 |
| 208 | 8 | 0 | 1128 | 17 | 4 | 3368 | 16 | 13 | 7680 | 17 | 23 |
| 224 | 9 | 0 | 1160 | 12 | 6 | 3496 | 17 | 13 | 7808 | 16 | 24 |
| 240 | 6 | 2 | 1192 | 15 | 5 | 3624 | 14 | 15 | 8064 | 14 | 27 |
| 256 | 10 | 0 | 1224 | 13 | 6 | 3752 | 16 | 14 | 8192 | 17 | 24 |
| 272 | 11 | 0 | 1256 | 6 | 13 | 3824 | 9 | 22 | 8456 | 14 | 28 |
| 288 | 7 | 2 | 1288 | 16 | 5 | 3840 | 14 | 17 ; 16 | 8712 | 17 | 25 |
| 304 | 12 | 0 | 1320 | 14 | 6 | 3904 | 15 | 15 | 8968 | 15 | 28 |
| 320 | 8 | 2 | 1352 | 17 | 5 | 3968 | 17 | 14 | 9224 | 16 | 27 |
| 336 | 13 | 0 | 1416 | 15 | 6 | 4032 | 7 | 27 | 9480 | 17 | 26 |
| 352 | 14 | 0 | 1480 | 13 | 7 | 4096 | 16 | 15 | 9736 | 17 | 27 |
| 368 | 11 | 1 | 1544 | 16 | 6 | 4224 | 13 | 19 | 10248 | 17 | 28 |
| 384 | 15 | 0 | 1608 | 17 | 6 | 4352 | 17 | 15 |  |  |  |
| 408 | 16 | 0 | 1672 | 15 | 7 | 4480 | 16 | 16 |  |  |  |
| 432 | 17 | 0 | 1736 | 12 | 10 ; 9 | 4608 | 14 | 19 |  |  |  |

## B.1.2	Void

# B.2	Uplink TBS (informative)

The tables in this clause are depending on parameters provided by RRC signalling as described in subclause 7.1.2.3.2. LRBs is limited to the minimum value of NBWP=24 in accordance to Table 5.3.2-1 of TS 38.101-1/2 [5, 6].

For selection of the LRBs / IMCS pair for a particular TBS, the same criteria are applied, as specified for the DL in subclause 7.1.2.2.4.2 paragraph ‘LRBs / IMCS pair determination’.

## B.2.1	Uplink TBS using MCS index table 5.1.3.1-1

### B.2.1.1	Uplink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 0, number of CDM groups = 1

Table B.2.1.1-1: TBS for PUSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 0, number of CDM groups = 1, PUSCH-duration = 14

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 1 | 0 | 848 | 23 | 0 | 3496 | 19 | 8 | 8192 | 22 | 15 |
| 40 | 1 | 1 | 888 | 24 | 0 | 3624 | 22 | 7 | 8456 | 21 | 17 ; 16 |
| 56 | 1 | 2 | 928 | 19 | 1 | 3752 | 23 | 7 | 8712 | 23 | 15 |
| 72 | 2 | 0 | 984 | 20 | 1 | 3824 | 8 | 19 | 8968 | 24 | 15 |
| 88 | 2 | 1 | 1032 | 21 | 1 | 3840 | 24 | 7 | 9224 | 23 | 17 ; 16 |
| 104 | 3 | 0 | 1064 | 22 | 1 | 3904 | 19 | 10 ; 9 | 9480 | 24 | 17 ; 16 |
| 112 | 2 | 2 | 1128 | 23 | 1 | 3968 | 15 | 12 | 9736 | 23 | 18 |
| 136 | 3 | 1 | 1160 | 24 | 1 | 4032 | 22 | 8 | 9992 | 21 | 19 |
| 144 | 4 | 0 | 1192 | 20 | 2 | 4096 | 20 | 10 ; 9 | 10248 | 24 | 18 |
| 152 | 2 | 3 | 1224 | 16 | 3 | 4224 | 23 | 8 | 10504 | 22 | 19 |
| 160 | 1 | 7 | 1256 | 21 | 2 | 4352 | 24 | 8 | 10760 | 23 | 19 |
| 176 | 5 | 0 | 1288 | 22 | 2 | 4480 | 22 | 10 ; 9 | 11016 | 18 | 22 |
| 184 | 4 | 1 | 1320 | 17 | 3 | 4608 | 20 | 11 | 11272 | 24 | 19 |
| 208 | 1 | 10 ; 9 | 1352 | 23 | 2 | 4736 | 23 | 10 ; 9 | 11528 | 19 | 22 |
| 224 | 6 | 0 | 1416 | 24 | 2 | 4864 | 21 | 11 | 11784 | 23 | 20 |
| 240 | 5 | 1 | 1480 | 19 | 3 | 4992 | 24 | 10 ; 9 | 12040 | 20 | 22 |
| 256 | 7 | 0 | 1544 | 20 | 3 | 5120 | 13 | 17 | 12296 | 24 | 20 |
| 272 | 2 | 6 | 1608 | 21 | 3 | 5248 | 23 | 11 | 12552 | 19 | 23 |
| 288 | 8 | 0 | 1672 | 14 | 5 | 5376 | 18 | 13 | 12808 | 23 | 21 |
| 304 | 4 | 3 | 1736 | 22 | 3 | 5504 | 24 | 11 | 13064 | 20 | 23 |
| 320 | 2 | 7 | 1800 | 23 | 3 | 5632 | 19 | 13 | 13320 | 22 | 22 |
| 336 | 9 | 0 | 1864 | 24 | 3 | 5760 | 22 | 12 | 13576 | 24 | 21 |
| 352 | 6 | 2 | 1928 | 20 | 4 | 5888 | 20 | 13 | 13832 | 21 | 23 |
| 368 | 10 | 0 | 2024 | 21 | 4 | 6016 | 23 | 12 | 14088 | 23 | 22 |
| 384 | 8 | 1 | 2088 | 22 | 4 | 6144 | 13 | 19 | 14344 | 22 | 23 |
| 408 | 11 | 0 | 2152 | 23 | 4 | 6272 | 24 | 12 | 14600 | 24 | 22 |
| 432 | 12 | 0 | 2216 | 19 | 5 | 6400 | 19 | 14 | 14856 | 21 | 24 |
| 456 | 6 | 3 | 2280 | 24 | 4 | 6528 | 22 | 13 | 15112 | 23 | 23 |
| 480 | 13 | 0 | 2408 | 21 | 5 | 6656 | 20 | 14 | 15624 | 22 | 24 |
| 504 | 14 | 0 | 2472 | 18 | 6 | 6784 | 23 | 13 | 15880 | 24 | 23 |
| 528 | 11 | 1 | 2536 | 22 | 5 | 6912 | 8 | 28 | 16136 | 23 | 24 |
| 552 | 15 | 0 | 2600 | 19 | 6 | 7040 | 21 | 14 | 16392 | 22 | 25 |
| 576 | 12 | 1 | 2664 | 23 | 5 | 7168 | 24 | 13 | 16896 | 24 | 24 |
| 608 | 16 | 0 | 2728 | 20 | 6 | 7296 | 13 | 21 | 17424 | 23 | 25 |
| 640 | 17 | 0 | 2792 | 24 | 5 | 7424 | 22 | 14 | 17928 | 24 | 25 |
| 672 | 18 | 0 | 2856 | 21 | 6 | 7552 | 19 | 17 ; 16 | 18432 | 23 | 26 |
| 704 | 19 | 0 | 2976 | 18 | 7 | 7680 | 23 | 14 | 18960 | 24 | 26 |
| 736 | 20 | 0 | 3104 | 22 | 6 | 7808 | 21 | 15 | 19968 | 24 | 27 |
| 768 | 21 | 0 | 3240 | 23 | 6 | 7936 | 20 | 17 ; 16 | 21000 | 24 | 28 |
| 808 | 22 | 0 | 3368 | 24 | 6 | 8064 | 24 | 14 |  |  |  |

Table B.2.1.1-2: Void

### B.2.1.2	Uplink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 1, number of CDM groups = 1

Table B.2.1.2-1: TBS for PUSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 1, number of CDM groups = 1, PUSCH-duration = 14

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 1 | 0 | 848 | 23 | 0 | 3496 | 19 | 8 | 8192 | 22 | 15 |
| 40 | 1 | 1 | 888 | 24 | 0 | 3624 | 22 | 7 | 8456 | 21 | 17 ; 16 |
| 56 | 1 | 2 | 928 | 19 | 1 | 3752 | 23 | 7 | 8712 | 23 | 15 |
| 72 | 2 | 0 | 984 | 20 | 1 | 3824 | 8 | 19 | 8968 | 24 | 15 |
| 88 | 2 | 1 | 1032 | 21 | 1 | 3840 | 24 | 7 | 9224 | 23 | 17 ; 16 |
| 104 | 3 | 0 | 1064 | 22 | 1 | 3904 | 19 | 10 ; 9 | 9480 | 24 | 17 ; 16 |
| 112 | 2 | 2 | 1128 | 23 | 1 | 3968 | 15 | 12 | 9736 | 23 | 18 |
| 136 | 3 | 1 | 1160 | 24 | 1 | 4032 | 22 | 8 | 9992 | 21 | 19 |
| 144 | 4 | 0 | 1192 | 20 | 2 | 4096 | 20 | 10 ; 9 | 10248 | 24 | 18 |
| 152 | 2 | 3 | 1224 | 16 | 3 | 4224 | 23 | 8 | 10504 | 22 | 19 |
| 160 | 1 | 7 | 1256 | 21 | 2 | 4352 | 24 | 8 | 10760 | 23 | 19 |
| 176 | 5 | 0 | 1288 | 22 | 2 | 4480 | 22 | 10 ; 9 | 11016 | 18 | 22 |
| 184 | 4 | 1 | 1320 | 17 | 3 | 4608 | 20 | 11 | 11272 | 24 | 19 |
| 208 | 1 | 10 ; 9 | 1352 | 23 | 2 | 4736 | 23 | 10 ; 9 | 11528 | 19 | 22 |
| 224 | 6 | 0 | 1416 | 24 | 2 | 4864 | 21 | 11 | 11784 | 23 | 20 |
| 240 | 5 | 1 | 1480 | 19 | 3 | 4992 | 24 | 10 ; 9 | 12040 | 20 | 22 |
| 256 | 7 | 0 | 1544 | 20 | 3 | 5120 | 13 | 17 | 12296 | 24 | 20 |
| 272 | 2 | 6 | 1608 | 21 | 3 | 5248 | 23 | 11 | 12552 | 19 | 23 |
| 288 | 8 | 0 | 1672 | 14 | 5 | 5376 | 18 | 13 | 12808 | 23 | 21 |
| 304 | 4 | 3 | 1736 | 22 | 3 | 5504 | 24 | 11 | 13064 | 20 | 23 |
| 320 | 2 | 7 | 1800 | 23 | 3 | 5632 | 19 | 13 | 13320 | 22 | 22 |
| 336 | 9 | 0 | 1864 | 24 | 3 | 5760 | 22 | 12 | 13576 | 24 | 21 |
| 352 | 6 | 2 | 1928 | 20 | 4 | 5888 | 20 | 13 | 13832 | 21 | 23 |
| 368 | 10 | 0 | 2024 | 21 | 4 | 6016 | 23 | 12 | 14088 | 23 | 22 |
| 384 | 8 | 1 | 2088 | 22 | 4 | 6144 | 13 | 19 | 14344 | 22 | 23 |
| 408 | 11 | 0 | 2152 | 23 | 4 | 6272 | 24 | 12 | 14600 | 24 | 22 |
| 432 | 12 | 0 | 2216 | 19 | 5 | 6400 | 19 | 14 | 14856 | 21 | 24 |
| 456 | 6 | 3 | 2280 | 24 | 4 | 6528 | 22 | 13 | 15112 | 23 | 23 |
| 480 | 13 | 0 | 2408 | 21 | 5 | 6656 | 20 | 14 | 15624 | 22 | 24 |
| 504 | 14 | 0 | 2472 | 18 | 6 | 6784 | 23 | 13 | 15880 | 24 | 23 |
| 528 | 11 | 1 | 2536 | 22 | 5 | 6912 | 8 | 28 | 16136 | 23 | 24 |
| 552 | 15 | 0 | 2600 | 19 | 6 | 7040 | 21 | 14 | 16392 | 22 | 25 |
| 576 | 12 | 1 | 2664 | 23 | 5 | 7168 | 24 | 13 | 16896 | 24 | 24 |
| 608 | 16 | 0 | 2728 | 20 | 6 | 7296 | 13 | 21 | 17424 | 23 | 25 |
| 640 | 17 | 0 | 2792 | 24 | 5 | 7424 | 22 | 14 | 17928 | 24 | 25 |
| 672 | 18 | 0 | 2856 | 21 | 6 | 7552 | 19 | 17 ; 16 | 18432 | 23 | 26 |
| 704 | 19 | 0 | 2976 | 18 | 7 | 7680 | 23 | 14 | 18960 | 24 | 26 |
| 736 | 20 | 0 | 3104 | 22 | 6 | 7808 | 21 | 15 | 19968 | 24 | 27 |
| 768 | 21 | 0 | 3240 | 23 | 6 | 7936 | 20 | 17 ; 16 | 21000 | 24 | 28 |
| 808 | 22 | 0 | 3368 | 24 | 6 | 8064 | 24 | 14 |  |  |  |

Table B.2.1.2-2: Void

### B.2.1.3	Void

### B.2.1.4	Void

### B.2.1.5	Uplink TBS using MCS index table 5.1.3.1-1,
dmrs-AdditionalPosition = 2, number of CDM groups = 2

Table B.2.1.5-1: TBS for PUSCH using MCS index table 5.1.3.1-1 with dmrs-AdditionalPosition = 2, number of CDM groups = 2, PUSCH-duration = 14

| TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS | TBS | LRBs | IMCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 1 | 0 | 672 | 21 | 0 | 2856 | 21 | 7 | 7552 | 24 | 15 |
| 40 | 1 | 1 | 704 | 23 | 0 | 2976 | 22 | 7 | 7680 | 15 | 22 |
| 48 | 1 | 2 | 736 | 24 | 0 | 3104 | 23 | 7 | 7808 | 23 | 17 ; 16 |
| 56 | 2 | 0 | 768 | 19 | 1 | 3240 | 24 | 7 | 7936 | 22 | 18 |
| 64 | 1 | 3 | 808 | 20 | 1 | 3368 | 19 | 10 ; 9 | 8064 | 24 | 17 ; 16 |
| 72 | 1 | 4 | 848 | 21 | 1 | 3496 | 22 | 8 | 8192 | 23 | 18 |
| 80 | 2 | 1 | 888 | 22 | 1 | 3624 | 23 | 8 | 8456 | 21 | 19 |
| 88 | 3 | 0 | 928 | 23 | 1 | 3752 | 24 | 8 | 8712 | 24 | 18 |
| 96 | 2 | 2 | 984 | 24 | 1 | 3824 | 17 | 12 | 8968 | 19 | 21 |
| 112 | 1 | 6 | 1032 | 20 | 2 | 3840 | 22 | 10 ; 9 | 9224 | 23 | 19 |
| 120 | 4 | 0 | 1064 | 21 | 2 | 3904 | 20 | 11 | 9480 | 24 | 19 |
| 128 | 2 | 3 | 1128 | 22 | 2 | 3968 | 14 | 14 | 9736 | 22 | 20 |
| 144 | 3 | 2 | 1160 | 23 | 2 | 4032 | 23 | 10 ; 9 | 9992 | 23 | 20 |
| 152 | 5 | 0 | 1192 | 24 | 2 | 4096 | 21 | 11 | 10248 | 20 | 22 |
| 160 | 4 | 1 | 1224 | 19 | 3 | 4224 | 24 | 10 ; 9 | 10504 | 24 | 20 |
| 168 | 1 | 10 ; 9 | 1256 | 8 | 8 | 4352 | 13 | 17 ; 16 | 10760 | 21 | 22 |
| 184 | 6 | 0 | 1288 | 20 | 3 | 4480 | 23 | 11 | 11016 | 23 | 21 |
| 192 | 4 | 2 | 1320 | 3 | 20 | 4608 | 24 | 11 | 11272 | 22 | 22 |
| 208 | 5 | 1 | 1352 | 21 | 3 | 4736 | 21 | 12 | 11528 | 24 | 21 |
| 224 | 7 | 0 | 1416 | 22 | 3 | 4864 | 22 | 12 | 11784 | 23 | 22 |
| 240 | 8 | 0 | 1480 | 18 | 4 | 4992 | 20 | 13 | 12040 | 20 | 24 |
| 256 | 5 | 2 | 1544 | 23 | 3 | 5120 | 23 | 12 | 12296 | 24 | 22 |
| 272 | 9 | 0 | 1608 | 24 | 3 | 5248 | 21 | 13 | 12552 | 21 | 24 |
| 288 | 7 | 1 | 1672 | 21 | 4 | 5376 | 24 | 12 | 12808 | 23 | 23 |
| 304 | 10 | 0 | 1736 | 15 | 6 | 5504 | 22 | 13 | 13064 | 22 | 24 |
| 320 | 8 | 1 | 1800 | 22 | 4 | 5632 | 20 | 14 | 13320 | 24 | 23 |
| 336 | 11 | 0 | 1864 | 23 | 4 | 5760 | 23 | 13 | 13576 | 20 | 26 |
| 352 | 7 | 2 | 1928 | 24 | 4 | 5888 | 10 | 24 | 13832 | 23 | 24 |
| 368 | 12 | 0 | 2024 | 20 | 5 | 6016 | 24 | 13 | 14088 | 22 | 25 |
| 384 | 6 | 3 | 2088 | 21 | 5 | 6144 | 18 | 16 | 14344 | 24 | 24 |
| 408 | 13 | 0 | 2152 | 22 | 5 | 6272 | 22 | 14 | 14600 | 23 | 25 |
| 432 | 14 | 0 | 2216 | 19 | 6 | 6400 | 19 | 17 ; 16 | 14856 | 22 | 26 |
| 456 | 11 | 1 | 2280 | 23 | 5 | 6528 | 23 | 14 | 15368 | 24 | 25 |
| 480 | 15 | 0 | 2408 | 24 | 5 | 6656 | 21 | 15 | 15624 | 23 | 26 |
| 504 | 16 | 0 | 2472 | 18 | 7 | 6784 | 24 | 14 | 16136 | 24 | 26 |
| 528 | 17 | 0 | 2536 | 22 | 6 | 6912 | 22 | 15 | 16896 | 24 | 27 |
| 552 | 18 | 0 | 2600 | 19 | 7 | 7040 | 21 | 17 ; 16 | 17424 | 24 | 28 |
| 576 | 14 | 1 | 2664 | 23 | 6 | 7168 | 20 | 18 |  |  |  |
| 608 | 19 | 0 | 2728 | 20 | 7 | 7296 | 23 | 15 |  |  |  |
| 640 | 20 | 0 | 2792 | 24 | 6 | 7424 | 22 | 17 ; 16 |  |  |  |

Table B.2.1.5-2: Void

## B.2.2	Void

## B.2.3	Void