3GPP TSG RAN WG5 Meeting #50bis	 R5-111019r3

# Munich, Germany,

# 6 - 7 April 2011

| Title: | Discussion and text proposal for LTE Test Procedures |
| --- | --- |
| Source: | NTT DOCOMO, INC. |
| Agenda Item: | 5.3 |
| Document for: | Discussion and Endorsement |

1	Introduction

During RAN#50, the Study Item for UE Application Layer Data Throughput Perfomance was approved in [1]. This document provides discussion and text proposal for the basic LTE FDD test procedures for TR 37.901 [2].

2	Summary

The LTE FDD test procedures listed in the following text proposal are considered to be in-line with the goals and objectives of the Study Item for UE Application Layer Data Throughput Perfomance. The test procedures are a mix of downlink and uplink data performance test procedures using either FTP or UDP transfer protocols, which taking the same approach for HSPA test procedures discussed in [3]. The test procedures are based on the performance test cases for LTE specified in TS 36.521-1 [4] with the appropriate modifications to align with the practical environment and address the purpose of UE Application Layer Data Throughput Performance. The general test parameters listed in Table 1 are selected based on the following technical backgrounds:

- Transmission mode: the test procedures shall apply for all possible transmition modes to cover the general LTE network operations, e.g. SIMO, Transmit Diversity, Open loop MIMO and Closed loop MIMO.

- Fading model, Signal-Noise-Ratio (SNR), MIMO correlation: these parameters are based on [5] to test throughput performance in  typical environment.

Table 2.1.1: Test conditions and parameters for LTE FDD test procedures

| Basic conditions | Downlink | Uplink |
| --- | --- | --- |
| Transmission mode | SIMO, Transmit Diversity, Open loop MIMO, Closed loop MIMO | SISO |
| Fading model | Static, EPA5, EVA5, ETU70, ETU300 | Static |
| Signal-Noise-Ratio | 0dB, 10dB, 20dB | ∞ |
| MIMO correlation | 0(Low), 0.5(Mid) | N/A |

3	Proposal

It is proposed to consider the following text proposal as being a suitable starting point for the technical content for UE Application Layer Data Throughput Performance test procedures for LTE FDD.

4	References

[1] RP-101422 – “Study Item Proposal: UE Application Layer Data Throughput Performance”

[2] TR 37.901 - “UE Application Layer Data Throughput Performance Study Item Technical Report”

[3] R5-110854 - “TP for TR 37.901 on UTRA/UTRAN Test Procedure Structure and Content”

[4] TS 36.521-1 –

[5]R5-111014r3 – “Test conditions for the UE application-layer data throughput measurements”

[6]TS 36.211

[7]TS 36.508

A.3	UE Application Layer Data Throughput Performance Test Procedures for LTE – Tethered

A.3.1	General

The propagation conditions are specified in Annex B of TS 36.521-1 [4] and the Down link Physical channels are specified in Annex C TS 36.521-1 [xx]. Unless stated otherwise, DL power control is OFF.

The common RF test conditions of Performance requirements are defined in clause A.3 of TS 36.121-1 [4], and each test conditions in clause 9 should refer to clause A.5 of TS 36.521-1 [4]. Individual test conditions are defined in the paragraph of each test.

All throughput measurements in Annex A.3 shall be performed according to the following general rules:

-	The measured UE Application Layer Throughput, T, is defined as the the number of user bits per unit of time forwarded by the network from a certain source address to a certain destination, excluding protocol overhead and retransmitted data packets.

-	Minimum test time is necessary for both static and multipath fading profiles to ensure stabalization of the FTP or UDP Application throughput.  The following table provides statistical accuracy and simplifies the process by breaking it down into Static and faded profiles where the faded profile duration is sufficient for all faded standard fade models.

Table A.3.1-1: Transfer Duration Times

| Profile | Minimum Test Time |
| --- | --- |
| Static | 60s |
| Faded (EVA5, ETU70, ETU300) | FFS |

-	FTP file size shall be selected according to the test purpose and  receive design to meet the minimum test times.

-	UDP blast duration shall be selected to meet the minimum test times using a sufficient rate to prevent physical layer DTX based upon the UE Category.

The performance characteristics for a particular UE will be determined by all possible LTE transmition modes, i.e. SIMO, Transmit Diversity, Open loop MIMO and Closed loop MIMO. Results shall be recorded for the following sections which characterize the UE performance across a variety of application layer scenarios utilizing both FTP and UDP protocols.

-	FTP Downlink Performance

-	UDP Downlink Performance

-	FTP Uplink Performance

-	UDP Uplink Performance

IP header compression will not be used.

Ciphering and integrity of RRC and NAS messages and user plane data will be set to null for UDP and AES for FTP to check CPU processing ability of UE.. Refer to TS 33.401 [xx].

The recorded UE throughput for the FTP and UDP Downlink and Uplink Performance test procedures shall be based upon the simple average of 5 transfers for each test condition.

Unless otherwise stated, the UE output power for the tests shall be set0 dBm with ±2dBm torelance.

Unless stated otherwise, power control of the Downlink is OFF.

The physical channels specified in TS 36.211 [6] clause 6 (for downlink physical channels) shall be as defined in the respective sections below.

Unless otherwise specified, the downlink reference measurement channels are specified in Annex A of TS 36.521-1 [4],

The requirement for a UE that support E-UTRA in downlink shall be tested according to the declared UE PDSCH category.

The fading of the signals and the AWGN signals applied to each receiver antenna connector shall be uncorrelated. The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective test cases.

The test procedures defined herein are based on UE(s) that utilize a dual-antenna receiver.

For all test procedures, the SNR is defined as:

,

where the superscript indicates the receiver antenna connector. The SNR requirement applies for the UE categories given for each test.

The parameters specified in Table A.3.1-2 are valid for all FDD tests unless otherwise stated.

Table A.3.1-2: Common Test Parameters (FDD)

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| Inter-TTI Distance |  | 1 |  |
| Number of HARQ processes | Processes | 8 | For FDD, 8 HARQ processes in the DL, as specified in TS 36.213 [xx] clause 7. All 8 HARQ processes are used. |
| Scheduling of retransmissions |  |  | 1. Retransmissions use the same Transport Block Size (TBS) as the initial transmission. 2. HARQ processes are scheduled consecutively, independent of the fact, whether retransmissions (for negatively acknowledged HARQ processes) or new transmissions (for positively acknowledged HARQ processes) occur. |
| Maximum number of HARQ transmission |  | 4 | It is always 4 for FDD, as specified in TS 36.213 [xx] clause 8 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |  |
| Number of OFDM symbols for PDCCH | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths | The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [xx] clause 6.7 |
| Cyclic Prefix |  | Normal | CP consist of the following physical resource blocks (RBs) parameters: 12 consecutive subcarriers at a 15 kHz spacing and 7 OFDM symbols, as specified in TS 36.211 [xx] clause 6.2.3 |
| Cell ID |  | 0 | The Cell ID is uniquely defined by a number in the range of 0 to 503, representing the physical-layer cell identity, as specified in TS 36.211 [xx] clause 6.11. |

A.3.2	FTP Downlink Performance

A.3.2.1	PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols)

A.3.2.1.1	Definition

The UE application layer downlink performance for FTP under different multi-path fading environments is determined by the UE application layer FTP throughput T.

A.3.2.1.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.2.1.3	Test Parameters

Test parameters are defined in Table A.3.2.1.3-1 and A.3.2.1.3-2.

Table A.3.2.1.3-1: Test Parameters for FTP Downlink Testing

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0(1 Tx) |
|  |  | dB | 0(1 Tx)(Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.2.1.3-2: Test points for SISO Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | N/A |
| 3 | EVA5 | 10 | N/A |
| 4 | ETU70 | 0 | N/A |
| 5 | ETU300 | 0 | N/A |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.2.1.4	Test Description

A.3.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.9 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a FTP server.

3)	Tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.X.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.2.1.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables A.3.2.1.3-1 and A.3.2.1.3-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.2.1.3-2, as appropriate.

3)	Using the FTP client, begin FTP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.2.1.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.2.1.3-2, as appropriate.

### A.3.2.2	PDSCH Transmit Diversity Performance (Cell-Specific Reference Symbols)

A.3.2.2.1	Definition

The UE application layer downlink performance for FTP under different multi-path fading environments is determined by the UE application layer FTP throughput T.

A.3.2.2.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.2.2.3	Test Parameters

Test parameters are defined in Table A.3.2.3-1 and A.3.2.3-2.

Table A.3.2.2.3-1: Test Parameters for TxDiversity Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.2.2.3-2: Test points for TxDiversity Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.2.2.4	Test Description

A.3.2.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a FTP server.

3)	Tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.2.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.2.2.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables A.3.2.2.3-1 and A.3.2.2.3-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.2.2.3-2, as appropriate.

3)	Using the FTP client, begin FTP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.2.2.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.2.2.3-2, as appropriate.

A.3.2.3	PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols)

A.3.2.3.1	Definition

The UE application layer downlink performance for FTP under different multi-path fading environments is determined by the UE application layer FTP throughput T.

A.3.2.3.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.2.3.3	Test Parameters

Test parameters are defined in Table A.3.2.3-1 and A.3.2.3-2.

Table A.3.2.3.3-1: Test Parameters for Open Loop Spatial Multiplexing Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.2.3.3-2: Test points for Open Loop Spatial Multiplexing Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.2.3.4	Test Description

A.3.2.3.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a FTP server.

3)	Tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.X.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.2.3.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2A for C_RNTI to transmit the DL RMC according to Tables A. 3.2.3.3-1 and A3.2.3.3-2

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A. 3.2.3.3-2, as appropriate.

3)	Using the FTP client, begin FTP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.2.3.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.2.3.3-2, as appropriate.

A.3.2.4	PDSCH Closed Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols)

A.3.2.4.1	Definition

The UE application layer downlink performance for FTP under different multi-path fading environments is determined by the UE application layer FTP throughput T.

A.3.2.4.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.2.4.3	Test Parameters

Test parameters are defined in Table A.3.2.4.3-1 and A.3.2.4.3-2.

Table A.3.2.4.3-1: Test Parameters for TxDiversity Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.2.4.3-2: Test points for TxDiversity Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.2.4	Test Description

A.3.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a FTP server.

3)	Tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.2.4.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.2.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables A.3.2.3.4-1 and A.3.2.3.4-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.2.4.3-2, as appropriate.

3)	Using the FTP client, begin FTP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.2.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.2.3.4-2, as appropriate.

A.3.3	UDP Downlink Performance

A.3.3.1	PDSCH Single Antenna Port Performance (Cell-Specific Reference Symbols)

A.3.3.1.1	Definition

The UE application layer downlink performance for UDP under different multi-path fading environments is determined by the UE application layer UDP throughput T.

A.3.3.1.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.3.1.3	Test Parameters

Test parameters are defined in Table A.3.3.1.3-1 and A.3.3.1.3-2.

Table A.3.3.1.3-1: Test Parameters for UDP Downlink Testing

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0(1 Tx) |
|  |  | dB | 0(1 Tx)(Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.3.1.3-2: Test points for SISO Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | N/A |
| 3 | EVA5 | 10 | N/A |
| 4 | ETU70 | 0 | N/A |
| 5 | ETU300 | 0 | N/A |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.3.1.4	Test Description

A.3.3.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.9 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a UDP server.

3)	Tether the UE to a laptop configured with UDP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.3.1.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.3.1.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables A.3.3.1.3-1 and A.3.3.1.3-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.3.1.3-2, as appropriate.

3)	Using the UDP client, begin UDP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.3.1.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.3.1.3-2, as appropriate.

### A.3.3.2	PDSCH Transmit Diversity Performance (Cell-Specific Reference Symbols)

A.3.3.2.1	Definition

The UE application layer downlink performance for UDP under different multi-path fading environments is determined by the UE application layer UDP throughput T.

A.3.3.2.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.3.2.3	Test Parameters

Test parameters are defined in Table A.3.3.3-1 and A.3.3.3-2.

Table A.3.3.2.3-1: Test Parameters for TxDiversity Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.3.2.3-2: Test points for TxDiversity Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.3.2.4	Test Description

A.3.3.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a UDP server.

3)	Tether the UE to a laptop configured with UDP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.3.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.3.2.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables A.3.2.2.3-1 and A.3.2.2.3-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.3.2.3-2, as appropriate.

3)	Using the UDP client, begin UDP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.3.2.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.3.2.3-2, as appropriate.

A.3.3.3	PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols)

A.3.3.3.1	Definition

The UE application layer downlink performance for UDP under different multi-path fading environments is determined by the UE application layer UDP throughput T.

A.3.3.3.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.3.3.3	Test Parameters

Test parameters are defined in Table A.3.3.3-1 and A.3.3.3-2.

Table A.3.3.3.3-1: Test Parameters for Open Loop Spatial Multiplexing Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.3.3.3-2: Test points for Open Loop Spatial Multiplexing Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.3.3.4	Test Description

A.3.3.3.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a UDP server.

3)	Tether the UE to a laptop configured with UDP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.3.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.2.3.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2A for C_RNTI to transmit the DL RMC according to Tables A. 3.2.3.3-1 and A3.2.3.3-2

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A. 3.3.3.3-2, as appropriate.

3)	Using the UDP client, begin UDP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.3.3.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.3.3.3-2, as appropriate.

A.3.3.4	PDSCH Closed Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols)

A.3.3.4.1	Definition

The UE application layer downlink performance for UDP under different multi-path fading environments is determined by the UE application layer UDP throughput T.

A.3.3.4.2	Test Purpose

To measure the performance of the UE while downloading TCP based data within both static and multi path faded environments.

A.3.3.4.3	Test Parameters

Test parameters are defined in Table A.3.3.4.3-1 and A.3.3.4.3-2.

Table A.3.3.4.3-1: Test Parameters for TxDiversity Downlink Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: |  |  |  |

Table A.3.3.4.3-2: Test points for TxDiversity Downlink Testing

| Initial conditions |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz Note1 |  |  |
| Test pramaters for each bandwidth |  |  |  |
| Test No | Propagation Conditions | Ior/Ioc | Correlation |
| 1 | Static | 20 | N/A |
| 2 | EPA5 | 20 | Mid |
| 3 | EVA5 | 10 | Mid |
| 4 | ETU70 | 0 | Low |
| 5 | ETU300 | 0 | Low |
| Note 1: Bandwidth depends on Band No. |  |  |  |

A.3.3.4	Test Description

A.3.3.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a UDP server.

3)	Tether the UE to a laptop configured with UDP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.3.4.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.3.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables A.3.3.3.4-1 and A.3.3.3.4-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.3.4.3-2, as appropriate.

3)	Using the UDP client, begin UDP download from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PUSCH/PUCCH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.3.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.3.3.4-2, as appropriate.

A.3.4	FTP Uplink Performance

A.3.4.1	PUSCH Single Antenna Port Performance

A.3.4.1.1	Definition

The UE application layer uplink performance for FTP under Static environment is determined by the UE application layer FTP throughput T.

A.3.4.1.2	Test Purpose

To measure the performance of the UE while uploading FTP based data with static environment while transmitting on single antenna ports.

A.3.4.1.3	Test Parameters

Test parameters are defined in Table A.4.2.3.

Table A.3.4.3: Test points

| Initial conditions |  |  |
| --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| SNR | ∞ for Static |  |
| Test pramaters for each bandwidth and SNR |  |  |
| Test No | Propagation Conditions | Correlation |
| 1 | Static | N/A |

A.3.4.1.4	Test Description

A.3.4.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [4].

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a FTP server.

3)	Tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.4.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [4] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [4].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [4].

A.3.4.1.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2A for C_RNTI to transmit the DL RMC according to Tables A.3.4.3.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.4.2.3-2, as appropriate.

3)	Using the FTP client, begin FTP ipload from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PHICH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.4.2.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.4.2.3-2, as appropriate.

A.3.5	UDP Uplink Performance

A.3.5.1	PUSCH Single Antenna Port Performance

A.3.5.1.1	Definition

The UE application layer uplink performance for UDP under Static environment is determined by the UE application layer UDP throughput T.

A.3.5.1.2	Test Purpose

To measure the performance of the UE while uploading UDP based data with static environment while transmitting on single antenna ports.

A.3.5.1.3	Test Parameters

Test parameters are defined in Table A.3.2.3-1 and A.3.2.3-2.

Table A.3.4.3: Test points

| Initial conditions |  |  |
| --- | --- | --- |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| SNR | ∞ for Static |  |
| Test pramaters for each bandwidth and SNR |  |  |
| Test No | Propagation Conditions | Correlation |
| 1 | Static | N/A |

A.3.2.4	Test Description

A.3.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [xx].

Test Environment: Normal, as defined in TS 36.508 [xx] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [xx] clause 4.3.1.1.

1)	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [xx] Annex A, Figure A.10 for antenna configuration 2x2.

2)	Connect an application server to the IP output of the SS configured with a UDP server.

3)	Tether the UE to a laptop configured with UDP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

4)	The parameter settings for the cell are set up according to Tables A.3.1-2 and A.3.X.2.3-1 as appropriate.

5)	Downlink signals are initially set up according to Annex C0, C.1 and Annex C.3.2 in TS 36.521-1 [xx] and uplink signals according to Annex H.1 and H.3.2 in TS 36.521-1 [xx].

6)	Propagation conditions are set according to Annex B.0 in TS 36.521-1 [xx].

A.3.2.4.2	Procedure

1)	SS transmits PDSCH via PDCCH DCI format 2A for C_RNTI to transmit the DL RMC according to Tables A.3.X.2.3-1 and A.3.X.2.3-2.

2)	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table A.3.X.2.3-2, as appropriate.

3)	Using the UDP client, begin UDP upload from the application server of a file sufficient in size for the test duration outlined in Table A.3.1-1 and record Throughput T result. (This is iteration 1)

4)	Repeat transfer for itterations 2-5 within the same call as the first iteration.

5)	Calculate and record the average throughput of the five iterations.

6)	Count and record the overall number of NACK, ACK and statDTX on the PHICH during the test interval.

7)	If the RRC connection setup needs to be re-established after the subtest, perform the initial call setup in accordance with the initial conditions in clause A.3.X.2.4.1.

8)	Repeat steps 1 to 6 for each subtest in Table A.3.X.2.3-2, as appropriate.