3GPP TSG-RAN WG5 Meeting #62 	R5-14aaaa

- Prague, Czech Republic, 10th – 14th February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | CRNum | rev | - | Current version: | 10.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes to 16.2, 16.3, and 16.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  |  | Date: |  | yyyy-MM-dd |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | TC 16.3 and 16.4 were modelled according to the flow depicted in 3GPP 24.930 section 5.3 (remote UE having resources at start of flow while DUT does not) while TC 16.1 and 16.2 were modelled according to section 5.6.3 (both UEs having resources). After offline conversations with several parties, it was found that the latter is not realistic for VoLTE scenarios. The proposal was to use the 16.3 flow instead. Note: a preliminary draft of this CR also addressed 16.1. As it is expected that 16.1 will be deleted, this CR does not touch 16.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Therefore, TC 16.2 was re-written such that is uses the same flow as 16.3 and 16.4. After this rewriting, the differences between 16.2, 16.3, and 16.4 are in the SDP payload only: 16.2 is for narrowband with selected modes, 16.3 for wideband with all codec modes, 16.4 for wideband with selected modes. This was accomplished by taking the text from 16.3 and moving it to 16.2. The updated 16.3 and 16.4 then mostly refer to the updated 16.2, with the notable differences in SDP payload, see above. As 16.2 also refers to 16.1, such references were replaced by copying the referred to text into 16.2. This mostly applies to 16.2.2 and 16.2.4 (except for the “Specific Message Contents” part). In addition, minor fixes are being proposed, and the following changes of substance: - the UPDATE request and the corresponding 200 OK response (as originally shown in 16.3) are removed as they will not be sent in practice. The reason for this is that 24.229 forbids requests and responses that do not provide new information in SDP. - Offline comments suggested to explicitly list the point of time when the network triggers activation of the dedicated bearer. The appropriate point of time is after 183 Session Progress, PRACK and its 200 OK. As there is no other flow in 34.229-1 that lists such triggering as a separate step, a phrase on such triggering is added between steps 5 and 6. Note: it might be worthwhile to discuss if 34.229-1 in general should be more definite about when dedicated bearers are activated. - The possibility that the UE signals “curr:qos local sendrecv” in 183 Session Progress is taken away in all three flows as the proposal is to trigger the activation of a dedicated bearer at a later point of time (and without a dedicated bearer the UE cannot signal resources) The following issues are left open for now: - Should 183 Session Progress and 180 Ringing be mandatory rather than optional in a VoLTE and IR.92 environment? R2: reverted back to optional for the time being R2 additional explanations: the changes can be summarized in that we added 183 Session Progress with PRACK and 200 OK to 16.2, and removed UPDATE with 200 OK from 16.3 and 16.4, as well as minor changes to SDP plus more explicit statements about points of time when resources are available at endpoints and when the activation of the dedicated bearer is triggered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 16.2 cannot be verified in a VoLTE environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2, 16.3, 16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | R1: made steps 3, 4, 5, and 6 mandatory rather than optional R2: reverted back to numbering as is Reverted SDP changes concerning max-red and mode-change-capability Added Editor’s note on dedicated bearer Deleted some mentioning of default content |  |  |  |  |  |  |  |  |

## <Start of Change 1>

## 16.2	Speech AMR, indicate selective codec modes

### 16.2.1	Definition and applicability

Test to verify that the UE correctly performs IMS Multimedia Telephony speech call setup when selective AMR codec modes are offered. This process is described in 3GPP TS 24.173 [65], TS 24.229 [10] and TS 26.114 [66]. The test case is applicable for IMS security or GIBA.

### 16.2.2	Conformance requirement

[TS 24.229, clause 5.1.4.1]

If an initial INVITE request is received the terminating UE shall check whether the terminating UE requires local resource reservation.

NOTE 1:	The terminating UE can decide if local resource reservation is required based on e.g. application requirements, current access network capabilities, local configuration, etc.

If local resource reservation is required at the terminating UE and the terminating UE supports the precondition mechanism, and:

a) 	the received INVITE request includes the "precondition" option-tag in the Supported header or Require header, the terminating UE shall make use of the precondition mechanism and shall indicate a Require header with the "precondition" option-tag in any response or subsequent request it sends towards to the originating UE; or

…

[TS 26.114, clause 5.2.1]

MTSI terminals offering speech communication shall support:

-	AMR speech codec (3GPP TS 26.071, 3GPP TS 26.090, 3GPP TS 26.073 and 3GPP TS 26.104) including all 8 modes and source controlled rate operation ‎3GPP TS 26.093. The terminal shall be capable of operating with any subset of these 8 codec modes.

[TS 24.229, clause 6.1.1]

During session establishment procedure, SIP messages shall only contain SDP payload if that is intended to modify the session description, or when the SDP payload must be included in the message because of SIP rules described in RFC 3261.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

Reference(s)

3GPP TS 24.229 [10] clause 5.1.4.1. TS 26.114 [66] clauses 5.2.1, 6.2.5, and 7.3.1.

### 16.2.3	Test purpose

1) To verify that, when initiating MT MTSI speech AMR call with selective codec modes and with the remote UE already having resources available, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2) To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3) To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4) To verify that the UE is able to release the call.

### 16.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Related ICS/IXIT Statement(s)

IMS security (Yes/No)

GIBA (Yes/No)

Support for initiating a session (Yes/No)

Support for speech (Yes/No)

Support for IMS Multimedia Telephony (Yes/No)

Test procedure

- SS sends an INVITE request to the UE.

- Void.

3)	SS may receive 100 Trying from the UE.

3A)	SS may receive 183 Session Progress from the UE.

3B)	SS may send PRACK to the UE to acknowledge the 183 Session Progress.

3C)	SS may receive 200 OK for PRACK from the UE. 

SS triggers the activation of a dedicated bearer.
Editor’s note: the statement on bearer activation needs to be finalized

4)	SS may receive 180 Ringing from the UE.

5)	SS may send PRACK to the UE to acknowledge the 180 Ringing.

6)	SS may receive 200 OK for PRACK from the UE.

6A) The UE accepts the session invite.
If 180 Ringing is not received from the UE after 5s from step 1, the MMI command shall be started to trigger the UE to accept the call.

7)	SS expects and receives 200 OK for INVITE from the UE.

8)	SS send an ACK to acknowledge receipt of the 200 OK for INVITE

9)	SS sends BYE to the UE.

10)	SS expects and receives 200 Ok for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE with the first SDP offer. |
| 2 |  |  |  | Void |
| 3 | à |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 3A | à |  | 183 Session Progress | The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 3B |  |  | PRACK | SS acknowledges if a 183 Session Progress is received. |
| 3C | à |  | 200 OK | The UE responds if a PRACK is sent. |
| 4 | à |  | 180 Ringing | The UE responds to INVITE with 180 Ringing. |
| 5 |  |  | PRACK | (Optional) SS shall send PRACK if the 180 response contains 100rel option-tag in the Require header. |
| 6 | à |  | 200 OK | (Optional) The UE acknowledges the PRACK with 200 OK. |
| 6A |  |  |  | Make UE accept the speech AMR offer. |
| 7 | à |  | 200 OK | The UE responds INVITE with 200 OK. |
| 8 |  |  | ACK | The SS acknowledges the receipt of 200 OK for INVITE. |
| 9 |  |  | BYE | The SS releases the call with BYE. |
| 10 | à |  | 200 OK | The UE sends 200 OK for BYE. |

NOTE:	The default messages contents in annex A are used with condition “IMS security” or “GIBA” when applicable

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MT Call" in annex A.2.9, with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o= - 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=IMS conformance test - c=IN (addrtype) (connection-address for SS) - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 99 - b=AS:30 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=0,2,5,7; mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

100 Trying for INVITE (Step 3)

183 Session Progress (Step 3A)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

180 Ringing (Step )

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) mode-set=0,2,5,7; Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 7)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | not present if 180 Ringing (step 4) contained SDP. present if 180 Ringing (step 4) did not contain SDP. Contents if present: The same requirements for SDP types and values as specified in step 4. |

### 16.2.5	Test requirements

The UE shall send requests and responses as described in clause 16.2.4.

## 16.3	Speech AMR-WB, indicate all codec modes

### 16.3.1	Definition and applicability

Test to verify that the UE correctly performs IMS Multimedia Telephony speech call setup when all AMR-WB codec modes are offered. This process is described in 3GPP TS 24.173 [65], TS 24.229 [10] and TS 26.114 [66]. The test case is applicable for IMS security or GIBA.

### 16.3.2	Conformance requirement

[TS 24.229, clause 5.1.4.1]

If an initial INVITE request is received the terminating UE shall check whether the terminating UE requires local resource reservation.

NOTE 1:	The terminating UE can decide if local resource reservation is required based on e.g. application requirements, current access network capabilities, local configuration, etc.

If local resource reservation is required at the terminating UE and the terminating UE supports the precondition mechanism, and:

a) 	the received INVITE request includes the "precondition" option-tag in the Supported header or Require header, the terminating UE shall make use of the precondition mechanism and shall indicate a Require header with the "precondition" option-tag in any response or subsequent request it sends towards to the originating UE; or

…

[TS 26.114, clause 5.2.1]

MTSI terminals offering speech communication shall support:

-	AMR speech codec (3GPP TS 26.071, 3GPP TS 26.090, 3GPP TS 26.073 and 3GPP TS 26.104) including all 8 modes and source controlled rate operation ‎3GPP TS 26.093. The terminal shall be capable of operating with any subset of these 8 codec modes.

…

MTSI terminals offering wideband speech communication at 16 kHz sampling frequency shall support:

-	AMR wideband codec (3GPP TS 26.171, 3GPP TS 26.190, 3GPP TS 26.173 and 3GPP TS 26.204) including all 9 modes and source controlled rate operation ‎3GPP TS 26.193. The terminal shall be capable of operating with any subset of these 9 codec modes.

…

MTSI terminals offering wideband speech communication shall also offer narrowband speech communications. When offering both wideband speech and narrowband speech communication, wideband shall be listed as the first payload type in the m-line of the SDP offer (RFC 4566).

[TS 24.229, clause 6.1.1]

During session establishment procedure, SIP messages shall only contain SDP payload if that is intended to modify the session description, or when the SDP payload must be included in the message because of SIP rules described in RFC 3261.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

Reference(s)

3GPP TS 24.229 [10] clause 5.1.4.1, TS 26.114 [66] clauses 5.2.1, 6.2.5, and 7.3.1.

### 16.3.3	Test purpose

1) To verify that, when initiating MT MTSI speech AMR-WB call and with the remote UE already having resources available, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2) To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3) To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4) To verify that the UE is able to release the call.

### 16.3.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Related ICS/IXIT Statement(s)

IMS security (Yes/No)

GIBA (Yes/No)

Support for initiating a session (Yes/No)

Support for speech (Yes/No)

Support for speech, AMR wideband (Yes/No)

Support for IMS Multimedia Telephony (Yes/No)

Test procedure

1)	SS sends an INVITE request to the UE.

2) Void.

3)	SS may receive 100 Trying from the UE.

4)	SS may receive 183 Session Progress from the UE.

5)	SS may send PRACK to the UE to acknowledge the 183 Session Progress.

6)	SS may receive 200 OK for PRACK from the UE.

7)	Void

8)	Void

9)	SS may receive 180 Ringing from the UE.

10)	SS may send PRACK to the UE to acknowledge the 180 Ringing.

11)	SS may receive 200 OK for PRACK from the UE.

11A) The UE accepts the session invite.
If 180 Ringing is not received from the UE after 5s from step 1, the MMI command shall be started to trigger the UE to accept the call.

12)	SS expects and receives 200 OK for INVITE from the UE.

13)	SS send an ACK to acknowledge receipt of the 200 OK for INVITE

14)	SS sends BYE to the UE.

15)	SS expects and receives 200 Ok for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE with the first SDP offer. |
| 2 |  |  |  | Void |
| 3 | à |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 4 | à |  | 183 Session Progress | (Optional) The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 5 |  |  | PRACK | (Optional) SS acknowledges if a 183 Session Progress is received. |
| 6 | à |  | 200 OK | (Optional) The UE responds if a PRACK is sent. |
| 7 |  |  |  | void |
| 8 |  |  |  | void |
| 9 | à |  | 180 Ringing | (Optional) The UE responds to INVITE with 180 Ringing. |
| 10 |  |  | PRACK | (Optional) SS shall send PRACK if the 180 response contains 100rel option-tag in the Require header. |
| 11 | à |  | 200 OK | (Optional) The UE acknowledges the PRACK with 200 OK. |
| 11A |  |  |  | Make UE accept the speech AMR offer. |
| 12 | à |  | 200 OK | The UE responds INVITE with 200 OK . |
| 13 |  |  | ACK | The SS acknowledges the receipt of 200 OK for INVITE. |
| 14 |  |  | BYE | The SS releases the call with BYE. |
| 15 | à |  | 200 OK | The UE sends 200 OK for BYE. |

NOTE:	The default messages contents in annex A are used with condition “IMS security “ or “GIBA”  when applicable

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MT Call" in annex A.2.9, with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=IMS conformance test - c=IN (addrtype) (connection-address for SS) - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 99 - b=AS:30 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

100 Trying for INVITE (Step 3)

Use the default message “100 Trying for INVITE” in annex A.2.2

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

PRACK (step 5)

Use the default message "PRACK" in annex A.2.4. No content body is included in this PRACK message.

200 OK (Step 6)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

Use the default message "UPDATE" in annex A.2.5 with the following exceptions:

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

180 Ringing (Step 9)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | optional if 183 Session Progress is not used not present if 183 Session Progress is used (step 3) Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

PRACK (step 10)

Use the default message "PRACK" in annex A.2.4. No content body is included in this PRACK message

200 OK (Step 11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

200 OK for INVITE (Step 12)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | not present if 183 Session Progress is used (step 3) or 180 Ringing (step 6) contained SDP. present if 183 Session Progress is not used (step 3) and 180 Ringing (step 6) did not contain SDP. Contents if present: The same requirements for SDP types and values as specified in step 6. |

ACK (Step 13)

Use the default message “ACK” in annex A.2.7.

BYE (step 14)

Use the default message "BYE" in annex A.2.8.

200 OK (step 15)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

### 16.3.5	Test requirements

The UE shall send requests and responses as described in clause 16.3.4.

## 16.4	Speech AMR-WB, indicate selective codec modes

### 16.4.1	Definition and applicability

Test to verify that the UE correctly performs IMS Multimedia Telephony speech call setup when selective AMR-WB codec modes are offered. This process is described in 3GPP TS 24.173 [65], TS 24.229 [10] and TS 26.114 [66]. The test case is applicable for IMS security orGIBA.

### 16.4.2	Conformance requirement

Same as 34.229-1 clause 16.3.2.

### 16.4.3	Test purpose

1) To verify that, when initiating MT MTSI speech AMR-WB call with selective codec modes and with the remote UE already having resources available, the UE performs correct exchange of  SIP protocol signalling messages for setting up the session.

2) To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3) To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4) To verify that the UE is able to release the call.

### 16.4.4	Method of test

Same as 34.229-1 clause 16.3.4 except

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MT Call" in annex A.2.9, with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | Precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o= - 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=IMS conformance test - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 99 - c= IN (addrtype) (connection-address for SS) - b=AS:30 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-set=0,2,5,7,8; mode-change-capability=2; max-red=220 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=0,2,5,7; mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) mode-set=0,2,5,7,8; Attributes for preconditions: - a=a=curr:qos local none - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

Use the default message "UPDATE" in annex A.2.5, but with the following exceptions:

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

180 Ringing (Step 9)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | optional if 183 Session Progress is not used not present if 183 Session Progress is used (step 3) Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c= IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) mode-set=0,2,5,7,8; Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 12)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | Header not present if 183 Session Progress is used (step 3) or 180 Ringing (step 6) contained SDP. Header present if 183 Session Progress is not used (step 3) and 180 Ringing (step 6) did not contain SDP. Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=- (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=IMS conformance test - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c= IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) mode-set=0,2,5,7,8; Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

### 16.4.5	Test requirements

The UE shall send requests and responses as described in clause 16.4.4.

## <End of Change 1>