3GPP TSG-RAN WG5 Meeting #62 	R5-140220r1

# Prague, Czech Republic, 10 - 14 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | CRNum | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TC 7.4A.2 for CA_3C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom,CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12-UEConTest |  |  |  |  |  |  | Date: |  | 2014-01-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | A new WI proposal of Further REL-12 Configurations for LTE Advanced Carrier Aggregation was agreed in RAN#61 meeting and updates for new CA combinations should be available to technical specifications.Currently, test case of Reference sensitivity level for CA_3C is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Initial conditions for 20MHz +5MHz were added to test configuration table 7.4A.2.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 7.4A.2 will not be available to CA_3C. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision 1: File renamed. |  |  |  |  |  |  |  |  |

<Start of modified section>

7.4A.2	Maximum input level for CA (intra-band contiguous DL CA without UL CA)

7.4A.2.1	Test purpose

Same test purpose as in clause 7.4A.1.1

7.4A.2.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA but no UL CA.

7.4A.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.4A.1.3

7.4A.2.4	Test description

7.4A.2.4.1	Initial conditions

Same initial conditions as in clause 7.4A.1.4.1 with the following exceptions:

-	Instead of Table 7.4A.1.4.1-1  use Table 7.4A.2.4.1-1

Table 7.4A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 for different CA bandwidth classes. |  |  |  | C: Mid range |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | UL Allocation |  |  |  |
| PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| 75 | 75 | QPSK | 75+75 | QPSK | 75 | P_75@0 | NA |
| 75 | 75 | QPSK | 75+75 | QPSK | 50 | P_50@0 | NA |
| 75 | 75 | QPSK | 75+75 | QPSK | 25 | P_25@0 | NA |
| 75 | 75 | QPSK | 75+75 | QPSK | 20 | P_20@0 | NA |
| 100 | 25 | QPSK | 100+25 | QPSK | 50 | P_50@0 | NA |
| 100 | 25 | QPSK | 100+25 | QPSK | 25 | P_25@0 | NA |
| 100 | 25 | QPSK | 100+25 | QPSK | 20 | P_20@0 | NA |
| 100 | 50 | QPSK | 100+50 | QPSK | 100 | P_100@0 | NA |
| 100 | 50 | QPSK | 100+50 | QPSK | 75 | P_75@0 | NA |
| 100 | 50 | QPSK | 100+50 | QPSK | 50 | P_50@0 | NA |
| 100 | 50 | QPSK | 100+50 | QPSK | 25 | P_25@0 | NA |
| 100 | 50 | QPSK | 100+50 | QPSK | 20 | P_20@0 | NA |
| 100 | 100 | QPSK | 100+100 | QPSK | 100 | P_100@0 | NA |
| 100 | 100 | QPSK | 100+100 | QPSK | 75 | P_75@0 | NA |
| 100 | 100 | QPSK | 100+100 | QPSK | 50 | P_50@0 | NA |
| 100 | 100 | QPSK | 100+100 | QPSK | 25 | P_25@0 | NA |
| 100 | 100 | QPSK | 100+100 | QPSK | 20 | P_20@0 | NA |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA Configuration only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per CA Configuration Test CC combination. |  |  |  |  |  |  |  |

7.4A.2.4.2	Test procedure

Same test procedure as in clause 7.4A.1.4.2 with the following exceptions:

-	Instead of Table 7.4A.1.4.1-1  use Table 7.4A.2.4.1-1

7.4A.2.4.3	Message contents

Same message contents as in clause 7.4A.1.4.3

7.4A.2.5	Test requirement

Same test requirements as in clause 7.4A.1.5.

Table 7.4A.1.5-1: Maximum input level for intra-band contiguous CA

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in Transmission Aggregated Bandwidth Configuration | dBm |  |  | For carrier frequency f ≤ 3.0GHz: -22.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -23.0 |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in clause 6.2.5A. Note 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

<End of modified section>