3GPP TSG-RAN WG5 Meeting #62 	R5-140224r2

# Prague, Czech Republic, 10 - 14 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | CRNum | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TC 7.3A.1 for CA_3C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom,CATR,Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12-UEConTest |  |  |  |  |  |  | Date: |  | 2014-02-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | A new WI proposal of Further REL-12 Configurations for LTE Advanced Carrier Aggregation was agreed in RAN#61 meeting and updates for new CA combinations should be available to technical specifications.Currently, test case of Reference sensitivity level for CA_3C is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum conformance requirements for CA_3C were added; - CA configuration of 20MHz + 5 MHz for CA_3C were added to table 7.3A.1.3-1 and table 7.3A.1.5-2; - Initial conditions for 20MHz +5MHz were added to test configuration table 7.3A.1.4.1-1; - Test requirments of 20MHz + 5 MHz for CA_3C were added to table 7.3A.1.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 7.3A.1 will not be available to CA_3C. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision 1: File renamed. Revision 2: According to the contributions from Aritsu, table 7.3A.1.3-1, 7.3A.1.5-1 and 7.3A.1.5-2 were modified. And a 100+25RB configuration in table 7.3A.1.4.1-1 was removed. |  |  |  |  |  |  |  |  |

<Start of modified section>

7.3A.1	Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA)

7.3A.1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area of an e-NodeB.

7.3A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

7.3A.1.3	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1 and Table 7.3.3-2. The reference sensitivity is defined to be met with both downlink component carriers active and either of the uplink component carriers active. The UE shall meet the requirements specified in clause 7.3.1 with the following exceptions.

3Table 7.3A.1.3-0: Void

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3A.1.3-0a, exceptions are allowed when the uplink active in the lower-frequency operating band is within a specified frequency range as noted in Table 7.3A.1.3-0a. For these exceptions, the UE shall meet the requirements specified in Table 7.3A.1.3-0a, Table 7.3A.1.3-0b and Table 7.3A.1.3-0c.

Table 7.3A.1.3-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-8A4 | 3 |  |  |  | N/A | N/A | N/A | FDD |
|  | 8 |  |  | N/A | N/A |  |  |  |
| CA_4A-12A5 | 4 | [-89.2] | [-89.2] | [-90] | [-89.5] |  |  | FDD |
|  | 12 |  |  | - 96.5 | - 93.5 |  |  |  |
| CA_4A-17A5 | 4 |  |  | [-90] | [-89.5] |  |  | FDD |
|  | 17 |  |  | -96.5 | -93.5 |  |  |  |
| Note 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. Note 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: No requirements apply when there is at least one individual RE within the transmission bandwidth of the low band for which the 2nd harmonic is within the transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.3 apply). Note 5: These requirements apply when there is at least one individual RE within the transmission bandwidth of the low band for which the 3rd harmonic is within transmission bandwidth of the high band. The requirements should be verified for UL EARFCN of the low band (superscript LB) such that in MHz and with the carrier frequency of the high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

Table 7.3A.1.3-0b: Uplink configuration for the low band

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_4A-12A | 12 | 2 | 5 | 8 | 16 |  |  | FDD |
| CA_4A-17A | 17 |  |  | 8 | 16 |  |  | FDD |
| Note 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth Note 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.3-2 for the uplink bandwidth in which case the allocation according to Table 7.3.3-2 applies |  |  |  |  |  |  |  |  |

Unless given by Table 7.3A.1.3-0c, the minimum requirements specified in Tables 7.3A.1.3-0a and 7.3.A.1.3-0b shall be verified with the network signalling value NS_01 (Table 6.2.4.3-1) configured.

Table 7.3A.1.3-0c: Network signalling value for reference sensitivity

| E-UTRA CA Configuration | Uplink Band | Network Signalling value |
| --- | --- | --- |
| CA_4A-12A | 12 | NS_06 |
| CA_4A-17A | 17 | NS_06 |

For band combinations including operating bands without uplink band (as noted in Table 5.2-1), the requirements are specified in Table 7.3A.1-3-0d, Table 7.3A.1.3-0e and Table 7.3A.1.3-0f.

Table 7.3A.1.3-0d: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-29A | 2 |  |  | -98 | -95 |  |  | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| Note 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. Note 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port |  |  |  |  |  |  |  |  |

Table 7.3A.1.3-0e: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-29A | 2 |  |  | 25 | 50 |  |  | FDD |
|  | 29 |  | N/A | N/A | N/A |  |  |  |

Unless given by Table 7.3A.1.3-0f, the minimum requirements specified in Tables 7.3A.1.3-0d and 7.3A.1.3-0e shall be verified with the network signalling value NS_01 (Table 6.2.4.3-1) configured.

Table 7.3A.1.3-0f: Network signalling value for reference sensitivity

| EUTRA CA Configuration | Uplink Band | Network Signalling value |
| --- | --- | --- |
| CA_2A-29A | 2 | NS_03 |

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1 and table 7.3A.1.3-1. Table 7.3A.1.3-1 specifies the maximum number of allocated uplink resource blocks for which the intra-band contiguous carrier aggregation reference sensitivity requirement shall be met. The PCC and SCC allocations follow Table 7.3.3-2 and form a contiguous allocation where TX–RX frequency separations are as defined in Table 5.3-1. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2 and the downlink PCC carrier centre frequency shall be configured closer to uplink operating band than the downlink SCC centre frequency.

Table 7.3A.1.3-1: Intra-band CA uplink configuration for reference sensitivity

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | 100RB+25RB |  | 100RB+50RB |  |  | 75RB+75RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC |  | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_1C |  |  | N/A |  | N/A | 75 | 54 | N/A | N/A | 100 | 30 | FDD |
| CA_3C | 50 | 0 | 50 |  | 0 | N/A | N/A | 50 | 0 | 50 | 0 | FDD |
| CA_7C |  |  | N/A |  | N/A | 75 | 0 | N/A | N/A | 75 | 0 | FDD |
| CA_38C |  |  |  |  |  | 75 | 75 |  |  | 100 | 100 | TDD |
| CA_40C |  |  | 100 |  | 50 | 75 | 75 | N/A | N/A | 100 | 100 | TDD |
| CA_41C |  |  | 100 |  | 50 | 75 | 75 | 100 | 75 | 100 | 100 | TDD |
| Note 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. Note 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. Note 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1. The reference sensitivity is defined to be met with both downlink component carriers active and one uplink carrier active. For reference sensitivity measured on the downlink PCC, the parameters in Table 7.3.3-2 apply. For reference sensitivity measured on the downlink SCC, the parameters in Table 7.3A.1.3-2 apply. The minimum requirement for reference sensitivity in Table 7.3.3-1 shall be increased by the amount given in DRIBNC in Table 7.3A.1.3-2 for the downlink SCC.

Table 7.3A.1.3-2: Intra-band non-contiguous CA uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_25A-25A | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0.0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0.0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 104 | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0.0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 104 | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0.0 |  |
| CA_41A-41A | Note 6 | Note 7 | Note 8 | 0.0 | TDD |
| Note 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. Note 2: Wgap is the sub-block gap between the two sub-blocks. Note 3: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. Note 4: 4 refers to the UL resource blocks shall be located at RBstart=33. Note 5: For the TDD intra-band non-contiguous CA configurations, the minimum requirements apply only in synchronized operation between all component carriers. Note 6: All combinations of channel bandwidths defined in Table [5.2A-3]. Note 7: All applicable sub-block gap sizes. Note 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.4.2-1. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101[2] clause 7.3.1 and 7.3.1A.

7.3A.1.4	Test description

7.3A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | C: Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  |  | UL Allocation |  |  |  |  |  |  |
| PCC NRB | SCC NRB | CC MOD |  | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 75 | P_75@0 |  | S_0@0 | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 50 | P_50@25 |  | S_0@0 | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 25 | P_25@50 |  | S_0@0 | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 20 | P_20@55 |  | S_0@0 | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 130 | P_75@0 |  | S_55@0 | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 150 | P_75@0 |  | S_75@0 | - | - |
| 100 | 25 | QPSK |  | 100+25 | QPSK | 50 | P_50@0 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 100 | P_100@0 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 75 | P_75@25 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 50 | P_50@50 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 25 | P_25@75 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 20 | P_20@80 |  | S_0@0 | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 150 | P_100@0 |  | S_50@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 100 | P_100@0 |  | S_0@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 75 | P_75@25 |  | S_0@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 50 | P_50@50 |  | S_0@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 25 | P_25@75 |  | S_0@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 20 | P_20@80 |  | S_0@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 130 | P_100@0 |  | S_30@0 | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 200 | P_100@0 |  | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 and Table 5.4.2A.1-2. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.1.3-1 for UE supporting two uplink carriers is tested per Test CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate .

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.3A. 1.4.3.

7.3A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2. The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables 7.3A.1.4.1-1 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCC to the appropriate REFSENS value defined in Table 7.3A.1.5-1. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex G.2A.

7.3A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

7.3A.1.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.1.5-1 and table 7.3A.1.5-2.

Table 7.3A.1.5-1: Reference sensitivity QPSK PREFSENS for intra-band

|  | CA Band / Aggregated channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | 100RB+50RB |  | 75RB+75RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | 100RB | 50RB | 75RB | 75RB | 100RB | 75RB | 100RB | 100RB |  |
| CA_1C | - | - | -94.5 | -94.5 | - | - | -93.3 | -93.3 | FDD |
| CA_7C | - | - | -92.5 | -92.5 | - | - | -91.3 | -91.3 | FDD |
| CA_38C | - | - | -94.5 | -94.5 | - | - | -93.3 | -93.3 | TDD |
| CA_40C | -93.3 | -96.3 | -94.5 | -94.5 | - | - | -93.3 | -93.3 | TDD |
| CA_41C | -91.3 | -94.3 | -92.5 | -92.5 | -91.3 | -92.5 | -91.3 | -91.3 | TDD |
| Note 1: The transmitter shall be set to PUMAX as defined in clause 6.2.5A Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Table 7.3.3-1A shall be applied for applicable bands.

In case the UE supports more than one of the inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 7.3.3-1A, truncated to one decimal place that would apply for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL (i.e. bands listed in Table 7.3A.1.3-0a), then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 7.3.3-1A that would apply for that operating band among the supported CA configurations

The reference receive sensitivity (REFSENS) requirement for intra-band specified in Table 7.3A.1.5-1 shall be met for each uplink CA configurations less than or equal to that specified in Table 7.3A.1.5-2.

NOTE:	Table 7.3A.1.5-2 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.1.5-2: Intra-band CA uplink configuration for reference sensitivity

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | 100RB+25RB |  | 100RB+50RB |  |  | 75RB+75RB |  | 100RB+75RB |  | 100RB+100RB |  |  | Duplex Mode |
|  | PCC | SCC | PCC |  | SCC | PCC | SCC | PCC | SCC | PCC |  | SCC |  |
| CA_1C |  |  | n/a |  | n/a | 75 | 55 | n/a | n/a | 100 |  | 30 | FDD |
| CA_3C | 50 | 0 | 50 |  | 0 | n/a | n/a | 50 | 0 | 50 |  | 0 | FDD |
| CA_7C |  |  | n/a |  | n/a | 75 | 0 | n/a | n/a | 75 |  | 0 | FDD |
| CA_38C |  |  |  |  |  | 75 | 75 |  |  | 100 |  | 100 | TDD |
| CA_40C |  |  | 100 |  | 50 | 75 | 75 | n/a | n/a | 100 |  | 100 | TDD |
| CA_41C |  |  | 100 |  | 50 | 75 | 75 | 100 | 75 | 100 |  | 100 | TDD |
| Note 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. Note 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. Note 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2A-1). Note 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section>