3GPP TSG-RAN5 Meeting #73	R5-168039r1

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2994 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of 7.3A Refsens for CA_1A-3A-41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2016-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Refsens for new R14 CA combination CA_1A-3A-41A is currently missing in 36.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impacted Rel-14 CA configurations: CA_1A-3A-41A Updated minimum conformance requirement for CA_1A-3A-41A to 7.3A.0 Added test point of Refsens for CA_1A-41A-42A Added test requirement of Refsens for CA_1A-3A-41A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Refsens for 3DL CA configuration will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0, 7.3A.5.4, 7.3A.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1： Removed changes in minimum conformance requirement 7.3A.0 to avoid confliction with R14 common CR R5-168313 Updated test configurations table for CA_1A-41A-42A |  |  |  |  |  |  |  |  |

### <<Unchanged sections omitted>>

## 7.3A	Reference sensitivity level for CA

Editor's note: The Test Requirements for CA_2A-30A are missing.

### 7.3A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1, Table 7.3.3-1a and Table 7.3.3-2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. The uplink resource blocks shall be located as close as possible to the primary downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). The primary downlink operating band is the downlink band of the active uplink operating band. The UE shall meet the requirements specified in subclause 7.3.3 with the following exceptions.

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3A.0-1, exceptions to the aforementioned requirements are allowed when the uplink active in a lower-frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3A.0-1. For these exceptions, the UE shall meet the requirements specified in Table 7.3A.0-1 and Table 7.3A.0-1a.

### <<Unchanged sections omitted>>

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3A.0-1cE, UE shall meet the reference sensitivities specified in Table 7.3A.0-1cE and Table 7.3A.0-1cF.

Table 7.3A.0-1cE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |
| CA_1A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |  |
| CA_1A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |
| CA_1A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |
| CA_1A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |
| CA_1A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-5A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-5A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |
| CA_3A-7A-38A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 7 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |
| CA_3A-8A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-8A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-28A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |
|  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |
| CA_3A-28A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_3A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_3A-41A5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3A-41C5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  |  |  |  | [-100.2]11 | [-97.2]11 | [-95.4]11 | [-94.2]11 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | [-99.7]11 | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3A-41A-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |
| CA_3A-41A-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |
| CA_3A-41A-42A5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  |  |  |  | [-100.2]11 | [-97.2]11 | [-95.4]11 | [-94.2]11 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |
| CA_3A-41A-42A5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  |  |  |  | [-100.2]11 | [-97.2]11 | [-95.4]11 | [-94.2]11 |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
| CA_7A-40A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |
| CA_7A-40C | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |
| CA_7A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |
| CA_7A-42A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |
| CA_28A-40A | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_28A-40C | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_28A-40D | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 |  |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range. NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C and CA_1A-3A-19A-42A, CA 3A-41A-42A. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 7.3A.0-1cF: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-41A | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-5A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 8 |  | 15 | 25 | 251 |  |  | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-8A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 8 |  | 15 | 25 | 251 |  |  | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A CA_3A-40C CA_3A-5A-40A CA_3A-28A-40A CA_3A-28A-40C CA_1A-3A-5A-40A CA_1A-3A-8A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-40A, CA_7A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-42A, CA_7A-42A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40A, CA_28A-40C | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

### <<Unchanged sections omitted>>

### 7.3A.5	Reference sensitivity level for 3DL CA

#### 7.3A.5.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA, or 3DL with Inter-band CA, or 3DL with Intra-band contiguous and Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and Inter-band CA, or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

#### 7.3A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.5.4	Test description

##### 7.3A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.5.4.1-1 through table 7.3A.5.4.1-5 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.5.4.1-1: Test Configuration Table (Intra-band contiguous)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCCs RB allocation |  |  | CC MOD | PCC LCRB @ RBstart |
|  | Band | Range | PCC | SCC1 | SCC2 |  | SCC1 | SCC2 |  | PCC | SCC1 | SCC2 |  |  |
|  |  |  | CC | CC | CC |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low | CC1 | CC2 | CC3 | Highest NRB_agg |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 2 | X | Low | CC1 | CC2 | CC3 | Lowest NRB_agg |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 3 | X | High | CC1 | CC2 | CC3 | Highest NRB_agg |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 4 | X | High | CC1 | CC2 | CC3 | Lowest NRB_agg |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: X corresponds to the band of the CA Configuration. E.g. for CA_41D, X=41 Note 4: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 5: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-2: Test Configuration Table (Inter-band)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCCs RB allocation |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | SCC1 |  | SCC2 |  |  | SCC1 | SCC2 |  | PCC | SCC1 | SCC2 |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | Z | Mid | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  | QPSK | REFSENS |
| 2 | Y | Mid | X | Mid | Z | Mid | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  | QPSK | REFSENS |
| 3 | Z | Mid | X | Mid | Y | Mid | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  | QPSK | REFSENS |
| Test Settings for CA_1A-3A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 1 | Low | 3 | High | 5 | Mid | 100 | 25 | 50 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 29 | 1 | Low | 3 | High | 5 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Mid | 3 | High | 5 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 4 | 3 | High | 1 | Low | 5 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 5 | 5 | Mid | 1 | Mid | 3 | Mid | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
|  | Test Settings for CA_1A-3A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | High | 7 | Mid | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 2 | 1 | Low | 3 | High | 7 | Mid | 100 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Low | 3 | High | 7 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 4 | 1 | Mid | 3 | High | 7 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 5 | 1 | Mid | 3 | Mid | 7 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 3 | High | 1 | Low | 7 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 7 | 3 | Mid | 1 | Mid | 7 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 8 | 7 | Mid | 1 | Mid | 3 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 75@0 |
| Test Settings for CA_1A-3A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 1 | Low | 3 | High | 8 | Mid | 100 | 25 | 50 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 29 | 1 | Low | 3 | High | 8 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Mid | 3 | High | 8 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 4 | 3 | High | 1 | Low | 8 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 5 | 3 | Mid | 1 | Mid | 8 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 6 | 8 | High | 1 | Mid | 3 | Mid | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
| Test Settings for CA_1A-3A-19A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Mid | 3 | High | 19 | High | 75 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 2 | 1 | Mid | 3 | High | 19 | Mid | 75 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 39 | 1 | Low | 3 | High | 19 | Mid | 100 | 25 | 75 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 4 | 1 | Mid | 3 | High | 19 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 5 | 1 | Mid | 3 | High | 19 | Mid | 100 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 6 | 3 | High | 1 | Mid | 19 | High | 100 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 7 | 3 | High | 1 | Mid | 19 | Mid | 100 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 8 | 3 | High | 1 | Mid | 19 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 9 | 3 | High | 1 | Mid | 19 | Mid | 100 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 10 | 19 | High | 1 | Mid | 3 | High | 50 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 11 | 19 | High | 1 | Mid | 3 | High | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 12 | 19 | Mid | 1 | Mid | 3 | High | 75 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 13 | 19 | Mid | 1 | Mid | 3 | High | 75 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@50 |
|  | Test Settings for CA_1A-3A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | High | 20 | Mid | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 2 | 1 | Low | 3 | High | 20 | Mid | 100 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Low | 3 | High | 20 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 4 | 1 | Mid | 3 | High | 20 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 5 | 1 | Mid | 3 | Mid | 20 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 3 | High | 1 | Low | 20 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 7 | 3 | Mid | 1 | Mid | 20 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 8 | 20 | Mid | 1 | Mid | 3 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 20@16 |
| Test Settings for CA_1A-3A-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | High | 41 | Mid | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 2 | 1 | Low | 3 | High | 41 | Mid | 100 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Low | 3 | High | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 4 | 1 | Mid | 3 | High | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 5 | 1 | Low | 3 | High | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 1 | High | 3 | Mid | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 7 | 3 | High | 1 | Low | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 8 | 3 | Mid | 1 | Mid | 41 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@50 |
| Test Settings for CA_1A-3A-42A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | High | 42 | Mid | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 2 | 1 | Low | 3 | High | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 1 | Mid | 3 | High | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 45@0 |
| 410 | 1 | Mid | 3 | High | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 511 | 3 | High | 1 | Low | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 6 | 3 | Low12 | 1 | Mid | 42 | Low12 | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 50@25 |
| 7 | 3 | High13 | 1 | Mid | 42 | Mid13 | 100 | 25 | 100 | QPSK | All RBs |  |  | QPSK | 50@25 |
| 8 | 42 | Mid | 1 | Mid | 3 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 9 | 42 | Low | 1 | Mid | 3 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 10 | 42 | High | 1 | Mid | 3 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_1A-5A-7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 5 | High | 7 | Mid | 50 | 50 | 100 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 2 | 1 | Low | 5 | High | 7 | Mid | 100 | 50 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 5 | High | 1 | Low | 7 | Mid | 50 | 50 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 4 | 5 | High | 1 | Low | 7 | Mid | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 5 | 7 | Mid | 1 | Low | 5 | High | 100 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 75@25 |
| 6 | 7 | Mid | 1 | Low | 5 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 75@25 |
| Test Settings for CA_1A-8A-11A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | 8 | High | 11 | Low | 100 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 8 | High | 1 | High | 11 | Low | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 3 | 11 | Low | 1 | High | 8 | High | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| Test Settings for CA_1A-18A-28A Configuration (Note 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 18 | Low | 28 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 2 | 1 | Low | 18 | Low | 28 | Mid | 75 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 75@0 |
| 3 | 1 | Low | 18 | Low | 28 | Mid | 100 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 4 | 1 | Low | 18 | Mid | 28 | High | 100 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 5 | 18 | Low | 1 | Low | 28 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 6 | 18 | Low | 1 | Low | 28 | Mid | 50 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 7 | 18 | Low | 1 | Low | 28 | Mid | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 8 | 18 | Mid | 1 | Low | 28 | High | 75 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 9 | 18 | Mid | 1 | Low | 28 | High | 75 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 10 | 28 | High | 1 | Low | 18 | Mid | 25 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 11 | 28 | Mid | 1 | Low | 18 | Low | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 12 | 28 | High | 1 | High | 18 | Mid | 50 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@25 |
| Test Settings for CA_1A-19A-21A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Mid | 19 | High | 21 | Mid | 75 | 50 | 75 | QPSK | All RBs |  |  | QPSK | 75@0 |
| 2 | 1 | Mid | 19 | Mid | 21 | Mid | 75 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 75@0 |
| 3 | 1 | Mid | 19 | High | 21 | Mid | 100 | 50 | 75 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 4 | 1 | Mid | 19 | Mid | 21 | Mid | 100 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 5 | 19 | High | 1 | Mid | 21 | Mid | 50 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 6 | 19 | High | 1 | Mid | 21 | Mid | 50 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 7 | 19 | Mid | 1 | Mid | 21 | Mid | 75 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 8 | 19 | Mid | 1 | Mid | 21 | Mid | 75 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 9 | 21 | Mid | 1 | Mid | 19 | High | 75 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 10 | 21 | Mid | 1 | Mid | 19 | Mid | 75 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 11 | 21 | Mid | 1 | Mid | 19 | High | 75 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 12 | 21 | Mid | 1 | Mid | 19 | Mid | 75 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@50 |
| Test Settings for CA_1A-19A-28A Configuration (Note 14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Mid | 19 | Mid | 28 | Mid | 100 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 100@ |
| 215 | 19 | Mid | 1 | Mid | 28 | Mid | 75 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 3 | 19 | Mid | 1 | Mid | 28 | Mid | 75 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 18@0 |
| 4 | 28 | Mid | 1 | Mid | 19 | Mid | 50 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 25@25 |
| Test Settings for CA_1A-41A-42A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 41 | Mid | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 1 | Mid | 41 | Mid | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 1 | High | 41 | Mid | 42 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 4 | 42 | Mid | 41 | Mid | 1 | Mid | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_2A-4A-5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 4 | Mid | 5 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 2 | 4 | Mid | 2 | Mid | 5 | Low | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 3 | 4 | Mid | 2 | Mid | 5 | High | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 4 | 5 | Low | 2 | Mid | 4 | Mid | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 5 | 5 | High | 2 | Mid | 4 | Mid | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| Test Settings for CA_2A-4A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 4 | Mid | 12 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 2 | 4 | Mid | 2 | Mid | 12 | Low | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 4 | Mid | 2 | Mid | 12 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 5 | 12 | Low6 | 2 | Mid | 4 | Low6 | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 16@17 |
| 6 | 12 | High7 | 2 | Mid | 4 | High7 | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 16@17 |
| Test Settings for CA_2A-4A-13A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 4 | Mid | 13 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 2 | 4 | Mid | 2 | Mid | 13 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 13 | Mid | 2 | Low | 4 | Low | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 4 | 13 | Mid | 2 | Mid | 4 | Mid | 50 | 75 | 75 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 5 | 13 | Mid | 2 | High | 4 | High | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 20@0 |
| Test Settings for CA_2A-5A-13A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 5 | Mid | 13 | Mid | 100 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 2 | 5 | Mid | 2 | Mid | 13 | Mid | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 3 | 13 | Mid | 2 | Low | 5 | Low | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 4 | 13 | Mid | 2 | Mid | 5 | Mid | 50 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 5 | 13 | Mid | 2 | High | 5 | High | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| Test Settings for CA_3A-7A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Mid | 7 | Mid | 8 | High | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 2 | 3 | Mid | 7 | Mid | 8 | High | 75 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 50@25 |
| 3 | 3 | Mid | 7 | Mid | 8 | High | 100 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 4 | 3 | Mid | 7 | Mid | 8 | High | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 5 | 3 | Mid | 7 | Mid | 8 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 50@50 |
| 6 | 7 | Mid | 3 | Mid | 8 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 75@25 |
| 7 | 8 | Mid | 3 | Mid | 7 | Mid | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 16@17 |
| 8 | 8 | High | 3 | Mid | 7 | Mid | 50 | 50 | 100 | QPSK | All RBs |  |  | QPSK | 16@17 |
| 9 | 8 | High | 3 | Mid | 7 | Mid | 25 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 10 | 8 | High | 3 | Mid | 7 | Mid | 25 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@0 |
| 11 | 8 | High | 3 | Mid | 7 | Mid | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 16@17 |
| Test Settings for CA_3A-19A-42A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low12 | 19 | Mid | 42 | Low12 | 100 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 50@25 |
| 2 | 3 | High13 | 19 | Mid | 42 | Mid13 | 100 | 75 | 100 | QPSK | All RBs |  |  | QPSK | 50@25 |
| 3 | 19 | Mid | 3 | High | 42 | Mid | 75 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 25@50 |
| 4 | 42 | Low | 3 | Mid | 19 | Mid | 100 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 5 | 42 | High | 3 | Mid | 19 | Mid | 100 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 42 | Mid | 3 | High | 19 | Mid | 100 | 100 | 75 | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_4A-5A-13A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | Mid | 5 | Mid | 13 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 50@0 |
| 2 | 5 | Low | 4 | Mid | 13 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 3 | 5 | Mid | 4 | Mid | 13 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 4 | 5 | High | 4 | Mid | 13 | Mid | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 25@25 |
| 5 | 13 | Mid | 4 | Low | 5 | Low | 50 | 50 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 6 | 13 | Mid | 4 | Mid | 5 | Mid | 50 | 75 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| 7 | 13 | Mid | 4 | High | 5 | High | 50 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 20@0 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-19A,X=1,Y=3,Z=19 Note 5: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 6: Band 12: fUL = 706.7 MHz (NUL = 23087), fDL = 736.7 MHz (NDL = 5087) Band 4: fDL = 2120.1 MHz (NDL = 2051) Note 7: Band 12: fUL = 710.9 MHz (NUL = 23129), fDL = 740.9 MHz (NDL = 5129) Band 4: fDL = 2132.7 MHz (NDL = 2177) Note 8: CA_1A-28A test shall not be skipped due to CA_1A_18A_28A test being conducted. Note 9: Test points that fulfil criteria of Note 4 in Table 7.3A.5.5-3 Note 10: Only Band 1 and Band 42 need to be tested and Band 3 does not need to be tested Note 11: Only Band 1 and Band 3 need to be tested and Band 42 does not need to be tested Note 12: Band 3: fUL = 1720MHz (NUL = 19300), fDL = 1815MHz (NDL = 1300) Band 42: fUL/DL = 3440MHz (NUL/DL = 41990) Note 13: Band 3: fUL = 1775MHz (NUL = 19850), fDL = 1870MHz (NDL = 1850) Band 42: fUL/DL = 3520MHz (NDL = 42790) Note 14: CA_1A-28A test shall not be skipped due to different frequency band definition of Band 28 between CA_1A-28A and CA_1A-19A-28A. Note 15: Only Band 1 and Band 19 need to be tested and Band 28 does not need to be tested |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-3: Test Configuration Table (Intra-band contiguous + Inter-band)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB |  | SCCs NRB |  |  |  |  | CC MOD | PCC & SCCs RB allocation |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  | SCC1 |  | SCC2 |  |  |  | PCC | SCC1 | SCC2 |  |  |
|  | Band | Range /CC | Band | Range /CC | Band | Range /CC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XC-YA and CA_XB-YA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low /CC1 | X | Low /CC2 | Y | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 2 | X | Low /CC1 | X | Low /CC2 | Y | Mid | Lowest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 3 | X | High /CC1 | X | High /CC2 | Y | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 4 | X | High /CC1 | X | High /CC2 | Y | Mid | Lowest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 5 | Y | Mid | X | Mid /CC1 | X | Mid /CC2 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| Default Test Settings for CA_XA-YC and CA_XA-YB Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | Low /CC1 | Y | Low /CC2 | X | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 2 | Y | Low /CC1 | Y | Low /CC2 | X | Mid | Lowest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | X | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 4 | Y | High /CC1 | Y | High /CC2 | X | Mid | Lowest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 5 | X | Mid | Y | Mid /CC1 | Y | Mid /CC2 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| Test Settings for a CA_1A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 41 | Mid /CC1 | 41 | Mid /CC2 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 1 | High | 41 | Mid /CC1 | 41 | Mid /CC2 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for a CA_1A-3C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | High /CC1 | 3 | High /CC2 | 100 |  | 25 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| 2 | 1 | Low | 3 | High /CC1 | 3 | High /CC2 | 100 |  | 75 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| 3 | 1 | Low | 3 | High /CC1 | 3 | High /CC2 | 100 |  | 100 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| 4 | 1 | Mid | 3 | High /CC1 | 3 | High/ CC2 | 100 |  | 100 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| 5 | 3 | High /CC1 | 3 | High /CC2 | 1 | Low | 100 |  | 100 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| 6 | 3 | High /CC1 | 3 | High /CC2 | 1 | Low | 100 |  | 25 |  |  |  |  | 100 | QPSK |  |  | All RBs | QPSK |
| Test Settings for CA_3A-42C configuration (Note 7, Note 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 42 | Low7 | 42 | Low7 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | 50@25 |
| 2 | 3 | High | 42 | Mid8 | 42 | Mid8 | Highest NRB |  | Highest NRB_agg |  |  |  |  | QPSK | All RBs |  |  | QPSK | 50@25 |
| 3 | 42 | Low /CC1 | 42 | Low /CC2 | 3 | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 4 | 42 | High /CC1 | 42 | High /CC2 | 3 | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_8A-42C configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Mid | 42 | Mid /CC1 | 42 | Mid /CC2 | Highest NRB |  |  | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 211 | 8 | Mid | 42 | High/ CC1 | 42 | High/ CC2 | 25 |  |  | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 42 | Low /CC1 | 42 | Low /CC2 | 8 | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 4 | 42 | High /CC1 | 42 | High /CC2 | 8 | Mid | Highest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| 5 | 42 | High /CC1 | 42 | High /CC2 | 8 | Mid | Lowest NRB_agg |  |  | Highest NRB |  |  |  | QPSK | All RBs |  |  | QPSK | REFSENS |
| Test Settings for a CA_26A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | Low | 41 | Mid /CC1 | 41 | Mid /CC2 | 75 |  |  | 100 | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@50 |
| 2 | 41 | Low /CC1 | 41 | Low /CC2 | 26 | Low | 100 |  |  | 50 | 75 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 41 | Low /CC1 | 41 | Low /CC2 | 26 | Low | 100 |  |  | 100 | 75 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 4 | 41 | High /CC1 | 41 | High /CC2 | 26 | Low | 100 |  |  | 50 | 75 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 5 | 41 | High /CC1 | 41 | High /CC2 | 26 | Low | 100 |  |  | 100 | 75 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for a CA_28A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Low | 41 | Low /CC1 | 41 | Low /CC2 | 50 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@25 |
| 2 | 28 | Mid | 41 | Mid /CC1 | 41 | Mid /CC2 | 50 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@25 |
| 3 | 28 | High | 41 | Mid /CC1 | 41 | Mid /CC2 | 50 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@25 |
| 4 | 28 | High | 41 | High /CC1 | 41 | High /CC2 | 50 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@25 |
| 5 | 41 | Low /CC1 | 41 | Low /CC2 | 28 | Low | 100 | 50 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 41 | Low /CC1 | 41 | Low /CC2 | 28 | Low | 100 | 100 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 7 | 41 | Mid /CC1 | 41 | Mid /CC2 | 28 | Mid | 100 | 100 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 8 | 41 | Mid /CC1 | 41 | Mid /CC2 | 28 | High | 100 | 100 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 9 | 41 | High /CC1 | 41 | High /CC2 | 28 | High | 100 | 50 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 10 | 41 | High /CC1 | 41 | High /CC2 | 28 | High | 100 | 100 |  |  | 50 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for a CA_28A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110 | 28 | Low | 42 | Mid | 42 | Mid | 25 | 100 |  |  | 25 |  |  | QPSK | All RBs |  |  | QPSK | 25@0 |
| 29 | 28 | Mid | 42 | High | 42 | High | 25 | 100 |  |  | 25 |  |  | QPSK | All RBs |  |  | QPSK | 25@0 |
| 3 | 28 | Low | 42 | Low /CC1 | 42 | Low /CC2 | 100 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@75 |
| 4 | 28 | High | 42 | High /CC1 | 42 | High /CC2 | 100 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 25@75 |
| 5 | 42 | Low /CC1 | 42 | Low /CC2 | 28 | Low | 100 | 25 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 6 | 42 | Low /CC1 | 42 | Low /CC2 | 28 | Low | 100 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 7 | 42 | High /CC1 | 42 | High /CC2 | 28 | High | 100 | 25 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| 8 | 42 | High /CC1 | 42 | High /CC2 | 28 | High | 100 | 100 |  |  | 100 |  |  | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_39C-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | Low/ CC1 | 39 | Low /CC2 | 41 | Mid | 100 |  | 50 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 2 | 39 | Low/ CC1 | 39 | Low /CC2 | 41 | Mid | 25 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 25@0 |
| 3 | 39 | High/ CC1 | 39 | High /CC2 | 41 | Mid | 100 |  | 50 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 4 | 39 | High/ CC1 | 39 | High /CC2 | 41 | Mid | 25 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 25@0 |
| 5 | 39 | Mid/ CC1 | 39 | Mid /CC2 | 41 | Mid | 25 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 25@0 |
| 6 | 41 | Mid | 39 | Low/ CC1 | 39 | Low /CC2 | 100 |  | 100 |  |  | 50 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 7 | 41 | Mid | 39 | High/ CC1 | 39 | High /CC2 | 100 |  | 100 |  |  | 50 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| Test Settings for CA_39A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | Low | 41 | Mid/ CC1 | 41 | Mid /CC2 | 100 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 2 | 39 | High | 41 | Mid/ CC1 | 41 | Mid /CC2 | 50 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 50@0 |
| 3 | 41 | Mid /CC1 | 41 | Mid/ CC2 | 39 | Low | 100 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 4 | 41 | Mid/ CC1 | 41 | Mid /CC2 | 39 | Low | 100 |  | 100 |  |  | 50 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 5 | 41 | Mid/ CC1 | 41 | Mid /CC2 | 39 | High | 100 |  | 100 |  |  | 100 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| 6 | 41 | Mid/ CC1 | 41 | Mid /CC2 | 39 | High | 100 |  | 100 |  |  | 50 | QPSK |  | All RBs |  |  | QPSK | 100@0 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table. Otherwise use the Default Test Settings test points. Note 3: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: X,Y correspond to the different bands in the CA Configuration. e.g. for CA_1C-3A, X=1,Y=3, for CA_1A_42C, X=1,Y=42. Note 5: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 6: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 7: Band 42: fUL/DL for SCC1 = 3430.2MHz (NUL/DL = 41892), fUL/DL for SCC2 = 3450MHz (NUL/DL = 42090) Note 8: Band 42: fUL/DL for SCC1 = 3500.2MHz (NUL/DL = 42592), fUL/DL for SCC2 = 3520MHz (NUL/DL = 42790). Note 9: Band 28: fUL = 719.3MHz (NUL = 27373), fDL = 774.3MHz (NDL = 9373). Note 10: Band 42: fUL/DL for SCC1 = 3515.8MHz (NUL = 42748), fUL/DL for SCC2 = 3527.5MHz (NDL = 42865). Note 11: Band 8: fUL = 897.5MHz (NUL = 21625), fDL = 942.5MHz (NDL = 3625), Band 42: fUL/DL = 3590MHz (NUL/DL = 43490). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-4: Test Configuration Table (Intra-band non-contiguous + Inter-band)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCCs RB allocation |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | Wgap [MHz] | SCC1 |  | Wgap [MHz] | SCC2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | SCC1 | SCC2 |  | PCC | SCC1 | SCC2 |  |  |
|  | Band | Range /CC |  | Band | Range /CC |  | Band | Range /CC |  |  |  |  |  |  |  |  |  |
| Group Test Settings for CA_2A-2A-XA or CA_XA-2A-2A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | CC2 | 35 | 2 | CC1 | - | X | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 16@57 |
| 2 | 2 | CC2 | 20 | 2 | CC1 | - | X | Mid | 100 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 16@57 |
| 3 | X | Mid | - | 2 | CC2 | 20 | 2 | CC1 | Highest NRB | 100 | 100 | QPSK | All RBs |  |  | QPSK | REFSENS |
| Group Test Settings for CA_3A-3A-XA or CA_XA-3A-3A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | CC2 | 50 | 3 | CC1 | - | X | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 16@50 |
| 2 | 3 | CC2 | 15 | 3 | CC1 | - | X | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 32@68 |
| 3 | 3 | CC2 | 35 | 3 | CC1 | - | X | Mid | 100 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 16@50 |
| 4 | X | Mid | - | 4 | CC2 | 35 | 4 | CC1 | Highest NRB | 100 | 100 | QPSK | All RBs |  |  | QPSK | REFSENS |
| Group Test Settings for CA_4A-4A-XA or CA_XA-4A-4A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 20 | 4 | CC1 | - | X | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 4 | CC2 | 5 | 4 | CC1 | - | X | Mid | 100 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | X | Mid | - | 4 | CC2 | 5 | 4 | CC1 | Highest NRB | 100 | 100 | QPSK | All RBs |  |  | QPSK | REFSENS |
| Test Settings for CA_4A-4A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 20 | 4 | CC1 | - | 12 | Mid | 100 | 25 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 4 | CC2 | 5 | 4 | CC1 | - | 12 | Mid | 100 | 100 | 50 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 12 | Mid | - | 4 | CC2 | 20 | 4 | CC1 | 50 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 20@30 |
| 4 | 12 | Mid | - | 4 | CC2 | 5 | 4 | CC1 | 50 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 8@426 |
| Test Settings for CA_4A-4A-13A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 | CC2 | 20 | 4 | CC1 | - | 13 | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 4 | CC2 | 5 | 4 | CC1 | - | 13 | Mid | 100 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | 13 | Mid | - | 4 | CC2 | 5 | 4 | CC1 | Highest NRB | 100 | 100 | QPSK | All RBs |  |  | QPSK | REFSENS |
| 4 | 4 | CC1 | 10 | 4 | CC2 | - | 13 | Mid | 50 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 50@0 |
| Group Test Settings for CA_42A-42A-XA or CA_XA-42A-42A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 42 | CC2 | 175 | 42 | CC1 | - | X | Mid | 100 | 25 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 42 | CC2 | 160 | 42 | CC1 | - | X | Mid | 100 | 100 | Highest NRB | QPSK | All RBs |  |  | QPSK | 100@0 |
| 3 | X | Mid | - | 42 | CC2 | 160 | 42 | CC1 | Highest NRB | 100 | 100 | QPSK | All RBs |  |  | QPSK | REFSENS |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 or Y=5, X =2 Note 5: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 6: Configuration for the low band for which the 3rd harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-1 Note 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-5: Test Configuration Table (Intra-band non-contiguous + Intra-band contiguous)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCCs RB allocation |  |  | CC MOD | PCC LCRB @ RBstart |
|  | Band | PCC | Wgap [MHz] | SCC1 | Wgap [MHz] | SCC2 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | SCC1 | SCC2 |  | PCC | SCC1 | SCC2 |  |  |
|  |  | CC |  | CC |  | CC |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_41C-41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 149 | CC2 | - | CC1 | 100 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 41 | CC3 | 134.2 | CC2 | - | CC1 | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| Test Settings for a CA_41A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | - | CC2 | 149.2 | CC1 | 100 | 100 | 25 | QPSK | All RBs |  |  | QPSK | 100@0 |
| 2 | 41 | CC3 | - | CC2 | 134.2 | CC1 | 100 | 100 | 100 | QPSK | All RBs |  |  | QPSK | 100@0 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 5: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 6: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.55.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-5 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.3A.5.4.3.

##### 7.3A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2. The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-5 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-5 as appropriate on PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in Table 7.3A.5.5-1 through Table7.3A.5.5-7. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex G.2A.

##### 7.3A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

#### 7.3A.5.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-1: Reference sensitivity QPSK PREFSENS for Intra-band contiguous

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_40D | 40 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
| CA_41D | 41 | - | - | - | -94.3 | -92.5 | -91.3 | TDD |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-2) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port |  |  |  |  |  |  |  |  |

Table 7.3A.5.5-2: Void

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-3 (originated from single carrier Tables 7.3.5-1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-3: Reference sensitivity QPSK PREFSENS for inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_1A-3A-5A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_1A-3A-7A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 7 | - | - | - | -94.3 | -92.5 | -91.3 |  |
| CA_1A-3A-8A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
| CA_1A-3A-19A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 19 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_1A-3A-41A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3(Note 12) | - | - | -93.7 | -90.8 | -89.7 | -88.3 |  |
|  | 3(Note 13) | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 41(Note 14) | - | - | -92.6 | -90 | -88.5 | -91.3 | TDD |
| CA_1A-3A-42A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 42 | - | - | -98 | -95 | -93.2 | -92 | TDD |
|  | 42 (Note 9) | - | - | -71.2 | -71.2 | -71.2 | -71.2 |  |
|  | 42 (Note 10) | - | - | -96.6 | -94.2 | -92.7 | -92 |  |
| CA_1A-3A-20A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 |  |
| CA_1A-3A-26A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  | 3 (Note 4) | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
|  | 26 |  |  | -96.86 | -93.86 |  |  |  |
| CA_1A-5A-7A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
| CA_1A-7A-20A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 7 | - | - | - | -94.3 | -92.5 | -91.3 |  |
|  | 20 | - | - | -96.3 | -93.3 | - | - |  |
| CA_1A-8A-11A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 8 | - | - | -96.3 | -93.3 | - | - |  |
|  | 11 | - | - | -99.3 | -96.3 | - | - |  |
| CA_1A-18A-28A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 18 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 28 | - | - | -97.8 | -94.8 | - | - |  |
|  | 28 (Note 7) | - | - | -93.3 | -91.8 | - | - |  |
| CA_1A-19A-21A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 19 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 21 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_1A-19A-28A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 19 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 28 | - | - | -97.8 | -94.8 | - | - |  |
|  | 28 (Note 11) | - | - | -93.3 | -91.3 | - | - |  |
| CA_1A-19A-42A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 19 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| CA_1A-21A-42A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 21 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| CA_1A-41A-42A | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 41 | - | - | - | -94.3 | -92.5 | -91.3 | TDD |
|  | 42 | - | - | - | -94.5 | -92.7 | -91.5 |  |
| CA_2A-4A-5A | 2 | - | - | -97 | -94 | -92.2 | -91 | FDD |
|  | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_2A-4A-12A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 2 (Note 8) | - | - | -97 | -94 | -92.2 | -91 |  |
|  | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
|  | 4 (Note 8) | - | - | -89.3 | -88.8 | -88.3 | -87.8 |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
|  | 12 (Note8) | - | - | -95.8 | -92.8 | - | - |  |
| CA_2A-4A-13A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
|  | 13 | - | - | - | -93.3 | - | - |  |
| CA_2A-4A-29A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2A-5A-12A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2A-5A-13A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 13 | - | - | - | -93.3 | - | - |  |
| CA_2A-5A-30A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_2A-12A-30A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_2A-29A-30A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_3A-7A-8A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 7 | - | - | - | -94.3 | -92.5 | -91.3 |  |
|  | 7 (Note8) | - | - | - | -86.7 | -86.3 | -86 |  |
|  | 8 | - | - | -96.3 | -93.3 | - | - |  |
|  | 8 (Note8) | - | - | -96.1 | -93.1 | - | - |  |
| CA_3A-7A-20A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 7 | - | - | - | -94.3 | -92.5 | -91.3 |  |
|  | 20 | - | - | -96.3 | -93.3 | -90.5 | -89.3 |  |
| CA_3A-19A-42A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 19 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 42 | - | - | -98 | -95 | -93.2 | -92 | TDD |
|  | 42 (Note 9) | - | - | -71.2 | -71.2 | -71.2 | -71.2 |  |
|  | 42 (Note 10) | - | - | -96.6 | -94.2 | -92.7 | -92 |  |
| CA_4A-5A-12A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_4A-5A-13A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 13 | - | - | - | -93.3 | - | - |  |
| CA_4A-5A-30A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_4A-7A-12A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_4A-12A-30A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_4A-29A-30A | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
| CA_7A-8A-20A | 7 | - | - | - | -94.3 | -92.5 | -91.3 | FDD |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
|  | 20 | - | - | -96.3 | -93.3 | - | - |  |
| CA_19A-21A-42A | 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 21 | - | - | -99.3 | -96.3 | -94.5 | - |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-4) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Reference sensitivity for the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. Note 5: Reference sensitivity for the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.2A-2). Note 8: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 and 6 in Table 7.3A.0-1. Note 9: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 9 and 10 in Table 7.3A.0-1. Note 10: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 11 in Table 7.3A.0-1. Note 11: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.2A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations

The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration.

The reference receive sensitivity (REFSENS) requirement for inter-band specified in Table 7.3A.5.5-3 shall be met for an uplink CA configurations than or equal to that specified in Table 7.3.5-2.

NOTE:	Table 7.3A.5.5-4 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.5.5-4: Void

For combinations of intra-band contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-5. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-5: Reference sensitivity QPSK PREFSENS for intra-band contiguous + inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_1A-3C (Note 9) | 1 | - | - | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 | - | - | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C (Note 10) | 1 | - | - | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 | - | - | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-41C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| CA_1A-42C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| CA_2C-29A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
| CA_3C-5A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_3A-7C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
| CA_8A-42C | 8 | -101.3 | -98.3 | -96.1 | -93.1 | - | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  | 42 (Note 8) | - | - | -83.8 | -83.7 | -83.6 | -83.5 |  |
| CA_3A-42C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 42 | - | - | -98 | -95 | -93.2 | -92.0 | TDD |
|  | 42 (Note 4) | - | - | -71.2 | -71.2 | -71.2 | -71.2 | TDD |
|  | 42 (Note 5) | - | - | -96.6 | -94.2 | -92.7 | -92.0 | TDD |
| CA_3C-7A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
| CA_19A-42C | 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| CA_21A-42C | 21 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -91.0 | TDD |
| CA_25A-41C | 25 | - | - | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| CA_26A-41C | 26 | - | - | -96.86 | -93.86 | -926 | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| CA_28A-41C | 28 | - | - | -97.8 | -94.8 | - | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| CA_28A-42C | 28 | - | - | -97.3 | -94.3 | -92.5 | -89.8 | FDD |
|  | 42 | - | - | -96.5 | -93.5 | -91.7 | -90.5 | TDD |
|  | 42 (Note 7) | - | - | -84.9 | -84.6 | -84.3 | -84.1 | TDD |
| CA_39A-41C | 39 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
| CA_39C-41A | 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
|  | 41 | - | - | - | - | - | -91.3 |  |
| CA_41A-42C | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_41C-42A | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-4) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 9 and 10 in Table 7.3A.0-1. Note 5: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 11 in Table 7.3A.0-1. Note 6: 6indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: Reference sensitivity for the high band for which the 5th harmonic of the low band is within transmission bandwidth, as specified Note 15 and 16 in Table 7.3A.0-1. Note 8: Reference sensitivity for the high band for which the 4th harmonic of the low band is within transmission bandwidth, as specified Note 12 and 13 in Table 7.3A.0-1. Note 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. Note 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-6. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-6: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + inter-band

| CA Configuration | Testpoint ID | PCC |  |  | SCC1 |  |  | SCC2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band | NRB | Requirement (dBm) | Band | NRB | Requirement (dBm) | Band | NRB | Requirement (dBm) |
| CA_2A-2A-XA or CA_XA-2A-2A | 1 | 2 | 100 | -91.3 | 2 | 25 | -90.11 | X | Highest NRB | PREFSENS |
|  | 2 | 2 | 100 | -91.3 | 2 | 100 | -86.71 | X | Highest NRB | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS | 2 | 100 | -91.3 | 2 | 100 | -91.3 |
| CA_3A-3A-XA or CA_XA-3A-3A | 1 | 3 | 100 | -90.3 | 3 | 25 | -89.81 | X | Highest NRB | PREFSENS |
|  | 2 | 3 | 100 | -90.3 | 3 | 25 | -96.3 | X | Highest NRB | PREFSENS |
|  | 3 | 3 | 100 | -90.3 | 3 | 100 | -86.26 | X | Highest NRB | PREFSENS |
|  | 4 | X | Highest NRB | PREFSENS | 3 | 100 | -90.3 | 3 | 100 | -90.3 |
| CA_4A-4A-XA or CA_XA-4A-4A | 1 | 4 | 100 | -93.3 | 4 | 25 | -99.3 | X | Highest NRB | PREFSENS |
|  | 2 | 4 | 100 | -93.3 | 4 | 100 | -93.3 | X | Highest NRB | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS | 4 | 100 | -93.3 | 4 | 100 | -93.3 |
| CA_42A-42A-XA or CA_XA-42A-42A | 1 | 42 | 100 | -92 | 42 | 25 | -98 | X | Highest NRB | PREFSENS |
|  | 2 | 42 | 100 | -92 | 42 | 100 | -92 | X | Highest NRB | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS | 42 | 100 | -92 | 42 | 100 | -92 |
| CA_4A-4A-12A | 1 | 4 | 100 | -93.3 | 4 | 25 | -99.3 | 12 | 50 | -93.5 |
|  | 2 | 4 | 100 | -93.3 | 4 | 100 | -93.3 | 12 | 50 | -93.5 |
|  | 3 | 12 | 50 | -93.5 | 4 | 100 | -93.3 | 4 | 25 | -99.3 |
|  | 4 | 12 | 50 | -93.5 | 4 | 100 | -87.87 | 4 | 100 | -87.87 |
| CA_4A-4A-13A | 1 | 4 | 100 | -93.3 | 4 | 25 | -99.3 | 13 | 50 | -93.3 |
|  | 2 | 4 | 100 | -93.3 | 4 | 100 | -93.3 | 13 | 50 | -93.3 |
|  | 3 | 13 | 50 | -93.3 | 4 | 100 | -93.3 | 4 | 100 | -93.3 |
|  | 4 | 4 | 50 | -96.3 | 4 | 100 | -93.3 | 13 | 50 | -93.3 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-4. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: PREFSENS refers to the PCC band’s single carrier reference sensitivity according to table 7.3.5-1. Note 6: Void Note 7: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 in Table 7.3A.0-1. |  |  |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and intra-band contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-7. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-7: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + intra-band contiguous

| CA Configuration | Testpoint ID | PCC |  | SCC1 |  | SCC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | Requirement (dBm) | NRB | Requirement (dBm) | NRB | Requirement (dBm) |
| CA_41A-41C | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
| CA_41C-41A | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-5. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: PREFSENS refers to the PCC band’s single carrier reference sensitivity according to table 7.3.5-1. |  |  |  |  |  |  |  |