3GPP TSG-RAN5 Meeting#82	R5-191796r1

# Athens, Greece, 25th Feb. – 1st Mar. 2019

Title:							On the FR2 OBW MU

Source:			Anritsu

Agenda Item:			5.3.8.17

Document for:			Endorsement

# 1.	Introduction

# In [1-2], the discussion for FR2 OBW MU has been provided. [2] discussed the QoQZ impact for OBW. This paper provides the analysis of the FR2 MU for OBW.

# 2.	Discussion

The OBW MU elements are comprised of conducted parts and OTA parts. Hence the basic idea to evaluate the FR2 OBW MU is that we can assume 1.5% of channel BW(same as FR1 NR) for contribution from conducted part and plus delta % from OTA parts. In addition to that, the impact from noise needs to be considered separately as the SNR is expected not so good compared to conducted test environment.

Observation 1 : MU for FR2 OBW comprises 1.5% of channel BW from conducted part, plus delta % from OTA parts and another delta % from noise.

In the below section, the analysis of the additional delta % MU from OTA parts is provided.

The frequency dependency of the MU will affect the measured occupied BW. How the frequency dependency affects the occupied BW depends on the shape of the error in frequency domain within the OBW measurement window. Here we evaluate the impact to the occupied bandwidth based on the simulation. The similar simulation as in [2] but with the different assumption for the shape of the frequency dependency. We adopt the scenario that both side of the spectrum is lower or higher compare to the centre of the channel as it is though that it affects the occupied bandwidth more. This scenario is possible when the frequency dependency has contiguous wave form and OBW measurement window is located at local max or local min as shown in Figure 1.

Figure 1 Image of frequency characteristic and OBW measurement window

To evaluate such scenario, the simulation is done for the model shown in Figure 2. Rectangular spectrum is assumed and the frequency characteristic is modeled by linear interpolation with 4 points.

Figure 2 Simulation model

Table 1 shows the list of MU elements for which impact for OBW is evaluated with the simulation. Only the elements in OTA parts having relatively big absolute uncertainty are chosen and other MU elements in OTA parts are considered as ignorable. Strictly says the amplifier uncertainty is conducted part element but can be considered as OTA part element in wide sense as it is used to compensate the big attenuation loss of OTA environment. We regard the absolute uncertainty values as the frequency characteristic of MU to calculate the impact to OBW, which could be a worst case assumption. The right most column shows the standard uncertainty for OBW for 400MHz bandwidth.

Table 1 Impact to OBW

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty () [dB] | Standard uncertainty () for OBW [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | Quality of Quiet Zone | 0.5 | Actual | 1 | 0.5 | 0.47 |
| 8 | Amplifier Uncertainties | 2 | Normal | 2 | 1.00 | 0.96 |
| 9 | Random Uncertainty | 0.5 | Normal | 2 | 0.25 | 0.23 |
| 22 | Uncertainty of the absolute gain of the calibration antenna | 0.6 | Normal | 2 | 0.30 | 0.27 |
| 25 | Quality of the Quiet Zone for calibration process | 0.2 | Actual | 1 | 0.20 | 0.18 |

# Taking the RSS of the standard uncertainties and also with the 3.06MHz(1.5% of 400MHz / 1.96) as impact from conducted part, we obtain +/- 3.27MHz as a standard uncertainty, and is corresponding to 1.6% of channel BW as expanded uncertainty.

# Proposal 1 : 1.6% of channel BW except for the impact from noise for FR2 OBW MU.

# Impact from noise to OBW can be calculated based on the assumed spectrum shape the target signal. Table 2 summarizes the result of the impact from the noise. OBW is calculated as the frequency width containing the power of 1% of total power in 2*BW. Rectangular spectrum for channel and flat spectrum for adjacent channel power of the target signal is assumed as depicted in Figure 3.

Table 2 Impact of noise to OBW ( 400MHz BW )

| ACLR [dBc] | SNR for channel power [dB] (NOTE1) | Measured OBW [MHz] | Measured OBW when SNR=+inf [MHz] | Difference [MHz] | Ratio to channel BW [%] |
| --- | --- | --- | --- | --- | --- |
| 16 | 0 |  | 788.2 |  | 636.76 |  | 151.44 |  | 37.86 |  |
|  | 10 |  | 760.83 |  |  |  | 124.08 |  | 31.02 |  |
|  | 15 |  | 723.28 |  |  |  | 86.52 |  | 21.63 |  |
|  | 20 |  | 680.96 |  |  |  | 44.2 |  | 11.05 |  |
|  | 25 | 654.12 |  | 17.36 | 4.34 |
|  | 30 |  | 642.7 |  |  |  | 5.94 |  | 1.49 |  |
|  | 40 | 637.37 |  | 0.62 | 0.15 |
| 100 | 0 | 788.0 | 396.0 | 392.0 | 98.0 |
| (NOTE2) | 10 | 752.0 |  | 352.0 | 89.0 |
|  | 15 | 665.51 |  | 269.51 | 57.38 |
|  | 20 | 399.92 |  | 3.92 | 0.98 |
|  | 25 | 397.25 |  | 1.25 | 0.31 |
|  | 30 | 396.4 |  | 0.4 | 0.1 |
|  | 40 | 396.04 |  | 0.04 | 0.01 |
| NOTE 1: SNR is -3dB of channel power – floor noise of the test system because twice the noise power will affect the measured OBW NOTE 2: Simulating the rectangular spectrum without no leakage to adjacent channel region |  |  |  |  |  |

Figure 3 Assumed spectrum

# The calculation spreadsheet used to derive the above values is attached below for information.

# We can observe from Table 2 that the noise impacts the measured OBW significantly. Ideally say, to make the noise ignorable level, SNR of >= 40dB will be required. However, it is thought to be challenging in the FR2 system. As the discussion of the achievable SNR and its impact to the MU is under the discussion, impact from noise to measured OBW needs to be aligned with that discussion. OBW is measured by the EIRP while setting the output power maximum, then the SNR should be aligned with that is defined for minimum peak EIRP in MOP test.

# Proposal 2 : Depending on the outcome of SNR discussion for minimum peak EIRP in MOP test, decide the impact from noise to measured OBW.

# 4.	Conclusion

Observation 1 : MU for FR2 OBW comprises 1.5% of channel BW from conducted part, plus delta % from OTA parts and another delta % from noise.

Proposal 1 : 1.6% of channel BW except for the impact from noise for FR2 OBW MU.

# Proposal 2 : Depending on the outcome of SNR discussion for minimum peak EIRP in MOP test, decide the impact from noise to measured OBW.

# 5.	References

[1] R5-188189, “On the FR2 MU for occupied BW and ACLR”, Anritsu, RAN5#81

[2] R5-190390, “On QoQZ MU Impact on OBW”, Keysight, RAN5 NR#4 AdHoc