3GPP TSG-RAN WG4 Meeting #82

Athens, GR, 25 Feb - 1 Mar, 2019

Source:        Samsung

Title:           TP analysis on FR1 6.5A.2.4.1.1 NR ACLR for CA (2UL CA)

# Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC NR ACLR, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR ACLR of 2UL inter-band CA case.

# Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined as table below.

Table 6.6.2.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 6 | 25 |  | 16QAM | 2 | P_1@5 | S_1@24 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@0 | S_8@0 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@1 | S_8@17 |  |  |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@11 | S_8@17 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@11 | S_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for SEM for single carrier mode is defined as table below.

Table 6.5.2.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest and Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration |  | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Low |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 23 | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 33 | Default |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 5 | High |  |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 16 | Low |  |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 17 | High |  |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 18 | Default |  |  |  |  | CP-OFDM QPSK | Outer_Full |
| 19 | Low |  |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 20 | High |  |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 21 | Default |  |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 22 | Low |  |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 25 | Low |  |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For Power Class 3 testing, include two steps for UE operating in bands n40, n77, n78 and n79, with IE [P-Boost-BPSK] set to 1 and 0 separately. |  |  |  |  |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

## Test Environment

In LTE, SEM for CA is tested under Normal condition. Hence it is proposed to apply normal condition as test environment to align with LTE case.

Proposal 1: Define test environment as Normal for SEM in FR1 for UL inter-band CA.

## Test Subcarrier Spacing

In [3] the smallest and highest SCS are selected for Single Carrier SEM verification. It is proposed to reuse the same way for inter-band UL CA.

Proposal 2: Select smallest and highest SCS for SEM in FR1 for inter-band UL CA.

## Test Frequencies

In LTE, SEM for Inter-band CA is tested for both Low range and High range. The same test coverage as for LTE would be enough to ensure that the SEM obeys the test requirement at low range and high range.

Proposal 3: Test SEM for Low range and High range frequencies in FR1 for inter-band UL CA.

## Test Channel Bandwidth

In LTE, SEM for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for SEM test for inter-band UL CA.

Proposal 4: Test SEM for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

## Test Modulations and waveforms

For single carrier TP analysis presented in [3] both DFT-s-OFDM and CP-OFDM for all modulation permutation are selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test SEM for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

## RB allocation

In LTE, SEM for CA are tested with edge partial allocation is located on the lower and upper channel edge for QPSK and 16QAM without test on 64QAM and 256QAM. For NR single carrier case, Outer_1RB_Left , Outer_1RB_Right and Outer_Full are selected in [3] . It’s proposed to verify the inter-band CA for SEM with Outer_1RB_Left and Outter_1RB_Right for QPSK and 16QAM. And Test on Outer_Full only for 64QAM and 256QAM.

Proposal 6: Test SEM with Outer_1RB_Left and Outer_1RB_Right for PI/2 BPSK , QPSK and 16QAM.

Proposal 7: Test SEM with Outer_Full for 64QAM and 256QAM.

## Number of test points for CA SEM

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 14 | 112 |

# Conclusion

As the inter-band UL CA NR ACLR requirement is defined the same as single carrier requirement with the update of condition of 2UL and 2DL CA active. Hence the test condition is proposed in the same way of NR single

Proposal 1: Define test environment as Normal for SEM in FR1 for UL inter-band CA.

Proposal 2: Select smallest and highest SCS for SEM in FR1 for inter-band UL CA.

Proposal 3: Test SEM for Low range and High range frequencies in FR1 for inter-band UL CA.

Proposal 4: Test SEM for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

Proposal 5: Test SEM for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

Proposal 6: Test SEM with Outer_1RB_Left and Outer_1RB_Right for PI/2 BPSK , QPSK and 16QAM.

Proposal 7: Test SEM with Outer_Full for 64QAM and 256QAM.

# Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-180886, "Discussion on test point selection for NR SEM in FR1"

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).