3GPP TSG-RAN5 Meeting #82	R5-192077r1

# Athens, Greece, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 0313 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR1 6.3A.3 Transmit ON/OFF time mask for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Transmit ON/OFF time mask for CA is currently missing from TS 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of 6.3A.3.0 minimum conformance requirements Addition of 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case Transmit ON/OFF time mask for CA will remain missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.0 6.3A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Core specification source: < 38101-1-f40 dec-18> R1: Modified 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA to 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) based on updated WP Typo corrections for minimum conformance requirements. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.3A.3	Transmit ON/OFF time mask for CA

6.3A.3.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in subclause 6.3.3.2 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in subclause 6.3.3.2 shall only be applicable for each component carrier when all the component carriers are OFF.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.3.

<<< Unchanged section omitted >>>

6.3A.3.1	Transmit ON/OFF time mask for CA (2UL CA)

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test config table is FFS

- Connection diagram is FFS in TS 38.508-1.

- MU/TT is FFS in Annex F

6.3A.3.1.1	Test purpose

To verify that the general ON/OFF time mask for CA (2UL CA) meets the requirements given in 6.3A.3.1.5

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power and transmit ON power symbols for CA.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3A.3.1.2.	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.3A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.

6.3A.3.1.4	Test description

6.3A.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.1.4.1-1: Test Configuration Table

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure [FFS] for TE diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.3.1.4.3.

6.3A.3.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause [TBD]. Message contents are defined in clause 6.3A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3A.3.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment for each component carrier is such that the UE transmits on slots 4 and 9 for 15kHz SCS, on slots 9 and 19 for 30kHz SCS and on slots 4, 9, 14, 19, 24, 29, 34 and 39 for 60kHz SCS.

5.	Measure the UE transmission OFF power for each component carrier during the slot prior to the PUSCH transmission, excluding a transient period of 10 µs in the end of the slot.

6.	Measure the output power of the UE PUSCH transmission for each component carrier during one slot.

7.	Measure the UE transmission OFF power of each component carrier during the slot following the PUSCH transmission, excluding a transient period of 10 µs at the beginning of the slot.

6.3A.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3A.3.3.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[5], Table 4.6.3-90 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -106 |  |  |
| } |  |  |  |

6.3A.3.3.5	Test requirement

The requirement for the power of each component carrier measured in steps 5, 6 and 7 of the test procedure shall not exceed the values specified in Table 6.3A.3.1.5-1.

Table 6.3A.3.1.5-1: General ON/OFF time mask

|  | SCS | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |
| Expected Transmission ON | 15 | -9.62±9.0 ±TT | -6.44±9.0 ±TT | -4.62±9.0 ±TT | -3.35±9.0 ±TT | -2.36±9.0 ±TT | -1.56±9.0 ±TT | -0.26±9.0 ±TT | 0.71±9.0 ±TT | N/A | N/A | N/A | N/A |
| Measured power for CP-OFDM | 30 | -10.18±9.0 ±TT | -6.79±9.0 ±TT | -4.79±9.0 ±TT | -3.51±9.0 ±TT | -2.46±9.0 ±TT | -1.67±9.0 ±TT | -0.34±9.0 ±TT | 0.65±9.0 ±TT | 1.51±9.0 ±TT | 2.77±9.0 ±TT | 3.30±9.0 ±TT | 3.77±9.0 ±TT |
|  | 60 | N/A | -7.17±9.0 ±TT | -5.03±9.0 ±TT | -3.78±9.0 ±TT | -2.67±9.0 ±TT | -1.78±9.0 ±TT | -0.50±9.0 ±TT | 0.55±9.0 ±TT | 1.40±9.0 ±TT | 2.71±9.0 ±TT | 3.25±9.0 ±TT | 3.72±9.0 ±TT |
| Expected Transmission ON | 15 | -9.62±9.0 ±TT | -6.61±9.0 ±TT | -4.85±9.0 ±TT | -3.60±9.0 ±TT | -2.53±9.0 ±TT | -1.56±9.0 ±TT | -0.26±9.0 ±TT | 0.71±9.0 ±TT | N/A | N/A | N/A | N/A |
| Measured power for DFT-s-OFDM | 30 | -10.59±9.0 ±TT | -6.79±9.0 ±TT | -5.03±9.0 ±TT | -3.60±9.0 ±TT | -2.53±9.0 ±TT | -1.84±9.0 ±TT | -0.59±9.0 ±TT | 0.48±9.0 ±TT | 1.51±9.0 ±TT | 2.75±9.0 ±TT | 3.21±9.0 ±TT | 3.72±9.0 ±TT |
|  | 60 | N/A | -7.58±9.0 ±TT | -5.03±9.0 ±TT | -3.78±9.0 ±TT | -2.81±9.0 ±TT | -2.02±9.0 ±TT | -0.59±9.0 ±TT | 0.48±9.0 ±TT | 1.17±9.0 ±TT | 2.42±9.0 ±TT | 3.21±9.0 ±TT | 3.72±9.0 ±TT |
| NOTE 1: TT of each transmit antenna for each frequency and channel bandwidth is specified in Table 6.3A.3.1.5-2 and Table 6.3A.3.1.5-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3A.3.1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |
| 40MHz < BW ≤ 100MHz | FFS | FFS |

Table 6.3A.3.1.5-3: Test Tolerance for ON powe

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |
| 40MHz < BW ≤ 100MHz | FFS | FFS |

<< End of changes >>