3GPP TSG RAN WG5 #82	R5-192195r1

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.521-4 | CR | N/A | rev |  | Current version: | 0.3.0 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for modification of PDSCH and PDCCH Config before measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM CDMA TECHNOLOGIES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | N/A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 38.521-4 WP requires addition of PDSCH & PDCCH Config params |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added parameters for PDSCH and PDCCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.521-4 WP cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R1: Added PTRS Configuration in Table C.1.2-2 |  |  |  |  |  |  |  |  |

<< Unchanged Section >>

Table C.1.1-2: Common reference channel parameters for FR1

| Parameter | Unit | Value |
| --- | --- | --- |
| CORESET frequency domain allocation |  | Full BW |
| CORESET time domain allocation |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  | Type A |
| PDSCH start symbol index (S) |  | 2 |
| Number of consecutive PDSCH symbols (L) |  | 12 |
| PDSCH PRB bundling | PRBs | 2 |
| Dynamic PRB bundling |  | false |
| Overhead value for TBS determination |  | 0 |
| First DMRS position for Type A PDSCH mapping |  | 2 |
| DMRS type |  | Type 1 |
| Number of additional DMRS |  | 2 |
| FDM between DMRS and PDSCH |  | Disable |
| TRS configuration |  | 2 slots, periodicity 10 ms, offset 0 |
| PTRS configuration |  | PTRS is not configured |

<< Start of changes >>

Table C.1.1-3: Additional reference channels parameters for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ Processes |  | 4 |
| K1 value |  | 2 for all slots |

<< Unchanged Section >>

## C.1.2 FR2 Setup

Table C.1.2-1 describes the downlink Physical Channels that are required for FR2 connection set up.

Table C.1.2-1: Downlink Physical Channels required for FR2 connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

PTRS

<< Start of changes >>

The following common PDSCH and PDCCH configuration parameters shall be used to bring up the connection setup for FR2 NR cell.

Table C.1.2-2: Common reference channel parameters for FR2

| Parameter | Unit | Value |
| --- | --- | --- |
| CORESET frequency domain allocation |  | Full BW |
| CORESET time domain allocation |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  | Type A |
| PDSCH start symbol index (S) |  | 2 |
| Number of consecutive PDSCH symbols (L) |  | 12 |
| PDSCH PRB bundling | PRBs | 2 |
| Dynamic PRB bundling |  | false |
| MCS table for TBS determination |  | 64QAM |
| Overhead value for TBS determination |  | 0 |
| First DMRS position for Type A PDSCH mapping |  | 2 |
| DMRS type |  | Type 1 |
| Number of additional DMRS |  | 2 |
| FDM between DMRS and PDSCH |  | Disable |
| TRS configuration |  | 1 slot, periodicity 10 ms, offset 0 |
| PTRS configuration |  | Every other RB, every symbol (K=2, L=1) |

Table C.1.2-3: Additional test parameters for TDD

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) |
| UL-DL configuration | referenceSubcarrierSpacing | 60 kHz | 120 kHz |
|  | dl-UL-TransmissionPeriodicity | 1.25 ms | 0.625 ms |
|  | nrofDownlinkSlots | 3 | 3 |
|  | nrofDownlinkSymbols | 4 | 10 |
|  | nrofUplinkSlot | 1 | 1 |
|  | nrofUplinkSymbols | 4 | 2 |
| Number of HARQ Processes |  | 8 | 8 |
| K1 value |  | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,39} | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} |