3GPP TSG-RAN WG4 Meeting # 110         	R4-2402956

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0058 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_netcon_repeater-Core) CR to TS 38.106 with corrections to NCR part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4#109 meeting finalized and agreed changes to repeater core specification with NCR introduction. However, some errors were done. This CR provide some corrections for NCR part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of clarification on mixed type of NCR in the scope section - Clause 3.1: Addition of definition of NCR type 1-C, 1-H and 2-O - Clause 3.3: Addition of abbreviations of NCR, NCR-MT, NCR-Fwd - Clause 4.5: addition of tables 4.5-1a and 4.5-1b to description of applicability of requirements. - Clause 6.5.2.2: removal of reference to table 6.5.2.2-3, as this is for UL WA and not UL LA. Editorial: “n” to “N” in tables 6.5.2.2-1, 6.5.2.2-1a, 6.5.2.2-3, 6.5.2.2-3a, 6.5.2.2-4, 6.5.2.2-4a - Clause 6.5.3.1: “basic” added for limit for multi-band connector of UL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will include missing or inconsistent parts. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 4.5, 6.5.2.2, 6.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of R4-2402242. |  |  |  |  |  |  |  |  |

<Start of CR to TS 38.106>

# 1	Scope

The present document establishes the minimum RF characteristics of NR Repeater and network controlled repeater. For network controlled repeater (NCR), requirements for NCR-Fwd and NCR-MT apply. In this version of the specification mixed type NCR are not considered.

<Next modified section>

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Antenna connector: connector at the conducted interface of the repeater type 1-C

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

Beam peak direction: direction where the maximum EIRP is found

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

directional requirement: requirement which is applied in a specific direction within the OTA coverage range.

Equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

Fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

gap between passbands: frequency gap between two consecutive passbands that belong to the same operating band, where the RF requirements in the gap are based on co-existence for un-coordinated operation

Inter-passband gap: The frequency gap between two supported consecutive passbands that belong to different operating bands.

Maximum passband output power: mean power level measured per passband at the antenna connector, during the transmitter ON state in a specified reference condition

Maximum passband TRP output power: mean power level measured per passband during the transmitter ON state in a specified reference condition and corresponding to the declared rated passband TRP output power (Prated,p,,TRP)

Measurement bandwidth: RF bandwidth in which an emission level is specified

multi-band connector: Antenna Connector for a Multi-band repeater.

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

NCR type 1-C: NCR-MT or NCR-Fwd at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

NCR type 1-H: NCR-MT or NCR-Fwd operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

NCR type 2-O: NCR-MT or NCR-Fwd operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

Nominal channel bandwidth: Bandwidth calculated as min(100MHz, BWpassband) in FR1 or min(400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

Non-contiguous spectrum: spectrum consisting of two or more passbands separated by inter-passband gap(s).

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA coverage range: a common range of directions within which OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

Passband: The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.

passband edge: Frequency at the edge of the passband

Radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

Rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction during the transmitter ON state

Rated passband output power: mean power level associated with a passband the manufacturer has declared to be available at the antenna connector, during the transmitter ON state in a specified reference condition

Rated passband TRP output power: mean power level declared by the manufacturer per passband, that the manufacturer has declared to be available at the RIB during the transmitter ON state

Rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the antenna connector, during the transmitter ON state in a specified reference condition

Rated total TRP output power: mean power level associated with a particular operating band, that the manufacturer has declared to be available at the RIB during the transmitter ON state in a specified reference condition

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

Repeater type 1-C: Repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

Repeater type 2-O: Repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Requirement set: one of the NR requirements set as defined for NR repeater

single-band connector: Repeater type 1-C antenna connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the repeater.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction.

Transmitter ON state: Time period during which the repeater is transmitting downlink or uplink signals in the corresponding direction.

Transmitter transient period: Time period during which the repeater is changing from the OFF state to the ON state or vice versa.

<Next modified section>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

AoA	Angle of Arrival

BW	Bandwidth

CACLR	Cumulative ACLR

CP-OFDM	Cyclic Prefix-OFDM

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

EIRP	Effective Isotropic Radiated Power

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NCR-Fwd	NCR Forward

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RIB	Radiated Interface Boundary

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

TRP	Total Radiated Power

UL	Uplink

WA	Wide Area

<Next modified section>

## 4.5	Applicability of requirements

In tables4.5-1, 4.5-1a and 4.5-1b the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | Repeater type 1-C | Repeater type 2-O |
| Repeater output power | 6.2 |  |
| Frequency stability | 6.3 |  |
| Out of band gain | 6.4 |  |
| Unwanted emissions | 6.5 |  |
| Error Vector Magnitude | 6.6 | NA |
| Input intermodulation | 6.7 |  |
| Output intermodulation | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 |  |
| Transmit ON/OFF power | 6.10 |  |
| Repeater output power |  | 7.2 |
| OTA frequency stability |  | 7.3 |
| OTA out of band gain |  | 7.4 |
| OTA unwanted emissions |  | 7.5 |
| OTA Error Vector Magnitude | NA | 7.6 |
| OTA input intermodulation |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  | 7.8 |
| OTA transmit ON/OFF power |  | 7.9 |

Table 4.5-1a: Requirement set applicability for NCR-Fwd

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-Fwd type 1-C | NCR-Fwd type 1-H | NCR-Fwd type 2-O |
| Repeater output power | 6.2 | 6.2 |  |
| Frequency stability | 6.3 | 6.3 |  |
| Out of band gain | 6.4 | 6.4 |  |
| Unwanted emissions | 6.5 | 6.5 |  |
| Error Vector Magnitude | 6.6 | 6.6 | NA |
| Input intermodulation | 6.7 | 6.7 |  |
| Output intermodulation | 6.8 | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 | 6.9 |  |
| Transmit ON/OFF power | 6.10 | 6.10 |  |
| Repeater output power |  |  | 7.2 |
| OTA frequency stability |  |  | 7.3 |
| OTA out of band gain |  |  | 7.4 |
| OTA unwanted emissions |  |  | 7.5 |
| OTA Error Vector Magnitude | NA | NA | 7.6 |
| OTA input intermodulation |  |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  |  | 7.8 |
| OTA transmit ON/OFF power |  |  | 7.9 |

Table 4.5-1b: Requirement set applicability for NCR-MT

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 2-O |
| Conducted transmitter power | 6.2.3.2 | 6.2.3.2 |  |
| Conducted output power dynamics | 6.11 | 6.11 |  |
| Conducted transmit signal quality | 6.12 | 6.12 |  |
| Conducted output RF spectrum emissions | 6.5 | 6.5 |  |
| Diversity characteristics | 6.13 | 6.13 | NA |
| Conducted reference sensitivity | 6.14 | 6.14 |  |
| Conducted maximum input level | 6.15 | 6.15 |  |
| Conducted adjacent channel selectivity | 6.16 | 6.16 |  |
| Conducted blocking characteristics | 6.17 | 6.17 |  |
| Conducted spurious response | 6.18 | 6.18 |  |
| Conducted intermodulation characteristics | 6.19 | 6.19 |  |
| Conducted spurious emissions | 6.5.5 | 6.5.5 |  |
| Radiated transmitter power |  |  | 7.2 |
| Radiated output power dynamics |  |  | 7.10 |
| Radiated transmit signal quality |  |  | 7.11 |
| Radiated output RF spectrum emissions |  |  | 7.5 |
| Diversity characteristics | NA | NA | 7.12 |
| Radiated reference sensitivity |  |  | 7.13 |
| Radiated maximum input level |  |  | 7.14 |
| Radiated adjacent channel selectivity |  |  | 7.15 |
| Radiated blocking characteristics |  |  | 7.16 |
| Radiated spurious response |  |  | 7.17 |
| Radiated intermodulation characteristics |  |  | 7.18 |
| Radiated spurious emissions |  |  | 7.19 |

<Next modified section>

#### 6.5.3.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) basic limits for DL are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. The values of ΔfOBUE are defined in table 6.5.1-1 for the NR operating bands.

Unless otherwise stated, the operating band unwanted emission (OBUE) basic limits for  UL are defined from ΔfOBUE below the lowest frequency of each supported uplink operating band up to ΔfOBUE above the highest frequency of each supported uplink operating band. The values of ΔfOBUE are defined in table 6.5.1-2 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification. In addition, for repeater operating in non-contiguous spectrum, the basic limits apply inside any gap between passbands. In addition, for a repeater operating in multiple bands, the basic limits apply inside any inter-passband gap.

Basic Limits are specified in the tables below, where:

-	Df is the separation between the passband edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the passband edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band of repeater type 1-C DL and uplink operating band of repeater type 1-C UL, where ΔfOBUE is defined in tables 6.5.1-1 and 6.5.1-2.

-	Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band connector inside any inter-passband gaps with Wgap < 2*ΔfOBUE, a combined basic limit shall be applied which is the cumulative sum of the basic limit requirement specified at the repeater type 1-C passband edges on each side of the inter-passband gap. The basic limit requirement for repeater type 1-C Bandwidth edge is specified in clauses 6.5.3.2.1 to 6.5.3.2.4 below, where in this case:

-	Df is the separation between the repeater type 1-C passband edge frequency and the nominal -3 dB point of the measuring filter closest to the repeater type 1-C passband edge.

-	f_offset is the separation from the repeater type 1-C passband edge frequency to the centre of the measuring filter.

-	f_offsetmax is equal to the inter-passband gap minus half of the bandwidth of the measuring filter.

-	Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band connector of DL, the operating band unwanted emission basic limits apply also in a supported downlink operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported downlink operating band. In this case, no cumulative basic limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and

-	In case the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted is less than 2*ΔfOBUE, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission  minimum requirement of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission basic limit of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (Dfmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

For a multi-band connector of UL, the operating band unwanted emission basic limits apply also in a supported uplink operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported uplink operating band. In this case, no cumulative basic limit is applied in the inter-band gap between a supported uplink operating band with carrier(s) transmitted and a supported uplink operating band without any carrier transmitted and

-	In case the inter-band gap between a supported uplink operating band with carrier(s) transmitted and a supported uplink operating band without any carrier transmitted is less than 2* ΔfOBUE, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported uplink operating bands and the operating band unwanted emission basic limit of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both uplink bands.

-	In other cases, the operating band unwanted emission basic limits of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (Dfmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported uplink operating band without any carrier transmitted.

In addition, inside any gap between passbands for a single-band connector operating in non-contiguous spectrum, a combined basic limit shall be applied which is the cumulative sum of the basic limit specified for the adjacent sub-blocks on each side of the gap between passbands. The basic limit for each sub-block is specified in clauses 6.5.3.2.1 to 6.5.3.2.4 below, where in this case:

-	Df is the separation between the sub-block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub-block edge.

-	f_offset is the separation between the sub-block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the gap between passbands bandwidth minus half of the bandwidth of the measuring filter.

-	Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For Wide Area repeater type 1-C, the basic limits of either clause 6.5.3.2.1 (Category A limits) or clause 6.5.3.2.2 (Category B limits) shall apply.

For Medium Range repeater type 1-C, the basic limits in clause 6.5.3.2.3 shall apply (Category A and B).

For Local Area repeater type 1-C, the basic limits of clause 6.5.3.2.4 shall apply (Category A and B).

The application of either Category A or Category B basic limits shall be the same as for Transmitter spurious emissions in clause 6.5.4.

In addition to, for the part of passband where there is no input signal at DL input port, the basic limits in Table 6.5.2.2-2 shall apply. In addition to, for the part of passband where there is no input signal at UL input port, the basic limits in clause 6.5.3.2.6 shall apply.

<End of CR to TS 38.106>