3GPP TSG-RAN4 Meeting RAN4#110	R4-2401621

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4154 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to TS 38.133 for RRM requirements for NR FR2 multi-Rx chain DL reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This Big CR is to capture contents for core part maintenance in endorsed draft CRs in the RAN4#110 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This big CR incorportes changes in the following draft CRs endorsed in RAN4#110 meeting: R4-2403340 DraftCR on maintaining L1-RSRP measurement requirements for FR2 multi-Rx, Huawei, HiSilicon R4-2403521 draft CR to 38.133 on capturing RTD side condition in the spec, Ericsson, Nokia |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for NR FR2 multi-Rx chain DL reception in Rel-18 is not finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1, 7.6.8 9.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

7.6	Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the MCG and the closest slot timing boundary of a cell belonging to SCG to be aggregated for EN-DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the SCG to be aggregated for NE-DC operation and the closest slot timing boundary of a cell belonging to MCG.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG in FR1 or FR2-1 and the closest slot timing boundary of a cell belonging to the SCG FR1 or FR2-1 to be aggregated for NR DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of a cell belonging to MCG in FR1 and the closest subframe timing boundary of a cell belonging to the SCG in FR2-2 to be aggregated for NR DC operation.

A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers in FR1 and/or FR2-1 to be aggregated in NR carrier aggregation.

A UE shall be capable of handling a relative receive timing difference among the closest subframe timing boundaries of different carriers to be aggregated in FR1 and FR2-2 NR inter-band carrier aggregation.

An FR2-1 PC6 UE supporting [simultaneousReceptionFR2HST-r18] shall be capable of handling a relative receive timing difference between the subframe boundaries of signals on the same CC received using two different Rx chains simultaneously in HST FR2 bidirectional deployment.

A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling a relative receive timing difference between the slot boundaries of signals on the same CC received from two TRPs.

A UE supporting [FG 30-1 or 30-2] shall be capable of handling a relative receive timing difference among the subframe timing boundary of different TCI states indicated for simultaneous reception on a single carrier.

The requirements defined in clause 7.6 are also applicable when UE is configured to receive multiple PDSCH transmission occasions from one or more QCL sources on any one of the aggregated NR carriers.

--- Unchanged clauses omitted ---

7.6.8	Minimum Requirements for Multi-TRPs

A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-1 below.

A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-2 below, provided that the UE indicates that it is capable of [FG 40-2-6].

Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [FG 40-2-6]

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR1 | CP lengthnote 1 |
| FR2-1 | CP lengthnote 1 |
| Note 1: CP length of the maximum SCS on the carrier. |  |

Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [FG 40-2-6]

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 |

A UE supporting [FG 30-1 or 30-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TCI states on the same carrier at the UE receiver as shown in Table 7.6.8-3.

Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR2-1 | CP lengthnote 1 |
| Note 1: CP length dependency on SCS is FFS |  |

<End of Change #1>

<Start of Change #2>

9.5.4	L1-RSRP measurement requirements

9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	The value of N in Table 9.5.4.1-2 and Table 9.5.4.1-3.is [reducedRxBeamNum] for UE supporting [faster beam switching capability] and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8.

Editor note: FFS how to capture UE is activated with multi-Rx operation.

-	The value of N in Table 9.5.4.1-2A is 8.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement GAP and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 9.1.8 and 9.1.10,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all -	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps , MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion nor non-dropped MUSIM gap occasion within the window W, and

-	Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	an SSB or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target SSB.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG measurement gap(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

P1= 11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P1 is PL1_sharing*Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P1=11-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P1= 11-TSSBmin(TSMTCperiod,xRP), when SSB is partially overlapped with GAP (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

-	If SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and P2 is valid accoding to 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P11-P1*TSSBP2*TSSB_CDP ,   if P1*TSSB < P2*TSSB_CDP.

-	P = P1, if P1*TSSB > P2*TSSB_CDP.

-	P = 2*P1, if P1*TSSB = P2*TSSB_CDP.

-	Otherwise, P = P1

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and Psharing_factor,CDP is used in 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-4 for UE incapable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-5 for UE capable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-6 for UE incapable of [capability of measurement with RTD>CP] in FR2 and Table 9.5.4.1-7 for UE capable of [capability of measurement with RTD>CP] in FR2 when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in Table 9.5.4.1-6 and Table 9.5.4.1-7.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

- 	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	PL1_sharing = 3.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet pervious conditions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based L1-RSRP measurement.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2A: Measurement period TL1-RSRP_Measurement_Period_SSB configured with groupBasedBeamReporting-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80ms< DRX ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: Scaling factor N1 = 2 when highSpeedMeasFlagFR2-r17 = set1 or scaling factor N1 = 6 when highSpeedMeasFlagFR2-r17 = [set2], if UE does not support [simultaneousReceptionFR2HST-r18] or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional. Scaling factor N1 = 1.5 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N1 = 4 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE supports [simultaneousReceptionFR2HST-r18] and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: If the actual RTD of serving cell and neighbor cell is larger than CP, [accuracy degradation is allowed or no requirements]. |  |

Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

Editor notes: The requirements for multiple layers will be updated based on newly achieved agreements.

Table 9.5.4.1-6: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.5.4.1-7: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

9.5.4.2	CSI-RS based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured CSI-RS resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 9.1.8 and 9.1.10,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps or NCSGs, MUSIM gap(s)and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any non-dropped measurement gap occasion nor non-dropped MUSIM gap occasion within the window W, and

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any non-dropped measurement gap occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	a CSI-RS or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG measurement gap(s), and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=Psharing factor, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=1, when aperiodic CSI-RS resource is not overlapped with GAP

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

Where:

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

TSMTCperiod = the configured SMTC period.

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if it overlapps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, a CSI-RS or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and CSI-RS resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the CSI-RS based L1-RSRP measurement.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. |  |

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Table 9.5.4.2-2A: Void

<End of Change #2>