3GPP TSG RAN Meeting #57 	RP-121201

# Chicago, USA, September 4th – 7th, 2012

# Source:	Nokia Siemens Networks

# Title:	proposal for a study item on LTE extensions to Minimization of Drive Tests

# Document for:	Discussion

Agenda Item:	13.2

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.

# Title * : LTE extensions to Minimization of Drive Tests

## Acronym * :

## Unique identifier *

## 1	3GPP Work Area *

| X | Radio Access |
| --- | --- |
|  | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification *

This work item is a … *

| X | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 460003 | Minimization of Drive Tests for E-UTRAN and UTRAN |  |
| 530133 | Rel-11 Enhancement of Minimization of Drive Tests for E-UTRAN and UTRAN |  |
| TBD | Small Cell Enhancements for E-UTRA and E-UTRAN – Physical-layer Aspects |  |
| TBD | Small Cell Enhancements for E-UTRA and E-UTRAN – Higher-layer Aspects |  |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) * |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is … *

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1		Stage 1

| Source of external requirements (if any) * |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2		Stage 2  *

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify: *

Go to §3.

#### 2.3.3		Stage 3 *

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) Or external document | Clause | Remarks |

If no identified source of stage 2 information, justify: *

Go to §3.

#### 2.3.4		Test spec *

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5		Other *

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4		Work task *

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification *

Manual drive tests for network optimization purposes are costly, contribute in environmental emission of CO2 and come with a limitation to measuring places along a drive route, though. Minimization of drive test (MDT) continue to progress in standardization as the underlying motives follow with the evolution of commercial market. It is highly beneficial to develop automated collection of MDT measurements, with both: UE and RAN node involvement, and thus minimize the need for operators to rely on manual process of monitoring and optimizing their networks.

Parent MDT Work Items aimed at defining methods in context of two use cases prioritized: coverage optimisation and QoS verification. The work towards completion of Rel-11 MDT content with a focus on the latter one, revealed a limitation to non-GBR (Guaranteed Bit Rate) type data statistics only. Hence, a support consisting of the network and potentially also UE involvement for QoS verification is needed. Therefore, Rel-12 work should follow Rel.11 objectives of QoS support in order to complement solutions for monitoring user experience and study a need for plain and evident enhancements on reflecting of user experience quality.

Successful roll-outs of the LTE network may require further analysing and prospective MDT procedures. The MDT evolution to Rel-12 should therefore ensure the feature serviceability in new upcoming deployments. In particular, small cell deployment appears to be up-and-coming change in future networks, hence standardization efforts on small cell enhancement solutions may yield a certain insight into MDT development . However, since small cell enhancements related requirements and features are not clearly defined yet it is therefore proposed to assign lower priority to potential studies of MDT support specific for small cell enhancements and consider relevant MDT improvements only if small cell enhancements studies determine such a need.

## 4	Objective *

The objective is to study MDT solutions towards:

- Enhanced QoS Verification use case support

  - Define QoS requirements for MDT measurements characterizing GBR traffic

  - Examine remaining motives for QoS verification where the radio interface is the bottleneck

  - Study logging and reporting capabilities for MDT measurements guaranteeing the negotiated bit rate, including a facilitated linkage of the measurements with location information

  - Analyze provision of improved feedback on QoS verification considering the impact to the UE

    - Consider a correlation feasibility of any new QoS measurement with location information provided as per MDT positioning principles

- Generic MDT improvements for distorted or redundant data collection

  - Study the need for MDT data filtering or MDT procedures adaptation for devices handling diverse traffic (e.g. IDC, eDDA)

- MDT extensions for small cell deployments

  - Based on the defined use cases, requirements and deployment scenarios identified under Small Cell Enhancements Study Item(s), study the necessity of adapting MDT measurements and/or procedures with respect to UE and NW impacts

Note: This study is projected based on Small Cell Enhancements Study Item(s) progress and outcome.

All the considerations for further MDT improvements should be based on analysis of the applicability, usefulness and feasibility, of the solutions before introducing additional standardised methods. In cases where there are alternative technical solutions, unless there is a clear advantage, priority should be given to solutions based on the existing UE and network measurements and procedures.

Further, the following principles should be followed when developing the solutions for MDT enhancements:

- Principles established for MDT in TS 37.320 are the baseline, i.e. both real time and non real time reporting are considered, measurements are configured to the UE by E-UTRAN by RRC signalling, based on Network management systems measurement definitions configured to E-UTRAN, the measurements from the UE can be combined/ processed with the network measurements already available in the E-UTRAN and sent to the MDT-entity outside the E-UTRAN.

- The use cases and related requirements should be refined with respect to the defined MDT baseline functionality.

- Duplication of the existing functionality should be avoided.

- Measurement configurations and functionalities should be identified for the targeted use cases.

- Impact to end user experience and system performance needs to be kept acceptable (e.g. MDT solutions should be developed so that UE power consumption can be kept reasonable when MDT is deployed and used in the networks as well as keeping complexity and processing requirements in network nodes to a reasonable level).

- UE complexity and memory requirements for MDT support need to be carefully considered.

The required activities to achieve these objectives include:

For RAN2:

- Study stage 1 aspects and define use cases and requirements for  Rel-12 MDT enhancements;

- Study how MDT measurements/logs fulfill the enhanced QoS use case requirements;

- Identify MDT requirements and use cases applicable for small cell enhancements taking into account the scenarios, requirements and detailed technical studies done under the Rel-12 small cell enhancements study item(s);

Other RAN WGs may be contacted if necessary.

## 5	Service Aspects

N/A

## 6	MMI-Aspects

N/A

## 7	Charging Aspects

N/A

## 8	Security Aspects

N/A

## 9	Impacts *

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

## 10	Expected Output and Time scale *

| New specifications * [If Study Item, one TR is anticipated] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| ??? | Study on further enhancements and extensions for MDT |  |  |  |  |  |  |  |  |
| Affected existing specifications * [None in the case of Study Items] |  |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |

## 11		Work item rapporteur(s) *

TBD

## 12		Work item leadership *

RAN2

Secondary responsible WG(s): RAN3, RAN1 and RAN4

## 13		Supporting Individual Members *

Supporting IM name