3GPP TSG RAN Meeting #79	RP-180183

Chennai, India,  19-23rd March 2018

Source: 	Thales, Ligado, Inmarsat, Globalstar, Dish network

Title:	NR-NTN update on spectrum L band GEO and S band HAPS

TDOC Type:	pCR

Agenda Item:	9.3.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification: 38.811  (SID = sFS_NR_nonterr_nw)

# Abstract

The objective of this document is to add a new band that can be used for NR operating via satellite.

# Proposed text for approval

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

## 4.8 Spectrum

This clause identifies the frequency bands that can be used for NR via Satellite or HAPS in the different ITU regions which are here below depicted:

Figure 4.8-1: ITU regions and the dividing lines between them.   Region 1 (Yellow), Region 2 (Blue), Region 3 (Purple)

### 4.8.1	Ka band spectrum for Satellite services

The following bands are identified for use by GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

The following bands are identified for use by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

### 4.8.2	S band spectrum for Satellite services

The following bands are identified for use by via GEO or Non GEO based Mobile Satellite services to provide International Mobile Telecommunications (IMT) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 2170-2200 MHz & 2483.5-2500 MHz | 2160-2200 MHz & 2483.5-2500 MHz | 2170-2200 MHz & 2483.5-2500 MHz |
| Uplink (earth to space) | 1980-2010 MHz | 1980-2025 MHz | 1980-2010 MHz |

### 4.8.3	S band spectrum for HAPS based services

The following bands as identified by ITU may be used by HAPS as base stations to provide International Mobile Telecommunications (IMT) on a worldwide basis

In FDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 2110-2170 MHz | 2110-2160 MHz (Note 1) | 2110-2170 MHz |
| Uplink (earth to aerial) | 1885-1980 MHz & 2010-2025 MHz | 1885-1980 MHz (Note 1) | 1885-1980 MHz & 2010-2025 MHz |

In TDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Up and Downlink (aerial to earth and earth to aerial) | 2110-2170 MHz | 2110-2160 MHz (Note 1) | 2110-2170 MHz |

Note 1: Specific regulatory limits and coexistence issues may prevent HAPS operation in some countries (such as USA) in some or all the above mentioned frequency bands.

### 4.8.4	L band spectrum for Satellite services

The following bands are used by GEO based Mobile Satellite Services (MSS) on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 1518 – 1559 MHz | 1518 – 1559 MHz | 1518 – 1559 MHz |
| Uplink (earth to space) | 1626.0 – 1660.5 MHz & 1668 – 1675 MHz | 1626.0 – 1660.5 MHz & 1668 – 1675 MHz (Note 2) | 1626.0 – 1660.5 MHz & 1668 – 1675 MHz |

Note 2: the frequency band 1668-1675 MHz is not allocated to MSS domestically in the US

The following bands are used by Non GEO based Mobile Satellite services  on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 1613.8 – 1626.5 MHz (Note 3) | 1613.8 – 1626.5 MHz (Note 3) | 1613.8 – 1626.5 MHz (Note 3) |
| Uplink (earth to space) | 1610.0 – 1626.5 MHz | 1610.0 – 1626.5 MHz | 1610.0 – 1626.5 MHz |

Note 3: The 1613.8-1626.5 MHz segment is allocated for downlink operations (space-to-Earth) only on a secondary basis.

### 4.8.5	Q/V band spectrum for Satellite services

The following bands are referred as Q and V bands and are applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 37.5 – 42.5 GHz, 47.5 - 47.9 GHz, 48.2-48.54 GHz & 49.44-50.2 GHz | 37.5 – 42.5 GHz | 37.5 – 42.5 GHz |
| Uplink (earth to space) | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz |

### 4.8.6	Ku band spectrum for Satellite services

The following bands are referred as Ku bands and are applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 10.7 - 12.75 GHz | 10.7 - 12.7 GHz | 10.7 - 12.75 GHz |
| Uplink (earth to space) | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz |

Note 1: the use of the bands 12.75-13.25 GHz, 10.7-10.95 GHz and 11.2-11.45 GHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

Note2: the use of the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3 by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30.

### 4.8.7	C band spectrum for Satellite services

The following bands are referred as C band and are applicable to GEO systems. The sub-bands 3700-4200 MHz and 5925-6425 MHz may be used by Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz |
| Uplink (earth to space) | 5725-7025 MHz | 5725-7025 MHz | 5725-7025 MHz |

Note: the use of the bands 4500-4800 MHz and  6725-7025 MHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

* * * End of Changes * * * *