3GPP TSG-RAN WG4 Meeting #95-e	R4-2008551

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.171 | CR | 0020 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS36.171, Introduction of BDS B1C in A-GNSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI |  |  |  |  |  | Date: |  |  | 2020-5-9 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the global B1C signal in the network-assisted BDS System, as part of A-GNSS positioning methods in LTE. Resubmission of endorsed Draft CR R4-2005286. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Endorsed in RAN4#94-e-bis: 1. Adding reference specification for BDS. 2. Adding symbol definition for B1C 3. BDS B1C case is added in section C.2 (Multi-path Case). 4. Models used for BDS B1C are added in section E.2 (GNSS Assistance Data) Additional Changes in RAN4#95-e: Further modifing reference [18] release from v2.0 to v3.0, and delete note in section B.1.5 based on comments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of specification support for Network-assisted BDS positioning method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, B.1.5, C.2, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[2]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[3]	3GPP TS 37.571-1: " User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance".

[4]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[5]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[6]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[7]	ETSI TR 102 273-1-2: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[8]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[9]	P. Axelrad, R.G. Brown, "GPS Navigation Algorithms", in Chapter 9 of "Global Positioning System: Theory and Applications", Volume 1, B.W. Parkinson, J.J. Spilker (Ed.), Am. Inst. of Aeronautics and Astronautics Inc., 1996.

[10]	S.K. Gupta, "Test and Evaluation Procedures for the GPS User Equipment", ION-GPS Red Book, Volume 1, p. 119.

[11]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)".

[12]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[13]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[14]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[15]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[16]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[17]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[18]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, December 2019.

[19]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[20]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[21]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[22]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017

========================= Second Change Request ===========================

3.2	Symbols

For the purposes of the present document, the following symbols apply:

B1I	BeiDou B1I navigation signal with carrier frequency of 1561.098 MHz.

B1C	BeiDou B1C navigation signal with carrier frequency of 1575.420 MHz.

E1	Galileo E1 navigation signal with carrier frequency of 1575.420 MHz.

E5	Galileo E5 navigation signal with carrier frequency of 1191.795 MHz.

E6	Galileo E6 navigation signal with carrier frequency of 1278.750 MHz.

G1	GLONASS navigation signal in the L1 sub-bands with carrier frequencies 1602 MHz ± k  562.5 kHz.

G2	GLONASS navigation signal in the L2 sub-bands with carrier frequencies 1246 MHz ± k  437.5 kHz.

k		GLONASS channel number, k = -7…13.

L1 C/A	GPS or QZSS L1 navigation signal carrying the Coarse/Acquisition code with carrier frequency of 1575.420 MHz.

L1C	GPS or QZSS L1 Civil navigation signal with carrier frequency of 1575.420 MHz.

L2C	GPS or QZSS L2 Civil navigation signal with carrier frequency of 1227.600 MHz.

L5	GPS or QZSS or NavIC L5 navigation signal with carrier frequency of 1176.450 MHz.

G	Geometry Matrix.

Measured pseudo-range of satellite i of GNSSm.

W	Weighting Matrix.

Line of sight unit vector from the user to the satellite i of GNSSm.

State vector of user position and clock bias.

========================= Third Change Request ===========================

B.1.5	Satellite constellation and assistance data

B.1.5.1	UE supports A-GPS L1 C/A only

In the case of test cases in clause 5 (UE supports A-GPS L1 C/A only), the GPS satellite constellation shall consist of 24 satellites. Almanac assistance data shall be available for all these 24 satellites. At least 9 of the satellites shall be visible to the UE (that is above 5 degrees elevation with respect to the UE). Other assistance data shall be available for 9 of these visible satellites. In each test, signals are generated for only a sub-set of these satellites for which other assistance data is available. The number of satellites in this sub-set is specified in the test. The satellites in this sub-set shall all be above 15 degrees elevation with respect to the UE. The HDOP for the test shall be calculated using this sub-set of satellites. The selection of satellites for this sub-set shall be selected consistent with achieving the required HDOP for the test.

B.1.5.2	UE supports other A-GNSSs

for GLONASS; 27 satellites for GPS, Modernized GPS and Galileo; 3 satellites for QZSS; 2 satellites for SBAS; 35 satellites for BDS (5 GEO, 27 MEO, 3 IGSO) and 7 satellites for NavIC (3GEO, 4 IGSO). Almanac assistance data shall be available for all these satellites. At least 7 of the satellites per GPS, Modernized GPS, Galileo, GLONASS and BDS constellation shall be visible to the UE (that is, above 15 degrees elevation with respect to the UE). At least 1 of the satellites for QZSS shall be within 15 degrees of zenith; at least 1 of the satellites of NavIC shall be visible to the UE above 15 degrees elevation angle over NavIC service area. (Service area as defined in ICD [20]) and at least 1 of the satellites for SBAS shall be visible to the UE. For BDS with reference location in Asia, at least 1 of the visible satellites shall be a GEO (above 15 degrees elevation with respect to the UE).All other satellite specific assistance data shall be available for all visible satellites.  In each test, signals are generated for only 6 satellites (or 7 if SBAS is included). The HDOP for the test shall be calculated using these satellites. The simulated satellites for GPS, Modernized GPS, Galileo, GLONASS and BDS shall be selected from the visible satellites for each constellation consistent with achieving the required HDOP for the test. For BDS with reference location in Asia, 1 of the simulated satellites shall be a GEO. For NavIC with reference location in the NavIC service area as defined in [20], the simulated satellite shall be an IGSO.

========================= Fourth Change Request ===========================

C.2	Multi-path Case

Doppler frequency difference between direct and reflected signal paths is applied to the carrier and code frequencies. The Carrier and Code Doppler frequencies of LOS and multi-path for GNSS signal are defined in table C.1.

Table C.1: Multipath Case

| Initial relative Delay [m] | Carrier Doppler frequency of tap [Hz] | Code Doppler frequency of tap [Hz] | Relative mean Power [dB] |
| --- | --- | --- | --- |
| 0 | Fd | Fd / N | 0 |
| X | Fd - 0.1 | (Fd-0.1) /N | Y |
| NOTE: Discrete Doppler frequency is used for each tap. |  |  |  |

Where the X and Y depends on the GNSS signal type and is shown in Table C.2, and N is the ratio between the transmitted carrier frequency of the signals and the transmitted chip rate as shown in Table C.3 (where k in Table C.3 is the GLONASS frequency channel number).

Table C.2: Parameter values

| System | Signals | X [m] | Y [dB] |
| --- | --- | --- | --- |
| Galileo | E1 | 125 | -4.5 |
|  | E5a | 15 | -6 |
|  | E5b | 15 | -6 |
| GPS/Modernized GPS | L1 C/A | 150 | -6 |
|  | L1C | 125 | -4.5 |
|  | L2C | 150 | -6 |
|  | L5 | 15 | -6 |
| GLONASS | G1 | 275 | -12.5 |
|  | G2 | 275 | -12.5 |
| BDS |  | B1I |  | 75 |  | -4.5 |  |
|  | B1C | 75 | -4.5 |

Table C.3: Ratio between Carrier Frequency and Chip Rate

| System | Signals | N |
| --- | --- | --- |
| Galileo | E1 | 1540 |
|  | E5a | 115 |
|  | E5b | 118 |
| GPS/Modernized GPS | L1 C/A | 1540 |
|  | L1C | 1540 |
|  | L2C | 1200 |
|  | L5 | 115 |
| GLONASS | G1 | 3135.03 + k 1.10 |
|  | G2 | 2438.36 + k 0.86 |
| BDS |  | B1I |  | 763 |  |
|  | B1C | 770 |

The initial carrier phase difference between taps shall be randomly selected between 0 and 2. The initial value shall have uniform random distribution.

========================= Fifth Change Request ===========================

E.2	GNSS Assistance Data

a)	GNSS- Reference Time IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.2: GNSS- Reference Time IE

| Information Element | All tests except Sensitivity Fine Time Assistance | Sensitivity Fine Time Assistance test |
| --- | --- | --- |
| GNSS-ReferenceTime |  |  |
| > gnss-SystemTime |  |  |
| >> gnss-TimeID | Yes | Yes |
| >> gnss-DayNumber | Yes | Yes |
| >> gnss-TimeOfDay | Yes | Yes |
| >> gnss-TimeOfDayFrac-msec | Yes | Yes |
| >> notificationOfLeapSecond | Yes if gnss-TimeID = ‘glonass’ | Yes if gnss-TimeID = ‘glonass’ |
| >> gps-TOW-Assist | Yes if gnss-TimeID = ‘gps’ | Yes if gnss-TimeID = ‘gps’ |
| > referenceTimeUnc | Yes | No |
| > gnss-ReferenceTimeForOneCell | No | Yes |
| >> networkTime |  | Yes |
| >>> secondsFromFrameStructureStart |  | Yes |
| >>> fractionalSecondsFromFrameStructureStart |  | Yes |
| >>> frameDrift |  | Yes |
| >>> cellID |  | Yes |
| >>>> physCellId |  | Yes |
| >>>> cellGlobalIdEUTRA |  | Yes |
| >> referenceTimeUnc |  | Yes |

b)	GNSS-ReferenceLocation IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.3: GNSS-ReferenceLocation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-ReferenceLocation | threeDlocation |

c)	GNSS-IonosphericModel IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.4: GNSS-IonosphericModel IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-IonosphericModel | KlobucharModelParameter |
|  | NeQuickModelParameter(1) |
|  | KlobucharModel2Parameter(2) |
| NOTE 1: Only required if GNSSs supported include Galileo. NOTE 2: Only required if GNSSs supported include BDS B1C. |  |

d)	GNSS-TimeModelList IE. This information element is only required for multi system tests, and is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.5: GNSS-TimeModelList IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-TimeModelList |  |
|  | gnssTOID For each GNSS included in the test. |
|  | deltaT |

e)	GNSS-NavigationModel IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.6: GNSS-NavigationModel IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-NavigationModel |  |

Table E.7: GNSS Clock and Orbit Model Choices

| GNSS | Clock and Orbit Model Choice |
| --- | --- |
| GPS | Model-2 |
| Modernized GPS | Model-3 |
| GLONASS | Model-4 |
| QZSS QZS-L1 | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3 |
| SBAS | Model-5 |
| Galileo | Model-1 |
| BDS B1I | Model-6 |
| BDS B1C | Model-7 |
| NavIC | Model-8 |

f)	GNSS-AcquisitionAssistance IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.8: GNSS-AcquisitionAssistance IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AcquisitionAssistance |  |

g)	GNSS-Almanac IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.9: GNSS-Almanac IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-Almanac |  |

Table E.10: GNSS Almanac Choices

| GNSS | Almanac Model Choice |
| --- | --- |
| GPS | Model-2 |
| Modernized GPS | Model-3,4 |
| GLONASS | Model-5 |
| QZSS QZS-L1 | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3,4 |
| SBAS | Model-6 |
| Galileo | Model-1 |
| BDS | Model-7 |
| NavIC | Model-8 |

h)	GNSS-UTC-Model IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.11: GNSS-UTC-Model IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-UTC-Model |  |

Table E.12: GNSS UTC Model Choices

| GNSS | UTC Model Choice |
| --- | --- |
| GPS | Model-1 |
| Modernized GPS | Model-2 |
| GLONASS | Model-3 |
| QZSS QZS-L1 | Model-1 |
| QZSS QZS-L1C/L2C/L5 | Model-2 |
| SBAS | Model-4 |
| Galileo | Model-1 |
| BDS | Model-5 |
| NavIC | Model-2 |

i)	GNSS-AuxiliaryInformation IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 37.355 [21].

Table E.13: GNSS-AuxiliaryInformation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AuxiliaryInformation |  |

Table E.10: GNSS Almanac Choices

| GNSS | Almanac Model Choice |
| --- | --- |
| GPS | Model-2 |
| Modernized GPS | Model-3,4 |
| GLONASS | Model-5 |
| QZSS QZS-L1 | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3,4 |
| SBAS | Model-6 |
| Galileo | Model-1 |
| BDS | Model-7 |

h)	GNSS-UTC-Model IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 36.355 [4].

Table E.11: GNSS-UTC-Model IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-UTC-Model |  |

Table E.12: GNSS UTC Model Choices

| GNSS | UTC Model Choice |
| --- | --- |
| GPS | Model-1 |
| Modernized GPS | Model-2 |
| GLONASS | Model-3 |
| QZSS QZS-L1 | Model-1 |
| QZSS QZS-L1C/L2C/L5 | Model-2 |
| SBAS | Model-4 |
| Galileo | Model-1 |
| BDS B1I | Model-5 |
| BDS B1C | Model-2 |

i)	GNSS-AuxiliaryInformation IE. This information element is defined in subclause 6.5.2.2 of 3GPP TS 36.355 [4].

Table E.13: GNSS-AuxiliaryInformation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AuxiliaryInformation |  |