Title:	Spatial Channel Model Text Description

File:	input.doc

Source:	Spatial Channel Model AHG (Combined ad-hoc from 3GPP & 3GPP2)

Date:	October 9, 2002

Notice

©2002 Third Generation Partnership Project Two (3GPP2).  All rights reserved.  Permission is granted for copying, reproducing, or duplicating this document only for the legitimate purposes of 3GPP2 and its organizational partners.  No other copying, reproduction, or distribution is permitted.

1	Introduction	3

1.1	Terminology (usage in 3GPP & 3GPP2)	3

1.2	Scope	4

1.2.1	System level evaluation.	4

1.2.2	Link level evaluation.	4

2	Link Level Channel Models for Calibration Purposes	4

2.1	Purpose	4

2.2	Link Level Channel Model Parameter Summary	4

2.3	Spatial Parameters per Path	6

2.4	BS and MS Array Topologies	6

2.5	Spatial Parameters for the BS	6

2.5.1	BS Antenna Pattern	6

2.5.2	Per Path BS Angle Spread (AS)	9

2.5.3	Per Path BS Angle of Arrival	9

2.5.4	Per Path BS Power Azimuth Spectrum	9

2.6	Spatial Parameters for the MS	10

2.6.1	MS Antenna Pattern	10

2.6.2	Per Path MS Angle Spread (AS)	10

2.6.3	Per Path MS  Angle of Arrival	10

2.6.4	Per Path MS Power Azimuth Spectrum	11

2.6.5	MS Direction of Travel	11

2.6.6	Per Path Doppler Spectrum	11

2.7	Generation of Channel Model	11

2.8	Calibration and Reference Values	12

3	System Level Spatial Channel Model	12

3.1	System Wide Definitions and Parameters	12

3.2	Scenarios	14

3.3	BS and MS Array Topologies	Error! Bookmark not defined.

3.3.1	Polarized arrays	16

3.3.2	Reference Antenna Configurations	17

3.4	Correlation Between Channel Parameters	17

3.4.1	Site-to-site correlation	17

3.4.2	Correlation between Delay Spread, Angle Spread, and Log Normal Shadowing	17

3.5	Generation of Channel Model	19

4	Reference	19

# Introduction

This document details the current discussion of the combined 3GPP-3GPP2 Spatial Channel Ad-hoc group.  A similar document, developed independently in the 3GPP2 Spatial Channel Modeling Ad-hoc group, was used for reference.

## Terminology (usage in 3GPP & 3GPP2)

In this document the following are terms that are commonly used interchangeably and are equivalent.  To promote consistency, the term on the left will be preferred in this document unless otherwise stated.

MS = Mobile Station = UE = User Equipment = Terminal = Subscriber Unit

BS = Base Station = Node-B = BTS

AS = Angle Spread = Azimuth Spread = PAS

Path = Ray

Path Component = Sub-ray

PAS = Power Azimuth Spectrum

DoT = Direction of Travel

AoA = Angle of Arrival

PDP = Power Delay Profile

## Scope

The scope of the 3GPP-3GPP2 SCM AHG is to develop and specify parameters and methods associated with the spatial channel modeling that are common to the needs of the 3GPP and 3GPP2 organizations (harmonization).  The scope includes development of specifications for:

### System level evaluation.

Within this category, a list of four focus areas are identified, however the emphasis of the SCM AHG work is on items a & b.

- Physical parameters (e.g. power delay profiles, angle spreads, dependencies between parameters)

- System evaluation methodology.

- Antenna arrangements, reference cases and definition of minimum requirements.

- Some framework (air interface) dependent parameters.

### Link level evaluation.

The link level models are defined only for calibration purposes. It is a common view within the group that the link level simulation assumptions will not be used for evaluation and comparison of proposals.

# Link Level Channel Models for Calibration Purposes

(Editor’s Note: A harmonized link level model is being proposed, and further discussion is expected prior to agreement.  This includes all aspects of the link level model described in Section 2.)

## Purpose

Link level simulations alone will not be used for algorithm comparison because they reflect only one snapshot of the channel behavior. Furthermore, they do not account for system attributes such as scheduling and HARQ. For these reasons, link level simulations do not allow any conclusions about the typical behavior of the system. Only system level simulations can achieve that. Therefore we require system level simulations for the final algorithm comparison.

Link level simulations will not be used to compare performance of different algorithms. Rather, they will be used only for calibration, which is the comparison of performance results from different implementations of a given algorithm.

## Link Level Channel Model Parameter Summary

(Editor’s Note: The following table is under discussion as a harmonized set of link level parameters and values.)

Table 2.2-1. Summary of Suggested SCM Link Level Parameters for Calibration Purposes

| Model |  |  | Case I |  | Case II |  | Case III |  | Case IV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding 3GPP Designator* |  |  | Case B |  | Case C |  | Case D |  | Case A |  |
| Corresponding 3GPP2 Designator* |  |  | Model A, D, E |  | Model C |  | Model B |  | Model F |  |
| PDP |  |  | Modified Pedestrian A |  | Vehicular A |  | Pedestrian B |  | Single Path |  |
| # of Paths |  |  | - 4+1 (LOS on, K = 6dB) - 4 (LOS off) |  | 6 |  | 6 |  | 1 |  |
| Relative Path Power (dB) |  | Delay (ns) | - 0.0 - -Inf | 0 | 0,0 | 0 | 0.0 | 0 | 0 | 0 |
|  |  |  | - -6.51 - 0.0 | 0 | -1.0 | 310 | -0.9 | 200 |  |  |
|  |  |  | - -16.21 - -9.7 | 110 | -9.0 | 710 | -4.9 | 800 |  |  |
|  |  |  | - -25.71 - –19.2 | 190 | -10.0 | 1090 | -8.0 | 1200 |  |  |
|  |  |  | - -29.31 - -22.8 | 410 | -15.0 | 1730 | -7.8 | 1730 |  |  |
|  |  |  |  |  | -20.0 | 2510 | -23.9 | 3700 |  |  |
| Speed (km/h) |  |  | - 3 - 30, 120 |  | 3, 30, 120 |  | 3, 30, 120 |  | 3 |  |
| UE/Mobile Station | Topology |  | Reference 0.5λ |  | Reference 0.5λ |  | Reference 0.5λ |  | N/A |  |
|  | PAS |  | - LOS on: Fixed AoA for LOS component, remaining power has 360 degree uniform PAS. - LOS off: PAS with a Lapacian distribution, RMS angle spread of 35 degrees per path |  | RMS angle spread of 35 degrees per path with a Lapacian distribution Or 360 degree uniform PAS. |  | RMS angle spread of 35 degrees per path with a Lapacian distribution |  | N/A |  |
|  | DoT (degrees) |  | 0 |  | 22.5 |  | -22.5 |  | N/A |  |
|  | AoA (degrees) |  | 22.5 (LOS component) 67.5 (all other paths) |  | 67.5 (all paths) |  | 22.5 (odd numbered paths), -67.5 (even numbered paths) |  | N/A |  |
| Node B/ Base Station | Topology |  | Reference: ULA with 0.5λ-spacing or 4λ-spacing or 10λ-spacing |  |  |  |  |  | N/A |  |
|  | PAS |  | Lapacian distribution with RMS angle spread of 2 degrees or 5 degrees, per path depending on AoA/AoD |  |  |  |  |  | N/A |  |
|  | AoD/AoA (degrees) |  | 50 for 2 RMS angle spread per path 20 for 5 RMS angle spread per path |  |  |  |  |  | N/A |  |

*Designators correspond to channel models previously proposed in 3GPP and 3GPP2 ad-hoc groups.

## Spatial Parameters per Path

Each resolvable path is characterized by its own spatial channel parameters (angle spread, angle of arrival, power azimuth spectrum). All paths are assumed independent. These assumptions apply to both the BS and the MS specific spatial parameters.  The above assumptions are in effect only for the Link Level channel model.

## BS and MS Array Topologies

The spatial channel model should allow any type of antenna configuration to be selected, although details of a given configuration must be shared to allow others to reproduce the model and verify the results.

Calibrating simulators at the link level requires a common set of assumptions including a specific set of antenna topologies to define a baseline case.  At the MS, the reference element spacing is 0.5. At the BS, three values for reference element spacing are defined: 0.5, 4, and 10.

## Spatial Parameters for the BS

### BS Antenna Pattern

The 3-sector antenna pattern used for each sector, Reverse Link and Forward Link, is plotted in Figure 2.5.1-1 and is specified by

(1)

θis defined as the angle between the direction of interest and the boresight of the antenna, θ3dB is the 3dB beamwidth in degrees, and  Am is the maximum attenuation.  For a 3 sector scenario θ3dB is 70o, Am=20dB,and the antenna boresight pointing direction is given by Figure 2.5.1-2.  For a 6 sector scenario θ3dB is 35o, Am=23dB, which results in the pattern shown in Figure 2.5.1-3, and the boresight pointing direction defined by Figure 2.5.1-4.  The boresight is defined to be the direction to which the antenna shows the maximum gain.  The gain specified by previous 3GPP documents[] for the 3-sector 70o antenna is 14dBi.  By reducing the beamwidth by half to 35o, the corresponding gain will be 3dB higher resulting in 17dBi. The antenna pattern shown is targeted for diversity oriented implementations (i.e. large inter-element spacings). For beamforming applications that require small spacings, alternative antenna designs may have to be considered leading to a different antenna pattern.

Figure 2.5.1-1  Antenna Pattern for 3-Sector Cells

Figure 2.5.1-2 Boresight pointing direction for 3-sector cells

Figure 2.5.1-3  Antenna Pattern for 6-Sector Cells

Figure 2.5.1-4 Boresight Pointing direction for 6-Sector Cells

### Per Path BS Angle Spread (AS)

The base station per path angle spread is defined as the root mean square (RMS) of angles with which an arriving path’s power is received by the base station array. The individual path powers are defined in the temporal channel model described in Table 2.2-1. Two values of BS angle spread (each associated with a corresponding mean angle of arrival, AoA) are considered:

-	AS: 2 degrees at AoA 50 degrees

- AS: 5 degrees at AoA 20 degrees

It should be noted that attention should be paid when comparing the link level performance between the two angle spread values since the BS antenna gain for the two corresponding AoAs will be different.  The BS antenna gain is applied to the path powers specified in Table 2.2-1.

### Per Path BS Angle of Arrival

The Angle of Arrival (AoA) or Angle of Departure (AoD) is defined to be the mean angle with which an arriving or departing path’s power is received or transmitted by the BS array with respect to the boresite. The two values considered are:

- AoA: 50 degrees (associated with the RMS Angle Spread of 2 degrees)

- AoA: 20 degrees (associated with the RMS Angle Spread of 5 degrees)

### Per Path BS Power Azimuth Spectrum

The Power Azimuth Spectrum (PAS) of a path arriving at the base station is assumed to have a Laplacian distribution. For an incoming AOA θ and RMS angle-spread , the BS per path PAS value at an angle  is given by expression  below:

P(θ,σ,θ)=Noexp−2θ−θσG(θ)

where both angles θand  are given with respect to the boresight of the antenna elements. It is assumed that all antenna elements’ orientations are aligned. Also, P is the average received power and G is the numeric base station antenna gain described in Section 2.5.1 by

G(θ)=100.1A(θ)

Finally, No is the normalization constant:

No−1=P(θ,σ,θ)dθ

In the above equation,  represents path components (sub-rays) of the path power arriving at an incoming AoA θ.  The distribution of these path components is TBD.

## Spatial Parameters for the MS

### MS Antenna Pattern

For each and every antenna element at the MS, the antenna pattern will be assumed omni directional with an antenna gain of -1 dBi.

### Per Path MS Angle Spread (AS)

The MS per path AS is defined as the root mean square (RMS) of angles of an incident path’s power at the MS array.  Two values of the path’s angle spread are considered:

-	AS: 104 degrees (results from a uniform over 360 degree PAS),

- AS: 35 degrees for a Laplacian PAS with a certain path specific Angle of Arrival (AoA).

### Per Path MS  Angle of Arrival

The per path Angle of Arrival (AOA) is defined as the mean of angles of an incident path’s power at the UE/Mobile Station array with respect to the broadside as shown Figure 2.6.3-5.

Figure 2.6.3-5. Angle of Arrival orientation at the MS.

Three different per path AoA values at the MS are suggested for the cases of a non-uniform PAS, see Table 2.2-1 for details:

- AoA: -67.5 degrees (associated with an RMS Angle Spread of 35 degrees)

- AoA: +67.5 degrees (associated with an RMS Angle Spread of 35 degrees)

- AoA: +22.5 degrees (associated with an RMS Angle Spread of 35 degrees or with an LOS component)

### Per Path MS Power Azimuth Spectrum

The Laplacian distribution and the Uniform distribution are used to model the per path Power Azimuth Spectrum (PAS) at the MS.

The Power Azimuth Spectrum (PAS) of a path arriving at the MS is modeled as either a Laplacian distribution or a uniform over 360 degree distribution. Since an omni directional  MS antenna gain is assumed, the received per path PAS will remain either Laplacian or uniform. For an incoming AOA θ and RMS angle-spread , the MS per path Laplacian PAS value at an angle  is given by expression  below:

P(θ,σ,θ)=Noexp−2θ−θσ,

where both angles θand  are given with respect to the boresight of the antenna elements. It is assumed that all antenna elements’ orientations are aligned. Also, P is the average received power and No is the normalization constant:

No−1=P(θ,σ,θ)dθ.

In the above equation,  represents path components (sub-rays) of the path power arriving at an incoming AoA θ.  The distribution of these path components is TBD.

### MS Direction of Travel

The mobile station direction of travel is defined with respect to the broadside of the mobile antenna array as shown in Figure 2.6.3-5.

### Per Path Doppler Spectrum

The per path Doppler Spectrum is defined as a function of the direction of travel and the per path PAS and AoA at the MS.  This should correspond to the per path fading behavior for either the correlation-based or ray-based method.

## Generation of Channel Model

The proponent can determine the model implementation. Examples of implementations include correlation or ray-based techniques.

Further details on examples of the model implementation are TBD.

Outline of methodology, including doppler spectrum filter required for correlation method.

## Calibration and Reference Values

For the purpose of link level simulations, reference values of the average correlation are given below in Table 2.8-2. The reference values are provided for the calibration of the simulation software and to assist in the resolution of possible errors in the simulation methods implemented. Specifically, the average complex correlation and magnitude of the complex correlation is reported between BS antennas and between MS antennas.  The spatial parameter values used are those defined already throughout Section 2. Nokia to provide formulas for computing correlation matrices

Table 2.8-2. Reference Correlation Values.

|  | Antenna Spacing | AS (degrees) | AOA (degrees) | Correlation (magnitude) | Complex Correlation |
| --- | --- | --- | --- | --- | --- |
| BS | 0.5λ | 5 | 20 | 0.9688 | 0.4743+0.8448i |
|  | 0.5λ | 2 | 50 | 0.9975 | -0.7367+0.6725i |
|  | 4λ | 5 | 20 | 0.3224 | -0.2144+0.2408i |
|  | 4λ | 2 | 50 | 0.8624 | 0.8025+0.3158i |
|  | 10λ | 5 | 20 | 0.0704 | -0.0617+i0.034 |
|  | 10λ | 2 | 50 | 0.5018 | -0.2762-i0.4190 |
| MS | λ/2 | 104 | 0 | 0.3042 | -0.3042 |
|  | λ/2 | 35 | -67.5 | 0.7744 | -0.6948-i0.342 |
|  | λ/2 | 35 | 22.5 | 0.4399 | 0.0861+0.431i |
|  | λ/2 | 35 | 67.5 | 0.7744 | -0.6948+i0.342 |

# System Level Spatial Channel Model

A Spatial Channel Model for use at the system level is described below. Parameter values and their ranges are specified to enable the generation of a Spatio-Temporal channel model for use in evaluating the system-level performance of multiple antenna schemes.

## System Wide Definitions and Parameters

The received signal at the MS consists of N time-delayed multipath replicas of the transmitted signal. These N paths are  defined by powers and delays  and are chosen randomly according to the channel generation proceedure. Each path consists of M subpaths which will be described later.

Figure 2.5.1-1 shows the angular parameters used in the proposed model. The following definitions are used:

ΩBS	Base Station orientation. It is defined as the difference between the broadside of the BS array and a North (N) reference direction.

θ0	  Angle of the LOS direction between the BS and MS, referenced to the broadside of the BS array.

δi	mean angle of arrival difference for the nth (n = 1 … N) path relative to the LOS AOA θ0.

θl,i	Absolute AOA for the mth (m = 1 … M) subpath of the nth path at the BS with respect to the BS broadside.

θ0,MS	Angle between the BS-MS LOS and the MS broadside.

ΩMS	MS orientation. It is defined as the difference between the broadside of the MS array and the North reference direction.

θv,MS	The angle reference of the MS velocity vector.

The angles shown in Figure 1 that are measured in a clockwise direction are assumed to be negative in value.

Figure 2.6.6-6  BS and MS angle parameters

For system level simulation purposes, the fast fading per-path will be evolved in time, although bulk parameters including Angle Spread, Delay Spread, Log Normal Shadowing, and MS location will remain fixed during the evaluation of the given MS.

## Scenarios

To limit the number of cases for consideration, the following represent a definition of the unique environments for simulation.

- Suburban Macro (approximately 3Km distance BS to BS)

- Urban Macro (approximately 3Km distance BS to BS)

- Urban Micro (less than 1Km distance BS to BS)

The macro cell definition applies when the base station antennas are above rooftop height. The micro cell definition applies otherwise. The following table describes the parameters used in each of the scenarios. Note that in addition to the parameters highlighted in yellow, some of the mu and epsilon parameters have also not been finalized.

Table 3. Scenario parameters

| Channel Scenario | Suburban Macro | Urban Macro | Urban Micro |
| --- | --- | --- | --- |
| Mean composite AS at BS | E(AS)=50 | E(AS)=80, 150 | N/A |
| Composite AS at BS as a lognormal RV when simulating with 6 paths AS=10Ax+A, x~N(0,1) | A= 0.69 A= 0.17 | A= 0.87 A= 0.36 | N/A |
| Per path AS at BS (Fixed) | 2 deg | 2 deg | 5 deg |
| Per path AoD Distribution st dev | N(0, AS) | N(0, AS) | U(-30deg, 30deg) |
| Mean of RMS composite AS at UE | E(AS, comp,UE)=720 | E(AS, comp, UE)=720 | E(AS, comp, UE)=720 |
| Per path AS at UE (fixed) | 350 | 350 | 350 |
| Per path AoA Distribution | N(0,AoA2(Pr)) Note 1 | N(0,AoA2(Pr)) Note 1 | N(0,AoA2(Pr)) Note 1 |
| Mean total RMS Delay Spread | E(DS)=0.17 s | E(DS)=0.65 s | U(0,0.8) s |
| Narrowband composite delay spread as a lognormal RV when simulating with 6 paths DS=10Dx+D, x~N(0,1) | D = - 0.80 D = 0.288 | D = -0.175 D = 0.17 |  |
| Lognormal shadowing standard deviation | 8dB | 8dB | N/A |

Note 1, Per path AoA is chosen from Normal distribution whose sigma is a function of the relative of path power,  N(0,AoA2(Pr)), where 0 = LOS, and AoA = 104.12(1-exp(-0.2175*|Pr|), with Pr equal to the fraction of power in the given path in dBr.

From these parameters, realizations of the user parameters such as the path delays, powers, and subpath angles of departure and arrival can be derived using the following 12-step procedure.

- Step 1: Choose a single channel scenario common to all drops (i.e. to be applied to the entire simulation).

- Step 2: Generate Drops. Assign geometry (LOS direction and distance of UE from NodeB), UE antenna structure orientation, UE speed vector direction and magnitude.

- Step 3: Select lognormal random draws for DS, AS (at NodeB: σASk), LN as described in Section 3.4.2 below.

- Step 4: Assign N = 6 paths (for macro channels). Assign 6 random delays. Delays are ordered and the minimum delay is subtracted from all so that the first delay is always zero. k is the drop and UE index. Ratio values r  are r=1.17 suburban macro, r=1.41 urban macro. Realization of random delays are made according to the model below:

τn=−rDSσDSklogzn        zn:U(0,1)

Step 5: Assign a power to each path n:

Pn=e(1−rDS)⋅τnrDS⋅σDSk⋅10−ξn    where    ξn:N(0,σ2RND)

where σRND is a shadowing randomization effect on the per-path powers. Value is TBD. Powers are normalized so that total power (for all six paths is equal to one).

- Step 6: Generation of AODs per path and ordering at NodeB. Random draws of AODs from N(0,r2AOD,CS⋅σ2ASk). The rAOD,CS=1.07 (0.3 in dB) (Suburban macro), rAOD,CS=1.3  (1.0 in dB) (urban macro).

- Step 7: The 6 AODs generated in Step 6 are ordered in increasing absolute value and each of the six delays (Step 4) is assigned to each AOD. Increasing delays are matched to increasing relative AOD angle δn is defined in Figure 2.6.6-6 based on their absolute value and deterministically.

- Step 8: 20 sub-rays are used to generate a 2o Laplacian spread for each path (ray) at the NodeB.  All 20 sub-rays have identical powers (1/20 of the path power) but random phase. Sub-components have a predefined fixed angle distribution as shown in Table 4.

- Step 9: At the UE, assign per-path (ray) AOA variance as a function of the path (ray) relative power. Draw the relative AOA (with respect to LOS) from a distribution: N(0,σ2AOA).  Where 0 = LOS, andAoA = 104.12(1-exp(-0.2175*|Pr|), with Pr equal to the fraction of power in the given path in dBr.

- Step 10: 20 sub-rays are used to generate a 35o Laplacian spread for each path (ray) at the UE.  All 20 sub-rays have identical powers (1/20 of the path power) but random phase.  Sub-components have a predefined fixed angle distribution as shown in Table 4.

- Step 11: Pairing of each NodeB sub-ray with a corresponding UE sub-ray is made to create the actual channel gain. Random pairing is used.

- Step 12: Assign antenna gains to NodeB paths (rays). Assign the antenna gain to each UE sub-ray.

Table 4. Fixed Sub-path components to produce per-path spreads

| Sub-path # | (+/-) degrees for a 2 deg Laplacian (Node-B Macro-cell) | (+/-) degrees for a 5 deg Laplacian (Node-B Micro-cell) | (+/-) degrees for a 35 deg Laplacian (UE) |
| --- | --- | --- | --- |
| 1, 2 | 0.08716 | 0.2179 | 1.525 |
| 3, 4 | 0.2754 | 0.6884 | 4.819 |
| 5, 6 | 0.4858 | 1.215 | 8.502 |
| 7, 8 | 0.7243 | 1.811 | 12.68 |
| 9, 10 | 0.9997 | 2.499 | 17.49 |
| 11, 12 | 1.325 | 3.313 | 23.19 |
| 13, 14 | 1.724 | 4.310 | 30.17 |
| 15, 16 | 2.238 | 5.595 | 39.16 |
| 17, 18 | 2.962 | 7.405 | 51.84 |
| 19, 20 | 4.201 | 10.50 | 73.51 |

## BS and MS Array Topologies

The spatial channel model should allow any type of antenna configuration to be selected, although details of a given configuration must be shared to allow others to reproduce the model and verify the results.  It is intended that the spatial channel  model be capable of operating on any given antenna array configuration.

### Polarized arrays

Practical antennas on handheld devices require spacings much less than l/2.

Polarized antennas are TBD.

### Reference Antenna Configurations

In order to compare algorithms, reference antenna configurations based on uniform linear array configurations with 0.5, 4, and 10 wavelength inter-element spacing will be used.

## Correlation Between Channel Parameters

### Site-to-site correlation

A value of 50% site-to-site correlation will be assumed for the log-normal shadow fading component.

### Correlation between Delay Spread, Angle Spread, and Log Normal Shadowing

In [], Greenstein presents a model for correlating delay spread (DS) with log normal (LN) shadow fading.  Since both are shown to be log-normal distributed, the correlation between the DS and LN are correlated by the coefficient .  The best value for suburban and urban data was shown to be  = -0.75, presented in [Error: Reference source not found] from data measured by [].

The result of the correlation between log normal shadowing and delay spread is significant because it indicates that for a strong signal (positive LN), the DS is reduced, and for a weak signal condition (negative LN), the DS is increased.

Cost 259[] presents the azimuth spread (AS) as also being log-normal distributed, and likewise being correlated to the DS and LN.  Since the correlation of these parameters is quite high, a spatial channel model needs to be specified that can reproduce this correlation behavior along with the expected probability and range of each parameter.  For a macro-cell environment, the following values are given in [Error: Reference source not found]:

 = Correlation between DS & AS = +0.5

 = Correlation between LN & AS = -0.75

 = Correlation between LN & DS = -0.75

The random variables for correlating DS, AS, and LN shadowing are generated using

αnβnγn=c11c12c13c21c22c23c31c32c33wn1wn2wn3,			(1)

where n refers to the nth base station (BS).  and are the correlated zero-mean unit variance Gaussian distributed random variables associated with the DS, AS, and LN, respectively, and wn1, wn2, and wn3 are unit-variance, independent Gaussian noise samples.  Equation 1 is performed once for each BS per drop.

c11c12c13c21c22c23c31c32c33=ρααραβραγρβαρββρβγργαργβργγ12,			(2)

Equation 2 incorporates the matrix square root function of correlation coefficients.

The LN shadow fading component is typically correlated between Base Stations (BS).  To produce correlated shadow fading components () between BS1 and BS2, a simplified approach is shown where the independent unit-variance Gaussian noise samples w13 and w23 are replaced with correlated, unit-variance Gaussian noise samples that are generated using:

w13=ξcccc33+1−ccc332ξ1				(3)

and

w23=ξcccc33+1−ccc332ξ2,			(4)

where c, 1, 2 are independent, unit-variance Gaussian noise samples and cc is the desired site-to-site shadow fading correlation between the MS and nodes BS1 and BS2.

The cumulative distribution functions of DS, LN,  and AS are described in terms of the correlated Gaussian random variables that characterize their distributions [Error: Reference source not found][Error: Reference source not found][].

Using nomenclature from[Error: Reference source not found] the distribution of DS is given by:

Dn = 10^(Dn +D)					(5)

where n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the DS for the nth BS, D = E{log10(DS)} is the logarithmic mean of the distribution of DS,  and D = Std{log10(DS)} is the logarithmic standard deviation of the distribution of DS.

Similarly the distribution of AS is given by:

An = 10^(An +A)					(6)

where	n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the AS, A = E{log10(AS)} is the logarithmic mean of the distribution of AS,  and A = Std{log10(AS)} is the logarithmic standard deviation of the distribution of AS.

Finally, the distribution for the LN is given by:

LN = 10^(SFn/10),					(7)

where	n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the LN, SF (shadow fading) is obtained from analysis of the standard deviation from the regression line of the path loss versus distance, and the value is given in dB.  Since the LN is normally used as a dB value, there is no need to convert it to linear, but rather simply use the quantity SFn.

Correlated channel parameters may be drawn within a system simulation based on the equations described above.

## System Level Calibration

The following examples are given for calibration purposes. A resolvable path at the receiver is assumed to be the energy from one (or more) paths falling within one chip interval. The Chip rate in UMTS is 3.84Mcps. The PDF of the number of resulting resolvable paths is recorded. Only the paths with power higher than    –15dB relative to the strongest path are recorded.  The shown statistics are the non-power weighted ones. In the same plots, the average relative powers of each resolvable path are given in the upper right corner. Per-resolvable path angle spread statistics at the BS/NodeB are also reported.

(The following figures labeled 2,3, & 4, are to be considered place holders until updated plots can be generated, which incorporate all the parameter values and attributes described above in the 12 step process and the table of parameters.)

-Channel Scenario: Suburban Macro

Figure 2. Statistics for Suburban Macro Channel for chip-rate frequency = 3.84MHz (clockwise from top left): (a) CDF of total rms DS, (b) Probabilities for number of resolvable paths present (in occurrences per 10,000 drops). (c) CDF of 1st resolvable path’s AS, (d) CDF of Composite AS

-Channel Scenario: Urban Macro

Figure 3. Statistics for Urban Macro Channel for chip-rate frequency = 3.84MHz (clockwise from top left): (a) CDF of total rms DS, (b) Probabilities for number of resolvable paths present (in occurrences per 10,000 drops). (c) CDF of 1st resolvable path’s AS, (d) CDF of Composite AS

Channel Scenario: Urban Microcellular

Figure 4 Statistics for Urban Microcellular Environment with chip-rate frequency = 3.84MHz (clockwise from top left): (a) CDF of total rms DS, (b) Probabilities for number of resolvable paths present (in occurrences per 10,000 drops). (c) CDF of Composite AS

# Reference

[?] WG5 Evaluation Methodology – Addendum (V6), WG5 Evaluation AHG, July 25, 2001

[?] Nokia, Ericsson, Motorola.  Common HSDPA system simulation assumptions. TSG-R1 document, TSGR#15(00)1094, 22-25th, August, 2000, Berlin, Germany, 12 pp.

[?] L. Greenstein, V. Erceg, Y. S. Yeh, M. V. Clark, “A New Path-Gain/Delay-Spread Propagation Model for Digital Cellular Channels,” IEEE Transactions on Vehicular Technology, VOL. 46, NO.2, May 1997, pp.477-485.

[?] E. Sousa, V. Jovanovic, C. Daigneault, “Delay Spread Measurements for the Digital Cellular Channel in Toronto,” IEEE Transactions on Vehicular Technology, VOL. 43, NO.4, Nov 1994, pp.837-847.

[?] L. M. Correia,  Wireless Flexible Personalized Communications, COST 259: European Co-operation in Mobile Radio Research, Chichester:  John Wiley & Sons, 2001.

[?]  A. Algans, K. I. Pedersen, P. Mogensen, “Experimental Analysis of the Joint Statistical Properties of Azimuth Spread, Delay Spread, and Shadow Fading,” IEEE Journal on Selected Areas in Communications, Vol. 20, No. 3, April 2002, pp. 523-531.