3GPP TSG RAN WG1 Meeting #100-e 	R1-2000194

February 24 - March 6, 2020

Agenda Item:	7.2.8.5

Source:	Huawei, HiSilicon

Title:	RAN procedures for NR positioning

Document for:	Discussion and Decision

Introduction

In RAN3#106, the following NRPPa message structure was agreed [2] [3].

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |

In this contribution, we going to discuss the signaling details for the following messages:

POSITIONING INFORMATION REQUEST (LMF  serving gNB)

POSITIONING INFORMATION RESPONSE (serving gNB  LMF)

MEASUREMENT REQUEST (LMF  NG-RAN)

MEASUREMENT RESPONSE (NG-RAN  LMF)

- There was an LS from RAN3 (R3-197794) [4] to RAN2; although RAN1 is not in the recipient’s list, we also think that some analysis from RAN1 perspective is helpful in determining the signaling in NG-RAN to support DL-AoD. Therefore, we also provide our view on DL-AoD.

Discussion on UL measurements

POSITIONING INFORMATION REQUEST

The SRS configuration is request by the LMF to the serving gNB. In LTE, the request information includes [1]

Number of transmissions

Bandwidth

However, in NR, since we are envisioning a more complex SRS transmission feature including multiple SRS resources and multiple SRS resource sets, more information should be provided in the requested message.

The configuration may include

Number of expected frequency layers (in case UE supports CA)

Number of SRS resource sets per frequency layer

For each frequency layer

  - Number of transmissions for all SRS resources configured in that frequency layer

  - Bandwidth for all SRS resources configured in that frequency layer

  - SSB information that potentially may be used for pathloss reference signal, and spatial relation info reference signal

  - For each SRS resource set

    - Recommended pathloss reference signal, which can be PRS, or SSB

    - Number of SRS resources in the SRS resource set, and for each SRS resource

      - Recommended spatial relation info reference signal, which can be PRS, SSB, or SRS

Still, it is up to gNB to decide how SRSs are configured to the UE, and return the configuration to the LMF.

Proposal 1: For the message POSITIONING INFORMATION REQUEST, support LMF to request the configuration of SRS from the serving gNB with

Number of expected UL frequencies

Number of SRS resource set per UL frequency

For each UL frequency

  - Number of transmissions for all SRS resources

  - Bandwidth for all SRS resources

  - For each SRS resource set

    - Number of SRS resources

    - Recommended pathloss reference signal

    - For each SRS resource

      - Recommended spatial relation info reference signal

POSITIONING INFORMATION RESPONSE

In response, gNB should provide the SRS configuration, which can be either SRS for positioning specified Rel-16 or SRS specified in Rel-15

All fields that specify the SRS resource/resource set are reported to the LMF, which may contain the spatialRelationInfo and pathlossReferenceRS, recommended by the LMF, except those fields related to the periodicity. The reason is that even from RAN1 and RAN2 perspective, there may be periodic, semi-persistent, and aperiodic SRS configuration between serving gNB and UE, but those SRS should not be differentiated from NRPPa signaling perspective, and all SRS to be received by the neighbouring gNB should be described by

Number of transmissions

Periodicity and offset.

- It means that

For aperiodic SRS configuration, it should be converted to a pre-scheduled transmission similar to a periodic SRS with the periodicity and offset, and with the number of transmissions, which can be at least 1.

For semi-persistent SRS configuration, it should be converted to a periodic SRS with the same periodicity and offset.

Proposal 2: For the message POSITIONING INFORMATION RESPONSE, serving gNB provides all fields pertaining only to the periodic SRS configuration (sent to the UE) to the LMF.

The selection between Rel-15 SRS and Rel-16 SRS for positioning is up to the serving gNB.

MEASUREMENT REQUEST

MEASUREMENT REQUEST is used to request neighbouring gNB/serving gNB to measure UL-RTOA/UL-AOA/gNB Rx – Tx time difference based on SRS configuration.

- For SRS measurement request, LMF should provide the SRS configuration received from the serving gNB to the neighbouring gNB/serving gNB. It is possible that LMF sends only a part of SRS configuration to a specific gNB, given the knowledge that an SRS resource is already configured with the spatialRelationInfo corresponding to the DL RS from that gNB, shown in Figure 1.

LMF

Serving gNB

Neigh. gNB1

Neigh. gNB2

SRS configuration associated with neighbouring gNB 1

SRS configuration associated with neighbouring gNB 2

UL configuration provide

Location information request

Figure 1 Sending partial SRS configuration from LMF to a gNB

- Proposal 3: The message MEASUREMENT REQUEST includes

UL measurement request to trigger NG-RAN to provide the measurement based on SRS

  - SRS configuration for the NG-RAN node to measure

  - Requested measurement, including UL-RTOA, gNB Rx – Tx time difference, AOA/ZOA, SRS-RSRP.

MEASUREMENT RESPONSE

Upon request, NG-RAN should provide the requested measurements, including

UL-RTOA, and its quality

gNB Rx – Tx time difference, and its quality

AOA/ZOA, and its equality

SRS-RSRP

- Proposal 4: The message MEASUREMENT RESPONSE includes the measurement and the quality requested by the LMF.

Discussion on DL-AoD

The DL-AoD that achieves the commercial requirements in [5] actually needs multiple RSRP measurements under a fixed Rx beam across multiple Tx beams from the same TRP, so there exists a mapping between the received RSRPs based on DL PRS resources and the estimate of the angle of departure.

In the LS from RAN3 [4], RAN3 prefers that the angle conversion is at LMF, shown as follows.

From architecture perspective; there are two options for the measurements reporting and AOD calculation: Option 1: LMF calculates the AOD based on the UE measurements and RAN node assistance information. LMF then further calculates UE location based on the AODs. Option 2: The RAN nodes calculate the AoD with the RSRP information based on gNB configuration and send the AoD information back to LMF, who then calculates the UE position. RAN3 has discussed and prefers to support option 1 for DL-AOD positioning method in the stage 2 specifications.

In our view, it has larger specification impact if we go with Option 1. The RAN node assistance information may be huge, for example, the following table either in the form of (a) or (b) should be sent in the NRPPa assistance data information, where (a) assumes that the table is sorted by angle, and (b) assumes that the table is sorted by PRS resources.

For (a), to achieve 1 degree angle resolution for AoA and ZoA, at least 180x180 angles should be listed; for each angle, at least 8 resource IDs with the highest gain on the angle should be selected with at least 7 normalized radiation patterns, each of which should be quantized to 5 bits to represent the 0-31dB with 1dB granularity, and therefore for each TRP, the mapping would consume 180x180x7x5 = 1.1M bits for the radiation pattern; additional bits including angle and PRS ID would increase the overhead. Similar consumption is expected for (b).

Observation 1: The overhead of reporting beam pattern from NG-RAN to LMF is too high.

1.1 Mbits for a TRP.

Table 1 RAN node assistance information for beam pattern

| (a) | Angle; PRS resource ID; Normalized radiation pattern, where GxyPRSx is the radiation pattern of PRS resource ID#x_y from angle#x. Angle#0; ID#0_0; Normalized to 1. ID#0_1; G00PRS0G01PRS0. ID#0_2; G00PRS0G02PRS0. ...; .... Angle#1; ID#1_0; Normalized to 1. ID#1_1; G10PRS1G11PRS1. ID#1_2; G10PRS1G12PRS1. ...; .... ...; ...; .... |
| --- | --- |
| (b) | PRS resource ID; Angle; Radiation pattern, where GxyPRS is the radiation pattern of PRS resource ID#y from angle#x. ID#0; Angle#0; G00PRS. Angle#1; G10PRS. Angle#2; G20PRS. .... ID#1; Angle#0; G01PRS. Angle#1; G11PRS. Angle#2; G21PRS. ...; .... ...; ...; .... |

However, if we go with Option 2, the specification impact is small, LMF only needs to pass the measurements received from the UE to the NG-RAN, and NG-RAN returns angle measurement associated with the angle quality. Note that Option 2 does not make the RAN into a calculation node, it is similar to angle measurement based on SRS.

To achieve this, LMF may include those measurement in the message MEASUREMENT REQUEST. Instead of requesting the NG-RAN node to acquire timing or angle measurement from the SRS, LMF simply requests the NG-RAN node to acquire the angle measurement based on the provided RSRP measurement.

In response, NG-RAN returns the angle measurement to LMF via MEASUREMENT RESPONSE, including the quality, similar to measurement based on SRS.

Observation 2: Option 2 (NG-RAN translates the RSRP measurement to angle measurement) fits well in the message MEASUREMENT REQUEST/RESPONSE that used for UL measurement transfer.

- Proposal 5: The message MEASUREMENT REQUEST also includes

DL-AoD angle resolution request to trigger NG-RAN to provide AOA/ZOA based on RSRP measurements

  - {TRP ID, Cell ID} to identity the TRP

  - PRS resource set ID for that TRP

  - A list of PRS resource ID

  - A list of differential RSRPs, each of which is associated with the PRS resource ID in the reported resource set.

- Proposal 6: The measurement of AOA/ZOA, and their quality contained in the message MEASUREMENT RESPONSE for UL-AoA positioning can be used to transfer the angle measurement for DL-AoD positioning.

Conclusion

In this contribution, we discuss the issues related to angle resolution in DL-AoD and beam configuration aspects. Based on the discussion, we have the following observations and proposals:

Observation 1: The overhead of reporting beam pattern from NG-RAN to LMF is too high.

- 1.1 Mbits for a TRP.

Observation 2: Option 2 (NG-RAN translates the RSRP measurement to angle measurement) fits well in the message MEASUREMENT REQUEST/RESPONSE that used for UL measurement transfer.

Proposal 1: For the message POSITIONING INFORMATION REQUEST, support LMF to request the configuration of SRS from the serving gNB with

Number of expected UL frequencies

Number of SRS resource set per UL frequency

For each UL frequency

  - Number of transmissions for all SRS resources

  - Bandwidth for all SRS resources

  - For each SRS resource set

    - Number of SRS resources

    - Recommended pathloss reference signal

    - For each SRS resource

      - Recommended spatial relation info reference signal

Proposal 2: For the message POSITIONING INFORMATION RESPONSE, serving gNB provides all fields pertaining only to the periodic SRS configuration (sent to the UE) to the LMF.

The selection between Rel-15 SRS and Rel-16 SRS for positioning is up to the serving gNB.

- Proposal 3: The message MEASUREMENT REQUEST includes

UL measurement request to trigger NG-RAN to provide the measurement based on SRS

  - SRS configuration for the NG-RAN node to measure

  - Requested measurement, including UL-RTOA, gNB Rx – Tx time difference, AOA/ZOA, SRS-RSRP.

- Proposal 4: The message MEASUREMENT RESPONSE includes the measurement and the quality requested by the LMF.

- Proposal 5: The message MEASUREMENT REQUEST also includes

DL-AoD angle resolution request to trigger NG-RAN to provide AOA/ZOA based on RSRP measurements

  - {TRP ID, Cell ID} to identity the TRP

  - PRS resource set ID for that TRP

  - A list of PRS resource ID

  - A list of differential RSRP, each associated with the PRS resource ID in the reported resource set.

- Proposal 6: The measurement of AOA/ZOA, and their quality contained in the message MEASUREMENT RESPONSE for UL-AoA positioning can be used to transfer the angle measurement for DL-AoD positioning.

References

- 3GPP TS 36.455, LTE Positioning Protocol A (LPPa)

- 3GPP RAN3_106_agenda_with_Tdocs20191122_EOM, RAN3, 3GPP RAN3#106, Reno, US, Nov. 18 – 22, 2019.

- 3GPP R3-197688, “Support for UL Positioning Measurements in NG-RAN”, Ericsson LM, 3GPP RAN3#106, Reno, US, Nov. 18 – 22, 2019.

- 3GPP R3-197794, “LS on DL-AOD procedure”, RAN3, 3GPP RAN3#106, Reno, US, Nov. 18 – 22, 2019.

- 3GPP TR 38.855, Study on NR positioning support.