3GPP TSG RAN WG1 Meeting #100-e	R1-2000213

February 24 – March 6, 2020

Agenda Item:	6.2.4.1

Source:	Huawei, HiSilicon

Title:	Corrections to 36.211 and 36.213 for the 0.37kHz subcarrier spacing MBSFN

Document for:	Discussion and Decision

Introduction

In RAN#83 meeting, a new work item, i.e., LTE-based 5G terrestrial broadcast was approved [1]. One of its objectives is to specify a new numerology for PMCH to support rooftop reception in MPMT and HPHT-1 scenarios as described in TR 36.776. In RAN1#98 meeting, the new numerology (i.e. SCS=0.37 kHz) was agreed [2]:

Agreements

The numerology of Tcp=300us/Tu=2.7ms should be used for PMCH to support rooftop reception.

In this contribution, we discuss the corrections due to 0.37 kHz subcarrier spacing, including the MBSFN RS resource mapping, the PMCH scrambling and TBS index range. In addition, the corresponding Text Proposals to TS 36.211 and 36.213 are provided.

Discussion

RS pattern mapping issue

For PMCH transmissions using 0.37 kHz subcarrier spacing, a slot has a length of 3ms. There are 13 slots, numbered in increasing order from 0 to 12 within a 40ms period starting at nf mod 4=0 with slot 0 starting at 30720Ts in the 40 ms period. Two MBSFN reference signal pattern types 1 and 2 for 0.37 kHz SCS are defined with Dt =4 and Dt =2, respectively. In order to obtain the good performance by using the defined RS patterns, it was proposed to keep the RS pattern continuity in time domain as shown in Fig.1.

Figure 1: illustration of keeping MBSFN RS pattern continuity for 0.37 kHz SCS

When considering the MBSFN RS mapping to resource elements for 0.37 kHz SCS, the 3ms slot can continue being numbered instead of being restricted within 40ms period. The corresponding text proposal to TS 36.211 for RS mapping is provided in Appendix I.

PMCH scrambling issue

In addition, for PMCH channel mapping, clause 6.3 is referred to with some exceptions. However, one exception to clause 6.3.1 is missing where the scrambling sequence generator shall be initialized at the start of each slot for numerologies 1.25 kHz and 0.37 kHz instead of at the start of each subframe. The text proposal is provided in Appendix I.

TBS determination issue

In TS 36.213 V16.0.0 section 11.1, when determining the TBS of PMCH with , following the current specification, if TBS index ITBS is configured as 33B of Table 7.1.7.2.1-1, the TBS before scaling would be 100752 for each NPRB  configuration among 100~110. After scaling by 3, the size would be 302256 which cannot be found in Table 7.1.7.2.1-1 nor Table 7.1.7.2.4-1, so the determination needs correction.

Nevertheless, the TBS index 33B cannot be configured for MBMS actually since 33B is included only in the dedicated configuration for PDSCH nor 33A. Therefore, the correction is to limit the TBS index to be no larger than 33. The corresponding text proposal is provided in Appendix II.

Proposal : Agree the text proposals for TS 36.211 and 36.213 as Appendices I and II, respectively.

- References

RP-190732, “LTE-based 5G terrestrial broadcast,” Qualcomm Incorporated, RAN#83, Shenzhen, China, March 18-21, 2019.

R1-1911429, “Report of 3GPP TSG RAN WG1 #98,” Prague, Czech Rep, August 26-28, 2019.

- Appendix I: Text proposal for TS 36.211

- < Unchanged parts are omitted >

- 6.5 		Physical multicast channel

The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions:

-	No transmit diversity scheme is specified.

-	Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4.

-	The PMCH can only be transmitted in the MBSFN region. The index  in the first slot in the MBSFN subframe fulfils  where  is equal to the value given by the higher layer parameter non-MBSFNregionLength [9].

-	The PMCH shall use extended cyclic prefix.

-	The PMCH is not mapped to resource elements used for transmission of MBSFN reference signals.

-	In clause 6.3.1, for Δf = 1.25 kHz and Δf ≈ 0.37 kHz, the scrambling generator shall be initialised at the start of each slot.

-	For Δf=182944Ts≈0.37 kHz:

-	The text “which meet all of the following criteria in the current subframe” shall be replaced by “which meet all of the following criteria in the current slot”

-	The mapping to resource elements k,l on antenna port p not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l.

- < Unchanged parts are omitted >

6.10.2.2.4		Mapping to resource elements for 0.37 kHz subcarrier spacing

The reference-signal sequence rlm' in OFDM symbol l shall be mapped to complex-valued modulation symbols ak,l(p) with p=4 according to

ak,l(p)=rl,nsm'

when

0≤k<NscRB12NRBDL

and where ns is the 3 ms slot number given by ns=nf4×13+ns', where nfis the radio frame number and ns' is the 3 ms slot number as defined in clause 4.1.

-	for MBSFN reference signal pattern type 1

k=12m'-NscRB12Δ+3ns mod 4if ns mod 4∈0,112m'-NscRB12Δ+3ns mod 4if ns mod 4∈2,3
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB12NRBmax,DL-1

-	for MBSFN reference signal pattern type 2

k=6m'-NscRB6Δ+3ns mod 2
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB6NRBmax,DL-1

- < Unchanged parts are omitted >

- Appendix II: Text proposal for TS 36.213

- 11.1 	UE procedure for receiving the PMCH

- < Unchanged parts are omitted >

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to. For PMCH with  the UE shall scale the derived transport block size by , then round half-way towards positive infinity to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33B and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33B if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤26A and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤26A otherwise. The UE shall set the redundancy version to 0 for the PMCH.

- < Unchanged parts are omitted >