3GPP TSG RAN WG1 Meeting #100-e	R1-2000220

February 24 - March 6, 2020

Agenda Item:	6.2.1.3

Source:	Huawei, HiSilicon

Title:	Corrections on scheduling of multiple transport blocks

Document for:	Discussion and Decision

# Introduction

In RAN#86, CRs for scheduling of multiple transport blocks were approved [1]. However, there are still essential issues overlooked and some agreements not correctly captured. Discussions and text proposals are given in this contribution to make the specification more clear and complete.

- Discussion

  - Interleaving granularity

In RAN1 #99 meeting [2], the following was agreed for the interleaving granularity.

RAN1 #99 Agreement For unicast interleaved transmission without frequency hopping, the interleaving granularity is 1 repetition (RU) of a TB in CE mode A and 4 repetitions (RUs) of a TB in CE mode B in FDD and TDD • No changes to the legacy RV rules

Based on the agreement, different TBs are interleaved based on the actual repetitions not the absolute subframes. However, according to current specification, RV changes every Nacc absolute subframes, where Nacc=5 for PUSCH and Nacc=10 for PDSCH for TDD in CEMode B. Then in some cases, within one interleaving granularity (4 repetitions) different RVs will be applied to the TB, which will reduce the SNR gain by combining different RVs of one TB and will largely degrade the system performance. An example is shown in figure 1, where the uplink-downlink configuration 6 is assumed.

Figure 1. RV in an interleaving granularity

As illustrated in Figure 1, for PUSCH, the RV changes every 5 absolute subframes and the TB is changed every 4 repetitions. Then we can observe that the RV is 0 for the first three repetitions of TB0, and 2 for the last repetition within an interleaving granularity. As a result, for TB0 the repetitions within one granularity cannot be signal-combined together, which will largely degrade the SNR. Therefore, to resolve the above issue, the interleaving granularity should be based on the absolute subframes and be multiple of Nacc.

In addition, the TB interleaving may not be aligned with frequency hopping either. As a result, there are can be different TBs in the same PRB across subframes, and the signal combination cannot be used for a certain TB.

Observation 1: The system performance will be largely reduced if applying the current interleaving granularity due to the misalignment between TB interleaving and RV, and the misalignment between TB interleaving and frequency hopping.

In order to combine different repetitions of one TB in signal level to improve the SNR, it is beneficial for the consecutive repetitions of one TB to be in the same narrowband. For frequency hopping, Ych is configured from values below:

- CEModeA : {1,2,4,8} for FDD, {1,5,10,20} for TDD

- CEModeB : {2,4,8,16} for FDD, {5,10,20,40} for TDD

It can be observed that the value of Ych can be configured as multiple of the values of Nacc, so taking the RV and frequency hopping into consideration the value of interleaving granularity can be Ych.

Proposal 1: For unicast interleaved transmission, the interleaving granularity is Ych consecutive absolute subframes in CE mode B in FDD and TDD, where Ych is the frequency hopping granularity.

Proposal 2: Endorse the text proposal in Appendix I to 36.213.

## Frequency hopping

As illustrated in Figure 2, in some cases TB1 is only transmitted on narrowband2 and TB2 is only transmitted on narrowband1, and the benefits of frequency hopping cannot be achieved. In order to deal with this issue, frequency hopping locations can be shifted based on an offset, so that each TB can be transmitted with frequency hopping practically. As shown in Figure 3, the frequency location is shifted in every round of transmission. Based on the principle, the frequency hopping equation can be modified as below:

where y can be equal to the number of scheduled TBs. We can observe that when frequency hopping is done based on the above equation, the frequency location of the TBs in the Nth round will be shifted N*fNB,hop narrowbands.

Figure 2. Frequency hopping pattern for interleaving case

Figure 3. Frequency hopping location shift

Proposal 3: When interleaving is enabled for multiple TBs scheduling, the frequency hopping pattern is determined based on the following equation:

## Maximum number of TBs scheduled by one DCI

In RAN1#98 meeting [3], we had the following agreement on whether the maximum number of scheduled TBs with one single DCI was configured or not.

RAN1 #98 Agreement - For unicast, select option(s) from the following options - Option 1: Scheduling of up to 8 TBs is supported with a single DCI design. - Target for up to 6 bits overhead increase compared to legacy DCI - Option 2: For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {1, 2, [4], 8} in a UE specific manner. - The design methodology for the DCI for different maximum number of TBs is further studied - For the 2 TB case, target for up to 3 bits overhead increase compared to legacy DCI - Note: Option 2 will require modification on existing agreement - The following working assumption is confirmed. - For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported - For unicast, the new data indication is individually provided for each allocated HARQ process.

However, up to now we have not made any decision on this issue and the DCI design is only targeted for the 8 TBs case. In addition, [4] had a comprehensive analysis on the performance gain with different number of TBs scheduled by one DCI, and it was observed that with only 2 TBs being allocated via one DCI, a significant gain is achieved. Meanwhile, it is also noted that some essential user plane traffic is not very large and 2 TBs being scheduled by one DCI is enough, e.g. voice traffic (VoLTE). When the traffic is such that scheduling of more than two TBs with one DCI is rare, the DCI size can be further reduced, which can provide a better MPDCCH coverage.

Based on that a significant gain with respect to PUSCH/PDSCH coverage can be obtained, we can limit the maximum number of allocated TBs and it is proposed that the maximum number of TBs allocated with one DCI is configured in the set {1, 2, 4, 8}.

Proposal 4: For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {1, 2, 4, 8} in a UE specific manner.

  - HARQ ACK bundling

Based on the agreement achieved in RAN1 #98b meeting [2], HARQ-ACK bundling feature was introduced and Rel-14 HARQ-ACK bundling feature was reused.

Agreement - For UEs that support multi-TB scheduling with HARQ-ACK bundling, the maximum bundle size is 4. - Strive to reuse Rel-14 HARQ-ACK bundling feature as baseline at least for the non-interleaving case

Therefore, the HARQ-ACK bundling for multiple TB scheduling should also only be used for HD-FDD without repetition case. However, there is no limitation on the HARQ-ACK bundling in the current specification. In order to align with the agreement, we propose to add the used scenario for HARQ-ACK bundling.

Proposal 5: HARQ-ACK bundling for multi-TB scheduling is only supported for HD-FDD without repetition.

Proposal 6: Endorse the text proposal in Appendix II to 36.213.

  - Joint encoding of the DCI

In RAN1 #99 meeting [3], the following agreements were achieved for CEMode A and CEMode B:

RAN1 #99 Agreement (CE mode A) Bits; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. 1 HARQ; 0; 0; 0; 0; 0; HARQ ID; NDI; RV1; FH1. 2 HARQS; 0; 0; HARQ2 Index +8; N1; N2; RV2-1; RV/FH2-2. 4 HARQS; 0; HARQ4 index + 36; N1; N2; N3; N4. 6 HARQS; HARQ6 Index +27; N1; N2; N3; N4; N5; N6. 8 HARQS; 1; 1; 1; N1; N2; N3; N4; N5; N6; N7; N8; RV/FH8. Where Hi is the scheduled HARQ ID in the set {0-7} HARQ2 Index = nChoosek(8- H1,2)-H2+H1 HARQ4 Index = nChoosek (7- H1,4)+nChoosek (7- H2,3)+nChoosek(8- H3,2)- H4+H3 HARQ6 Index = nChoosek(8- H1,2)-H2+H1 H1 and H2 are the HARQ ID’s not scheduled. RV1 is RV for H1 in the set {0,1,2,3} FH1: Same behaviour as legacy frequency hopping bit. Set to 0 when frequency hopping and 64QAM are disabled If Repetition=1 and frequency hopping is enabled and 64QAM is not enabled then RV2-1 indicates the RV for H1 in the set {0,2} RV/FH2-2 indicates the RV for H2 in the set {0,2} Else if Repetition=1 and 64QAM is enabled then RV2-1 indicates the RV for H1 and H2 in the set {0,2} RV/FH2-2 indicates the extension to MCS table for 64QAM and RV for H2 is 0 Else if Repetition>1 and frequency hopping is enabled then RV2-1 indicates the RV for H1 and H2 in the set {0,2} RV/FH2-2 indicates if H1 and H2 are frequency hopped and RV for H2 starts at 0 Else RV2-1 indicates the RV for H1 in the set {0,2} RV/FH2-2 indicates the RV for H2 in the set {0,2} End RV/FH8: If Repetition=1 and frequency hopping is enabled and 64QAM is not enabled then RV/FH8 indicates the RV for H1 to H8 in the set {0,2} Else if Repetition=1 and 64QAM is enabled then RV/FH8 indicates the extension to MCS table for 64QAM and RV for H1 to H8 is 0 Else if Repetition>1 and frequency hopping is enabled then RV/FH8 indicates if H1 to H8 are frequency hopped and RV for H1 to H8 starts at 0 Else RV/FH8 indicates the RV for H1 to H8 in the set {0,2} End Note RV in above means the starting RV Agreement (CE mode B) For CE mode B MTBG DCI design, for DL grant and full PRB UL grant and for Sub-PRB UL grant, the following bit map to indicate HARQ ID, # of HARQ, NDI, MCS: Bits; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. 1 HARQ; 0; 0; 0; MCS; HARQ1; N1. 2 HARQS; 0; MCS+3; HARQ2 Index; N1; N2. 3 HARQS; MCS+15; HARQ3; N2; N3; N4. 4 HARQS; MCS +52; N1; N2; N3; N4. where: - HARQ1 = HARQ ID scheduled - HARQ2 index = nChoosek(4- H1,2)-H2+H1 - HARQ3 = HARQ ID not scheduled MCS is 11 legacy values for DL grant and 10 legacy values UL grant for full PRB and 8 legacy valued for UL grant for sub-PRB

For CEMode A, RV, FH, NDI and HARQ processes are jointly encoded, while for CEMode B the MCS, NDI and HARQ processes are jointly encoded. In current 36.212, the jointly encoded field is named as “Scheduling TBs for Unicast” and leave the definition to the 36.213. However, in the current 36.213, there is no definition for it.

In our opinion, the FH and RV determinations should be captured in Section 7.1.7.1, and the scheduled HARQ index should be captured in Section 7.1.7.2. In addition, in Section 7.1.7.2 of 36.213 the following description was used to determine the HARQ index.

For a BL/CE UE, if the UE is configured with higher layer parameter multi-TB-DL-config and multiple TB, , are scheduled in the corresponding DCI with CRC scrambled by C-RNTI, the HARQ process ID for each of the scheduled TBs are determined from the value of the HARQ index field in the correspoding DCI which is a combinatorial index r defined as , where - the set , () contains the sorted HARQ process IDs and is the extended binomial coefficient, resulting in unique label , - is the number of scheduled TB, and - if UE is configured with CEModeA, and if UE is configured with CEModeB, - if UE is configured with CEModeA, and ‘Multi-TB HARQ processes group’ field is present and set to '1' in the corresponding DCI, otherwise.

However, there is no HARQ index field in the current DCI and it should be modified according to the agreement.

Proposal 7: The determination of {RV, FH, NDI and HARQ process ID} for CEMode A and the determination of {MCS, NDI and HARQ process ID} for CEMode B need to be captured according to the agreement.

## Interleaving for unicast

In RAN1#99, the following agreements were made:

RAN1 #99 Agreement For unicast interleaved transmission, for the PUSCH sub-PRB 2-subcarrier case, the symbol counter is reset at the start of the first TB transmission and incremented for each symbol during the whole transmission of the TBs where Y is the number of scheduled TBs.

For the following description to capture the agreement in current specification:

If NTB >1 and interleaving between codewords is applied according to clause 8.0 of 3GPP TS 36.213 [4], then the symbol counter  is reset at the start of the transmission and incremented for each symbol during the transmission.

“The transmission” would introduce ambiguity as it may be interpreted as the transmission of each TB or the transmission of NTB TBs. Then it may lead to two different implementations for symbol counter reset.  One implementation is that the symbol counter  is reset at the start of the transmission of each TB, and the other implementation is that the symbol counter  is reset at the start of the transmission of NTB TBs. Based on the agreement, the symbol counter  should be reset at the start of the transmission of NTB TBs. Thus “the transmission” is proposed to be clarified as “the transmission of NTB PUSCH codewords”.

Proposal 8: Endorse the text proposal in Appendix III to TS 36.211.

# Conclusions

In this contribution, we discussed the issues that are not correctly captured in the current specifications and we have the following proposals:

Observation 1: The system performance will be largely reduced if applying the current interleaving granularity due to the misalignment between TB interleaving and RV, and the misalignment between TB interleaving and frequency hopping.

Proposal 1: For unicast interleaved transmission, the interleaving granularity is Ych consecutive absolute subframes in CE mode B in FDD and TDD, where Ych is the frequency hopping granularity.

Proposal 2: Endorse the text proposal in Appendix I to 36.213.

Proposal 3: When interleaving is enabled for multiple TBs scheduling, the frequency hopping pattern is determined based on the following equation:

Proposal 4: For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {1, 2, 4, 8} in a UE specific manner.

Proposal 5: HARQ-ACK bundling for multi-TB scheduling is only supported for HD-FDD without repetition.

Proposal 6: Endorse the text proposal in Appendix II to 36.213.

Proposal 7: The determination of {RV, FH, NDI and HARQ process ID} for CEMode A and the determination of {MCS, NDI and HARQ process ID} for CEMode B need to be captured according to the agreement.

Proposal 8: Endorse the text proposal in Appendix III to TS 36.211.

# References

- 3GPP, RP-192647 Introduction of Additional MTC Enhancements, Sitges, SPAIN, 9th Dec 2019 - 12th Dec 2019.

- 3GPP, “Chairman's Notes RAN1 #98b final”, Chongqing, China, October 14th – 20th, 2019

- 3GPP, “Chairman's Notes RAN1 #99 final”, Reno, USA, November 18th – 22th, 2019

# Appendix I TP for TS 36.213

<Unchanged parts are omitted>

7.1.11	PDSCH subframe assignment for BL/CE UE

<Unchanged parts are omitted>

-	if the UE is configured with higher layer parameter multi-TB-DL-Unicast-Interleaving-config, and PDSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and  where  for BL/CE UE configured with CEModeA, C= NNBch, DL for BL/CE UE configured with CEModeB, where  NNBch, DL is configured by interval-DlHoppingConfigCommonModeB,

<Unchanged parts are omitted>

8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

-	if the UE is configured with higher layer parameter multi-TB-UL-Unicast-Interleaving-config, and PUSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and  where  for BL/CE UE configured with CEModeA, C=NNBch, UL for BL/CE UE configured with CEModeB,  where  NNBch, UL is configured by interval-UlHoppingConfigCommonModeB,

# Appendix II TP for TS 36.213

<Unchanged parts are omitted>

7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is in half-duplex FDD operation and is configured with CEModeA without repetition, and if the UE is configured with higher layer parameter multi-TB-DL-HARQ-bundling and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,

<Unchanged parts are omitted>

# Appendix III TP for TS 36.211

<Unchanged parts are omitted>

5.6A.2	Modulation scheme π/2-BPSK

<Unchanged parts are omitted>

For MscRU=3 and π/2-BPSK modulation only 2-of-3 adjacent subcarriers are selected as described in 5.5.2.1A.2. The time-continuous signal  in SC-FDMA symbol  in an uplink slot is defined by

for  where , ,  is given by Table 5.6-1, and ak,l and  are respectively the modulation value for subcarrier index  and k+1 for symbol , and the values of  used on  and  are respectively obtained by subtracting  from the resulting set of allocated subcarriers as described in Table 8.1.6-1 of [4], and  represents the lower subcarrier index among the selected subcarriers and  is the subcarrier index adjacent to it. The phase rotation  is given by

ϕ=π2l mod 2+φavgkl

φavgkl=φavgkl-1+2πΔfk+1N+NCP,lTs  when  l>0

φavgk0=0

l=0,1,…,NTBNrepPUSCHMRUMslotsULMsymbUL-1

l= l mod MsymbUL

where NTB is the number of transport blocks defined in clause 8.0 of 3GPP TS 36.213 [4]. If NTB >1 and interleaving between codewords is applied according to clause 8.0 of 3GPP TS 36.213 [4], then the symbol counter  is reset at the start of the transmission of NTB PUSCH codewords and incremented for each symbol during the transmission. For other cases, the symbol counter  is reset at the start of each PUSCH codeword transmission and incremented for each symbol during the transmission of the PUSCH codeword.

<Unchanged parts are omitted>