3GPP TSG RAN WG1 Meeting #100-e	R1-2000225

February 24 – March 6, 2020

Agenda Item:	6.2.3.1.1

Source:	Huawei, HiSilicon

Title:	Corrections on additional SRS symbols

Document for:	Discussion and Decision

Introduction

In RAN #86, CRs for Rel-16 LTE DL MIMO were approved [1]. In this contribution, discussions and text proposals are given to correct the specification aligning with the agreements.

Discussion

Rate matching manner for PDSCH transmission

In RAN1#99, the following Agreement #1 and Agreement #2 were achieved.

RAN1 #99 Agreement #1 The number of antenna switching is: • 2 for 1T2R is configured or the number of pairs is configured as 2 for 2T4R • 3 if the number of pairs is configured as 3 for 2T4R • 4 for 1T4R Agreement #2 The number of frequency hops, N_FH (integer value), is determined according to the following formula - N=R*NAS*NFH+(NAS*GSAS-1)*GSAS+(NFH*GSFH-1)*GSFH R is the repetition, N_AS is the number of antenna switchings, N_FH is the number of frequency hops, GS_AS is the guard symbol configuration for antenna switching which is 0 or 1, GS_FH is the guard symbol configuration for frequency hopping, N is the duration in terms of OFDM symbols for the additional SRS symbols - It is up to network to make sure that N_FH is an integer - Note: It is up to the editor how to capture this in the specifications

Based on the Agreement #1, the number of antenna switching is defined as 2/3/4 based on the different configurations or UE transmit/receive antennas.  Agreement #2 also applies the restriction on the number of antenna switchings N_AS, where N_AS is guaranteed by the equation

N=R*NAS*NFH+(NAS*GSAS-1)*GSAS+(NFH*GSFH-1)*GSFH

Therefore, the antenna port derived will always be within the number of antenna switching and there is no need to perform mod operation when calculating the index a(n_SRS ) of the UE antenna in TS 36.213 and should be deleted.

Observation 1: There is no need to perform mod operation when calculating the index a(n_SRS ) of the UE antenna in TS 36.213.

However, if the frequency hopping is not configured, the number of antenna switching may not be the number that is agreed as below:

Agreement #1

The number of antenna switching is:

•	2 for 1T2R is configured or the number of pairs is configured as 2 for 2T4R

•	3 if the number of pairs is configured as 3 for 2T4R

•	4 for 1T4R

For example, for a UE with 1T2R, if 4 symbols are configured for additional SRS without frequency hopping and repetition, then the antenna port would be AP#0, AP#1, AP#0, AP#1, which have more antenna switching than that has been agreed.

Observation 2: The number of antenna switching for additional SRS symbols without frequency hopping may be larger than the number that has been agreed.

Proposal 1: Endorse the text proposal in Appendix I to 36.213.

Conclusions

In this contribution, the issue about calculating the index a(n_SRS ) of the UE antenna in TS 36.213 is discussed and the following observations and proposals are provided.

Observation 1: There is no need to perform mod operation when calculating the index a(n_SRS ) of the UE antenna in TS 36.213.

Observation 2: The number of antenna switching for additional SRS symbols without frequency hopping may be larger than the number that has been agreed.

Proposal 1: Endorse the text proposal in Appendix I to 36.213.

- References

3GPP, RP-192649 Introduction of DL MIMO efficiency enhancements for LTE, Sitges, SPAIN, 9th Dec 2019 - 12th Dec 2019.

- Appendix I

- Text proposal for 36.213

<Unchanged parts are omitted>

For trigger type 2 SRS transmission, when closed-loop or open-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports antenna selection, or for UE that can be configured with ue-TxAntennaSelection-SRS-1T4R-Config or ue-TxAntennaSelection-SRS-2T4R-NrOfPairs

-	when higher layer parameter ‘ue-TXAntennaSelection-SRS-1T4R-Config’ is configured for a given serving cell, the number of antenna switches NAS = 4, and the index anSRS of the UE antenna that transmits the SRS at time nSRS is given by anSRS=nSRSnSRS mod 4 when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod 4 when frequency hopping is enabled. UE is not expected to be configured with anSRS>4.

-	when higher layer parameter 'ue-TXAntennaSelection-SRS-2T4R-Config’ is configured for a given serving cell for a UE configured with Λ UE antenna pairs, where Λ={2 or 3} is given by higher layer parameter ‘ue-TXAntennaSelection-SRS-2T4R-Config’, the number of antenna switches NAS = 3, and the index for the UE antenna pair is 2anSRS,  2anSRS+1 when Λ =2, or 0,  anSRS+1 when Λ =3 that transmits the SRS at time nSRS, where anSRS is given by anSRS=nSRSnSRS mod Λ when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod Λ when frequency hopping is enabled. UE is not expected to be configured with anSRS>Λ.

-	otherwise, the number of antenna switches NAS = 2, and the index  of the UE antenna that transmits the SRS at time nSRS is given by anSRS=nSRSnSRS mod 2 when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod 2 when frequency hopping is enabled. UE is not expected to be configured with anSRS>2.

where NFH is the number of frequency hops defined in Subclause 5.5.3.2.2 of [3].

<Unchanged parts are omitted>