3GPP TSG RAN WG1 #100		R1-2000330

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Enhanced UL configured grant transmissions for URLLC

Agenda Item:	7.2.5.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues on enhanced UL configured grant for URLLC, including configuration for DCI format 0_2 with CS-RNTI scheduling a retransmission for UL configured grant.

- Discussion

  - Configuration for DCI format 0_2 with CS-RNTI scheduling a retransmission for UL configured grant

In RAN1#99 meeting, following working assumption was made for the configuration for DCI format 0_2 with CS-RNTI scheduling retransmission PUSCH for UL configured grant [1].

- Retransmission of the PUSCH scheduled by a new UL DCI format with CRC scrambled by CS-RNTI with NDI=1 shall follow the same higher layer configuration defined for dynamic PUSCH transmission associated with the new UL DCI format except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex, [mcs-Table], [mcs-TableTransformPrecoder] and [transformPrecoder].

The remaining issue is to determine the exceptions of parameters associated with DCI format 0_2 for scheduling retransmission of UL configured grant. In Rel.15, the MCS table for a PUSCH scheduled by a PDCCH with CRC scrambled by CS-RNTI follows the configuration by configuredGrantConfig by RRC. It is simple that DCI format 0_2 with CS-RNTI follows the same rule defined in Rel.15.

Regarding the waveform for retransmission of UL configured grant, following rules are defined in Rel.15. For DCI format 0_2 scheduling retransmission of UL configured grant, the same rule can be reused, considering a PUSCH scheduled by PDCCH with CS-RNTI is similar to a dynamic grant PUSCH.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

Proposal 1:  Confirm following working assumption with modification.

- Retransmission of the PUSCH scheduled by a new UL DCI format with CRC scrambled by CS-RNTI with NDI=1 shall follow the same higher layer configuration defined for dynamic PUSCH transmission associated with the new UL DCI format except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex, mcs-Table, mcs-TableTransformPrecoder and [transformPrecoder].

In RAN1 #AH 1901 meeting, following agreement was made for UL configured grant.

Agreement: - In Rel-16, for both Type 1 and Type 2 configured grant and when multiple active configurations are configured in a BWP, transmission of a TB based on the configured grant is associated with a single active configuration, even if the transmission is repeated

In current 38.214 [xx], it seems the above agreement is not clearly captured in section 6.1.2.3. Therefore, we suggest to capture the following text in 38.214 to reflect the above agreement.

Proposal 2:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant …omitted part… For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided more than one configuration for UL grant Type 1 or Type 2 PUSCH, for both Type 1 and Type 2 PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. …omitted part…

- Conclusion

In this contribution, we provide our views on remaining issues on UL configured grant. The observations and proposals are summarized below.

Proposal 1:  Confirm following working assumption with modification.

- Retransmission of the PUSCH scheduled by a new UL DCI format with CRC scrambled by CS-RNTI with NDI=1 shall follow the same higher layer configuration defined for dynamic PUSCH transmission associated with the new UL DCI format except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex, mcs-Table, mcs-TableTransformPrecoder and [transformPrecoder].

Proposal 2:  adopt the following TP in 38.214.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant …omitted part… For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberofrepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. If a UE is provided more than one configuration for UL grant Type 1 or Type 2 PUSCH, for both Type 1 and Type 2 PUSCH transmissions with a configured grant configuration, when K ≥ 1, the UE shall transmit the TB across the transmission occasion candidates within the same configured grant configuration. …omitted part…

- References

- Chairman’s Notes RAN1 #99.