3GPP TSG RAN WG1 #100	R1-2000332

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Discussion on remaining issues on MUCSI

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

Introductions

In this contribution, we discuss some of the remaining open issues on Rel-16 MUCSI enhancement.

UE feature

In the last RAN1 meeting, we have the following agreement related to UE capability:

Agreement

On the # of PMI subbands:

- Mandatory support for R=1

- Optional support for N3>19

FFS: whether it is mandatory or optional for R=2 and N3<=19

- To be discussed during the Rel-16 UE capability discussions

- In the above agreement there is one FFS point yet to be resolved. First of all, since the PMI computation complexity is dependent on whether R is supported or not, R = 2 requires a higher UE capability than not supporting R. Secondly, some gains are observed with R = 2 only for the case of large number CQI SBs as well as large bandwidth, while the benefit of R = 2 is not clear for small CQI subband number and small bandwidth. Thirdly, from UE’s perspective, we should be careful about those new features for keeping a successful UE implementation in real life. Therefore, it’s better to support optional UE capabilitiy for R = 2 and N3<=19 with a separate indication for explicitly enabling this complex calculation.

Proposal1: R = 2 and N3<=19 is an optional capability with a separate indication.

- Following agreement was achieved on maximum number of configured aperiodic CSI report settings:

Agreement

In Rel-16, for CSI report other than beam report:

- Increase the maximum number of configured aperiodic CSI Report Settings a UE can report per BWP (RRC parameter maxNumberAperiodicCSI-PerBWP-ForCSI-Report) up to 8, subject to UE capability

  - The maximum number of configured aperiodic CSI Report Settings a UE supports is part of UE capability

- Note: This increases the value range of the associated UE capability signaling from 1...4 to 1...8

- Note: FFS for FR2 whether UE can report a value larger than 4

- Whether UE can report a value larger than 4 for FR2 is FFS. The motivation for increasing the value range for FR2 is not very clear. In FR2, where highly directional antenna with analogue beamforming is deployed, most likely the number for CSI-RS ports will be small such as 2, then there is no motivation for supporting type II CSI. In this case, there is no need to increase supported value larger than 4 for aperiodic CSI Reporting settings.

- Proposal2: for FR2, the maximum number of configured aperiodic CSI reporting settings a UE can report is same as Rel-15

Conclusion

In this contribution we discussed two different aspects of Rel-16 MU CSI enhancement. Based on our analysis, we have following proposals:

Proposal1: R = 2 and N3<=19 is an optional capability with a separate indication.

- Proposal2: for FR2, the maximum number of configured aperiodic CSI reporting settings a UE can report is same as Rel-15

References

- RAN1 #99 Chairman’s Notes. Reno, USA, November 18th – 22nd, 2019