3GPP TSG RAN WG1 #100		R1-2000346

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Remaining issues on uplink power control for NR-NR DC

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #99 meeting, following agreements were achieved on semi-static and dynamic power sharing for NN-DC.

Agreements:

- Alt.2 of semi-static power sharing can be configured for both synchronous and asynchronous NR-DC scenarios.

Agreements:

- Alt.1-2 of semi-static power sharing can be configured for synchronous DC scenario only.

  - It is up to UE to determine whether the overlapping with UL transmission on the SCG is possible, if/when factors other than the TDD UL-DL configurations of the serving cells in the SCG (e.g., timing difference, drift) need to be taken into account.

Agreements:

- For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0,

- UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and

  - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG

  - Otherwise, pwr_SCG <= Ptotal;

- UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission.

  - (working assumption) No new RRC signaling is introduced for T_offset:

    - Alt.1: T_offset <= T_proc,2

    - Alt.2: T_offset <= 2*T_proc,2

    - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms

    - To be addressed in the CR stage

In this contribution, we present our views on remaining issues on NN-DC power control based on [1].

- Discussion

  - Semi-static power sharing

For semi-static power sharing, it was agreed Alt.1-2 can be configured for synchronous DC only. Regarding TDD UL-DL configurations for synchronous DC, at least tdd-UL-DL-ConfigurationCommon should be configured, whether tdd-UL-DL-ConfigurationDedicated is configured or not is optional. Therefore, both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated may not always be configured for a UE.

Proposal 1:

- Support the following text proposal to clarify the TDD UL/DL configuration for semi-static power sharing.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Subclauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Subclauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

…

--------------------------------------------------------End text proposal----------------------------------------------------------

  - Dynamic power sharing with power look-ahead

For dynamic power sharing with power look-ahead, it was agreed that the UE prioritizes MCG transmission before T0 - Toffset, while the T0 is the UL transmission starting time of SCG. From our point of view, the Tproc,2 specified in Rel-15 for UCI multiplexing can be reused as the maximum value of Toffset. In the case of different Tproc,2 values are generated from MCG and SCG, e.g., due to different SCSs deployed in different CGs, the maximum Tproc,2 value between them can be applied to simplify the specification effort and UE implementation.

Proposal 2:

- For NN-DC dynamic power sharing with power ‘look-ahead’, no new RRC signaling is introduced for T_offset.

  - Alt.1: T_offset <= T_proc,2 is supported, where T_proc,2= max{TMCG,proc,2, TSCG,proc,2}.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

Toffset = Tproc,2mux, where  Tproc,2muxis given by maximum of  {Tproc,2mux,  MCG,Tproc,2mux,  SCG} according to Subclause 9.2.5.

--------------------------------------------------------End text proposal----------------------------------------------------------

- Conclusion

In this contribution, we present our views on some remaining issues on NN-DC power control with the following proposals.

Proposal 1:

- Support the following text proposal to clarify the TDD UL/DL configuration for semi-static power sharing.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Subclauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Subclauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

…

--------------------------------------------------------End text proposal----------------------------------------------------------

Proposal 2:

- For NN-DC dynamic power sharing with power ‘look-ahead’, no new RRC signaling is introduced for T_offset.

  - Alt.1: T_offset <= T_proc,2 is supported, where T_proc,2= max{TMCG,proc,2, TSCG,proc,2}.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

Toffset = Tproc,2mux, where  Tproc,2muxis given by maximum of  {Tproc,2mux,  MCG,Tproc,2mux,  SCG} according to Subclause 9.2.5.

--------------------------------------------------------End text proposal----------------------------------------------------------

- References

- RAN1 #99 Chairman’s notes, Reno, USA, November 18th – 22nd , 2019.