3GPP TSG RAN WG1 #100			R1-2000351

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.11

Source: 	vivo

Title: 	On Rel-16 UE features

Document for:	Discussion & Decision

- Introduction

During the email discussion, an initial list of Rel-16 UE features has been discussed, as shown in the appendix. In this contribution, we present our comments and suggestions on the current list of Rel-16 UE features.

- Comments and suggestion on each UE feature

  - On NRU features

We have following comment/suggestions on NRU UE features

- General comment: most of the features introduced in NRU are applicable to unlicensed band operation only, therefore we should clarify that all the features with type “per band” are applicable to unlicensed band operation only, change “per band” -> “per band (unlicensed band only)”. Some features, however, can be discussed the applicability and potential extension to the licensed band operation, for example the following features

| 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 9, 10, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |
| --- | --- | --- |
| 10-9 | Search space set group switching | Two groups of search space sets |
| 10-11 | SRS starting position |  |

- For 10-1 (LBE channel access mechanism), 10-2 (FBE channel access mechanism)  and 10-3 (PRB interlace PUCCH and PUSCH), we suggest that they are optional with capability signaling

- For 10-7 (LBT bandwidth size), we suggest component 1 (20MHz) and 2 (10MHz) are split into separate UE features (e.g. 10-7 for 20MHz, 10-7a for 10MHz)  and the prerequisite feature groups for both 10-7 and 10-7a should be 10-1(LBE channel access mechanism) or 10-2(FBE channel access mechanism)

- For 10-14 (Non-numerical PDSCH to HARQ-ACK timing), 10-15(Enhanced dynamic HARQ codebook) and 10-16 (One-shot HARQ ACK feedback), these features are introduced to address the HARQ-ACK feedback issue due to channel access uncertainty over unlicensed band, therefore, to be consistent with other features for unlicensed band operation, we suggest adding 10-1 or 10-2 as prerequisite feature groups.

  - On URLLC UE features

We have following comment/suggestions on URLLC UE features

- On 11-1 (Monitoring DCI format 1_2 and DCI format 0_2), the components should be revised to cover the activation/release of UL Type 2 CG or DL SPS, suggest the following revisions

  - Supports monitoring DCI format 1_2 for DL scheduling and DL SPS activation/release

  - Supports monitoring DCI format 0_2 for UL scheduling and UL Type 2 CG activation/release

- On 11-3(More than one PUCCH for HARQ-ACK transmission within a slot), we suggest that sub-slot configuration of “2-symbol*7” and “7-symbol*2” be split into separate UE capabilities

- On 11-4 (Up to two HARQ-ACK codebooks simultaneously constructed for supporting different service types for a UE) component 4), if DCI format 0_2/1_2 is used, the prerequisite feature should be 11-1(Monitoring DCI format 1_2 and DCI format 0_2).

- On 11-5 (PUSCH repetition type B), the component 4) “PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG).” should be dependent on whether UE support DCI format 0_2 (i.e. feature 11-1)

- On 11-6 (PUSCH repetition Type A), we suggest to modify the component part (to focus on the delta compared to Rel-15 features) as the following: “1) PUSCH transmission with  slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16).”

- Additional feature 11-10 should be added for supporting Rel-16 UL Type 2 CG release by DCI format 0_1

  - IIOT UE features

We have following comment/suggestions on IIOT UE feature

- On 12-2, we suggest to split out the joint release of SPS configuration as a separate feature (12-2a) with feature 12-2 as prerequisite.

- Additional feature 11-10 should be added for supporting Rel-16 DL SPS release by DCI format 1_1.

  - Positioning UE features

We have following comment/suggestions on Positioning UE features

- On feature 13-1, suggest the following revisions to the components part

- Component 7 revised to : Support of inter-frequency DL PRS RSRP measurement and report in RRC_CONNECTED state

- Component 8 should be deleted as there was no agreement regarding RSRP measurement quality

- On feature 13-2, suggest the following revisions to the components part

- Component 7 revised to : Support of inter-frequency DL PRS RSTD measurement and report in RRC_CONNECTED state

- Adding additional component “Max number of DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets.” according to the following agreement

Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4

- Additional UE features (13-X) for DL PRS processing should be added according to the following agreement

Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability

- On feature 13b-1, suggest adding additional component as the following

- Component 4): Support of UE Rx-Tx time difference measurement quality metric

  - NR TEIs

About the TEI “Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336”, since Rel-15 UE may also report beamSwitchTiming value of 224 or 336, a new UE capability for Rel-16 TEI feature on Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336 is needed to differentiate UE and gNB behavior.

  - NR eMIMO

We have following comment/suggestions on NR eMIMO UE features

- On feature 16-1 (Multi-beam operation), suggest following revisions to the components part

  - Component 1: Support of L1-SINR based beam selection

    - Support ZP IMR configuration

    - Support NZP IMR configuration

  - Component 2: Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH

    - Band-combination list that supports this operation

    - Max number of different QCL-D RS resources in the active TCI states per band combination;

  - Component 3: Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS

    - Band-combination list that supports this operation

    - Max number of different RS resources in the active spatial relations states per band combination;

  - Component 4: Simultaneous spatial relation update for multiple PUCCH resources

    - max number of PUCCH resource groups per BWP)

  - Component 7: Pathloss reference RS update for PUSCH/SRS via MAC CE

    - max number of configured pathloss reference RS for PUSCH/SRS/PUCCH

  - Component 8: SCell beam failure recovery

    - max # SCells configured for BFR

    - Support of PUCCH-BFR configuration

  - In addition, component 1 (L1-SINR based beam selection) and 8 (SCell beam failure recovery) can be split out as separate UE features, e.g. 16-1a, 16-1b

- On feature 16-2 (Multi-TRP operation for eMBB/URLLC), suggest to split multi-DCI based and single-DCI based into separate UE features, i.e. 16-2a and 16-2b. In addition, suggest following revisions to the component part

  - For multi-DCI based (16-2a), adding following components

    - Increased number of PUCCH transmission within a slot;

    - Out of order PDSCH/PUCSH, including fully/partially time/frequency overlapped PDSCH;

    - Enhanced PDSCH scrambling;

  - For single-DCI based (16-2b), adding following components

    - Support of new DMRS entries;

    - Support of scheme 2a/2b/3/4;

- Feature 16-3 (eType-II codebook) can be split into two separate UE features, i.e. 16-3a Regular eType-II , 16-3b Port selection eType-II

- Feature 16-4 (Power-efficient DL transmission) should be optional

- On feature 16-6 (Low PAPR DMRS), suggest the following additions to component 3

  - Component 3: PUCCH with transform precoding and with pi/2 BPSK modulation

    - PUCCH format3 with pi/2 BPSK modulation

    - PUCCH format4 with pi/2 BPSK modulation

  - NR_CLI_RIM

We suggest feature 17-1 (CLI measurement) to be split in to CLI-RSSI and SRS-RSRP as separate UE features.

  - MR-DC/CA

We suggest the component of feature 18-4 (SCell dormancy with L1 indication) to be revised as the following

# SCell dormancy with L1 indication within DRX active time

  - UE power saving

We have following comment/suggestions on UE power saving features

- For 19-1 (DRX Adaptation), we suggest the following revisions

- Component (1) is revised to the following and the note can be deleted.

  - PS_offset for the detection of  DCI format 2_6 with CRC scrambling by PS-RNTI before DRX ON duration of long DRX

    - The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing

- Component (2) is revised to: DCI format 2_6

- Component (4) should be a separate UE feature (e.g. 19-1a) as UE supporting WUS may not support Scell dormancy behavior. And the description of feature 19-1a should be revised to: Indication of UE Scell dormancy-like behaviour by DCI format 2_6 outside active time

- Adding component (5): UE wakeup or not by configuration when DCI format 2_6 outside active time is not detected

- Adding component (6): UE CSI/L1-RSRP/SRS behavior impacted by DCI format 2_6

- For 19-3 (Maximum MIMO Layer Adaptation), we suggest the following revisions

- Component 1) is revised to : Maximum number of MIMO layer configuration per DL BWP

  - NR IAB

Regarding 20-4, we suggest to further distinguish non-TDM multiplexing schemes by separate capabilities, i.e., capability for MT-TX/DU-TX; MT-TX/DU-RX; MT-RX/DU-TX; MT-RX/DU-RX. Which reflects the agreement made at the RAN1#99 meeting as following.

Agreements:

The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair)

- MT-TX/DU-TX

- MT-TX/DU-RX

- MT-RX/DU-TX

- MT-RX/DU-RX

- Appendix – Current list of Rel-16 UE features

  - NR_2step_RACH

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection based on RSRP threshold - Fall back from 2-step RACH to 4-step RACH - Separate RO with 4-step RO configuration - Shared RO with 4-step RO configuration - msgA PUSCH occasions configuration - Intra-slot frequency hopping for msgA PUSCH - Up to two msgA PUSCH configurations in an UL BWP from UE perspective - Validation of msgA PRACH and PUSCH - Preamble and PUSCH occasion with DMRS mapping - msgA PUSCH scrambling - Multiple DMRS sequences for CP-OFDM based msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - DCI format with CRC scrambled by msgB-RNTI - Monitoring of msgB - HARQ-ACK feedback to a MsgB that contains successRAR - For MsgA PRACH support all preamble formats - For MsgA PRACH, support 15 kHz and 30 kHz in FR1 and 60 and 120 kHz in FR2. - For MsgA PUSCH, support 15 kHz, 30 kHz and 60 kHz in FR1 (maybe 60 kHz can be optional). Support 60 kHz and 120 kHz for FR2 - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1 |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-tep RACH configurations | per band | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-2 | Parallel MsgA and PRACH Msg1/SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and PRACH Msg1/SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA | 9-1, 2-52, 4-1, 2-12, 6-6 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-3 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |

  - NR-unlicensed

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | LBE channel access mechanism | 1. Cat 1 LBT 2. Cat 2 16us LBT 3. Cat 2 25us LBT 4. Cat 4 LBT |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for LBE operation, including Cat 4 LBT, Cat 2 LBT, Cat 1 LBT, contention window adjustment. UE supports either 16 us or 25 us or both 16 and 25 us for Cat-2 LBT. | TBD |
|  | 10-2 | FBE channel access mechanism FFS: if UE as initiating device is supported | 1. Cat 1 LBT 2. Cat 2 LBT |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | TBD |
|  | 10-3 | PRB interlace PUCCH and PUSCH | 1) PRB interlace PUCCH (EPF0, EPF1, EPF2, EPF3) 2) PRB interlace PUSCH | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of PRB interlace PUCCH and PUSCH | TBD |
|  | 10-4 | 16us Cat 2 LBT support Do we still need this as a separate capability, consider it is included in the basic group? |  | FBE channel access or LBE channel access | Yes | N/A |  | Per band | N/A | N/A |  | If 16us Cat 2 LBT is supported | Optional with capability signalling |
|  | 10-5 | 25us Cat 2 LBT support Do we still need this as a separate capability, consider it is included in the basic group? |  | FBE channel access or LBE channel access | Yes | N/A |  | Per band | N/A | N/A |  | If 25us Cat 2 LBT is supported | Optional with capability signalling |
|  | 10-6 | Cat 4 LBT support Do we still need this as a separate capability, consider it is included in the basic group? |  | LBE channel access | Yes | N/A |  | Per band | N/A | N/A |  | If Cat4 LBT with contention window adjustment is supported | Optional with capability signalling |
|  | 10-7 | LBT bandwidth size | 1. 20MHz 2. 10MHz |  | Yes | N/A |  |  | N/A | N/A |  |  |  |
|  | 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 9, 10, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |  | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Support of additional length (other than 2/4/7) for type B PDSCH | Optional with capability signalling |
|  | 10-9 | Search space set group switching | Two groups of search space sets |  | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-10 | RSSI measurement | RSSI measurement | 10-1 or 10-2 | Yes | N/A |  | per band | N/A | N/A |  | RSSI measurement for channel occupancy. |  |
|  | 10-11 | SRS starting position |  | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
|  | 10-12 | PRB interlace PF2 PF3 OCC FFS if need this capability | 1. No OCC 2. OCC2 3. OCC4 | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
|  | 10-13 | Extended CP range for PUSCH/PUCCH FFS if need this capability | Maximum length of CP extension a UE can generate before the first symbol | 10-1 Do we need this for 10-2? | Yes | N/A |  | Per band | N/A | N/A |  | How long a UE can generate the CP extension | Optional with capability signalling |
|  | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | Non-numerical value for dl-DataToUL-ACK |  | Yes | N/A |  | Per band | N/A | N/A |  | If non-numerical K1 value is supported | Optional with capability signalling |
|  | 10-15 | Enhanced dynamic HARQ codebook | Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 |  | Yes | N/A |  | Per band | N/A | N/A |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
|  | 10-16 | One-shot HARQ ACK feedback | HARQ-ACK codebook containing all configured HARQ processes for all configured CCs |  | Yes | N/A |  | Per band | N/A | N/A |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
|  | 10-16a | One-shot HARQ ACK feedback trigger with empty DCI 1_1 | Support of one-shot HARQ ACK feedback with a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | 10-16 | Yes | N/a |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-17 | Multi-PUSCH UL grant | Support of scheduling more than one PUSCH with one UL grant | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-18a | Configured grant with retransmission in CG resources with LBE | Configured grant with retransmission in CG resources with LBE | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-18b | Configured grant with retransmission in CG resources with FBE | Configured grant with retransmission in CG resources with FBE | 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-19 | Number of LBT bandwidth | Number of ED measurements the UE is able to perform simultaneously [Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |
|  | 10-20 | Support search space set association with frequency offsets | Only for new coreset configuration with rb-Offset and the coreset is confined within one LBT bandwidth | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is also the capability to support coreset configuration with rb-Offset | Optional with capability signalling |
|  | 10-21 | Support using ED threshold for UL to DL COT sharing | Support using ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL or CG-UL out of gNB COT to share COT with gNB for DL | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-22 | No gap 2-step RACH msgA transmission | Support transmitting PRACH and PUSCH of msgA without gap in between | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | For licensed case, a minimum gap between PRACH and PUSCH of msgA is introduced, but is not applicable to NR-U | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - To clarify that all the features with type “per band” are applicable to unlicensed band operation only, change “per band” -> “per band (unlicensed band only)” - For 10-1, 10-2 and 10-3, we suggest that they are optional with capability signaling - For 10-7, we suggest component 1 and 2 are split into separate UE features (e.g. 10-7 for 20MHz, 10-7a for 10MHz) and the prerequisite feature groups for both 10-7 and 10-7a should be 10-1 or 10-2 For 10-14, 10-15 and 10-16, to be consistent with other features, we suggest adding 10-1 or 10-2 as prerequisite feature groups. |

  - NR_L1enh_URLLC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in a certain search space set for scheduling the same cell |  | 11-1 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-2 | Rel-16 PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) | 3-5b | Yes | N/A |  | Per UE | No | No |  | This capability is necessary for SCS 15 kHz and 30 kHz. Separate UE capability signalling of the supported combination (X, Y, ). The support of each combination (X, Y, ) can be reported separately for different minimum processing capability | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 |
|  | 11-2a | Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells |  | 11-2 | Yes | N/A |  | Per UE | No |  |  | Can be smaller than 4 CCs | Optional with capability signalling |
|  | 11-2b | Capability on the number of CCs with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | - Capability on the number of CCs with Rel-15 PDCCH monitoring capability - Capability on the number of CCs with Rel-16 PDCCH monitoring capability | 11-2 | Yes | N/A |  | Per UE | No |  |  | Capability on the number of CCs with Rel-15 PDCCH monitoring capability can be smaller than 4 CCs; Capability on the number of CCs with Rel-16 PDCCH monitoring capability can be smaller than 4 CCs; The summation of the minimum of the capability on the number of CCs with Rel-15 PDCCH monitoring capability and the minimum of the capability on the number of CCs with Rel-16 PDCCH monitoring capability is not larger than 4 | Optional with capability signalling |
|  | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least supports sub-slot configurations “2-symbol*7” and “7-symbol*2”. At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-4 | Up to two HARQ-ACK codebooks simultaneously constructed for supporting different service types for a UE | 1) Supports up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-4a | Monitoring a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP |  | 11-1a, 11-4 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-5 | PUSCH repetition type B | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 6) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 7) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-6 | PUSCH repetition Type A | 1) PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be either semi-statically configured (as in Rel-15) or dynamically indicated (as agreed for Rel-16). | 2-12, 2-13, 2-14, 2-15 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-7 | UL cancelation scheme | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication 2) UL cancelation for PUSCH 3) UL cancelation for SRS |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-8 | Enhanced UL power control scheme | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set if SRI is present in the UL grant, and 1 or 2 bits is used to indicate the P0 value if SRI is not present in the UL grant |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On 11-1, the components should be revised to cover the activation/release of UL Type 2 CG or DL SPS, suggest the following revisions - Supports monitoring DCI format 1_2 for DL scheduling and DL SPS activation/release - Supports monitoring DCI format 0_2 for UL scheduling and UL Type 2 CG activation/release - On 11-3, we suggest that sub-slot configuration of “2-symbol*7” and “7-symbol*2” be split into separate UE capabilities - On 11-4 component 4), if DCI format 0_2/1_2 is used, the prerequisite feature should be 11-1. - On 11-5, the component 4) “PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG).” should be dependent on whether UE support DCI format 0_2 (i.e. 11-1) - On 11-6, we suggest to modify the component part (to focus on the delta compared to Rel-15 features) as the following: “1) PUSCH transmission with slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16).” - Additional feature 11-10 should be added for supporting Rel-16 UL Type 2 CG release by DCI format 0_1 |

  - NR_IIOT

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL channels/signals with two priority levels in physical layer - Configuration of priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Prioritization between UL channels/signals with different priority levels - Multiplexing/prioritization between UL channels/signals with the same priority level |  | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]). The relationship between this feature and the feature of up to two HARQ-ACK codebooks should be further discussed. | Optional with capability signaling |
|  | 12-2 | Multiple SPS configurations | - Support of up to 8 SPS configurations in a BWP of a serving cell, including separate activation/release and joint release of SPS configurations - The related HARQ-ACK enhancements to support multiple SPS configurations | downlinkSPS | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On 12-2, we suggest to split out the joint release of SPS configuration as a separate feature (12-2a) with feature 12-2 as prerequisite. - Additional feature 11-10 should be added for supporting Rel-16 DL SPS release by DCI format 1_1 |

  - NR positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR DL positioning | 13-1 | DL PRS RSRP Measurement and Reporting | - Max # of positioning frequency layers supported by UE for DL PRS RSRP measurement and report Values = {1,4}, other values are FFS - Max # of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSRP measurement and report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSRP measurement and report Values are FFS - Max # of DL PRS Resources supported by UE for DL PRS RSRP measurement and report (A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined) Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSRP measurement and report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSRP measurement and report. Values are FFS - Support of inter-frequency DL PRS RSRP measurement and report Support of DL PRS RSRP measurement quality metric |  |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | TBD |  |  | Optional with capability signaling |
|  | 13-2 | DL PRS RSTD Measurement and Reporting | - Max # of positioning frequency layers supported by UE for DL PRS RSTD measurement and report Values = {1,4}, other values are FFS - Max # of DL PRS Resource Sets per TRP supported by UE for DL PRS RSTD measurement and report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSTD measurement and report. Values are FFS - Max # of DL PRS Resources supported by UE for DL PRS RSTD measurement and report (A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined). Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSTD measurement and report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSTD measurement and report. Values are FFS - Support of inter-frequency DL PRS RSTD measurement and report Support of DL PRS RSTD measurement quality metric | 13-1 |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | NA |  |  | Optional with capability signaling |
|  | 13-3 | DL SSB RRM measurements for NR Positioning | Prerequisite – support of SSB measurements for RRM (SS-RSRP, SS-RSRQ, SS-RSRPB) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ, SS-RSRPB) based on Rel.15 for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |
|  | 13-4 | CSI-RS RRM measurements for NR Positioning | Prerequisite – support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  |  |
|  | 13-5 | UE Based Positioning | TBD specific content in RAN2 WG Estimation of location coordinate at UE side and support of corresponding signaling for DL PRS configuration / measurements | 13-2 |  | N/A |  | [per UE] | No | NA |  |  | Optional with capability signaling |
| 13a. NR UL positioning | 13a-1 | Transmission of SRS for NR positioning | - Max # of SRS Resource Sets for positioning supported by UE - Max # of SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP - Support of SSB for neighbor cell DL pathloss estimation and SRS for positioning OLPC - Support of DL PRS for neighbor cell DL pathloss estimation and SRS for positioning OLPC - Spatial Relation of SRS Resources for positioning with DL PRS - Spatial Relation of SRS Resources for positioning with SSB - Spatial Relation of SRS Resources for positioning with other SRS resources for positioning - Support of different SRS resource types for positioning (periodic, semi-persistent, aperiodic) |  |  | N/A |  | [ #1 – per UE #2 – per UE #3 – per UE #4 – per UE #5 – per UE #6 – per UE #7 – per UE ] | No | NA |  |  | Optional with capability signaling |
| 13b. NR DL and UL positioning | 13b-1 | UE Rx-Tx Time Difference Measurement and Reporting | 1. Support of UE Rx-Tx time difference measurement with serving cell 2. Support of UE Rx-Tx time difference measurement with neighbouring cells 3. Support of inter-frequency UE Rx-Tx time difference measurements | 13-2 13a-1 |  | N/A |  | [ #1 - per UE #2 - FFS #3 - per UE ] | No | NA |  |  | Optional with capability signaling |
| 13c. NR RAT Independent Positioning | 13c-1 | TBD in RAN2 WG | [TBD in RAN2 WG] |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | - On feature 13-1, suggest the following revisions to the components part - Component 7 revised to : Support of inter-frequency DL PRS RSRP measurement and report in RRC_CONNECTED state - Component 8 should be deleted as there was no agreement regarding RSRP measurement quality - On feature 13-2, suggest the following revisions to the components part - Component 7 revised to : Support of inter-frequency DL PRS RSTD measurement and report in RRC_CONNECTED state - Adding additional component “Max number of DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets.” according to the following agreement Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4. - Additional UE features (13-X) for DL PRS processing should be added according to the following agreement Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability. - On feature 13b-1, suggest adding additional component as the following - Component 4): Support of UE Rx-Tx time difference measurement quality metric |

  - NR TEI

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier | 5-28 (Rate-matching around LTE CRS) |  | N/A |  | Per UE | No | No |  | For DSS | Optional with capability signaling {2, 3, 4, 5, 6} |
|  | 14-2 | Multiple LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | - Up to two LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | 14-1 (Multiple LTE-CRS rate matching patterns), x-x (mTRP support) |  | N/A |  | [Per band] | No | No |  | For DSS | Optional with capability signaling {supported} |
|  | 14-3 | PDSCH Type B mapping of length 9 and 10 OFDM symbols | - PDSCH Type B mapping of length 9 OFDM symbols PDSCH Type B mapping of length 10 OFDM symbols | 5-6a (PDSCH mapping type B) |  | N/A |  | Per UE | No | No |  | For DSS | Optional with capability signaling {supported} |
|  | 14-4 | One slot periodic TRS configuration for FR1 | - UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | 2-51 (CSI-RS for tracking) |  | N/A |  | Per UE | No | No |  | This capability may not be necessary if 2-51(CSI-RS for tracking) is modified. Following options can be considered. Opt.1: 1 slot periodic TRS is optional as agreed. There is no change on 2-51. Even if UE reports value 2 for MaxBurstLength of csi-RS-ForTracking, support of 1 slot periodic TRS in FR1 depends on this new UE capability signaling. Opt.2: 1 slot periodic TRS is optional as agreed. 2-51 is updated so that UE can report one of three candidate values for MaxBurstLength (e.g., value 1 indicates 1 slot, value 2 indicates both 1 slot and 2 slots, new value 3 indicates 2 slots, where UE is mandated to report value 2 or 3 for FR1 band). New UE capability signaling may not be necessary. | Optional with capability signalling {supported} |
|  | 14-5 | SRS Tx switch with allowing downgrading configuration | 1) Support SRS Tx port switch | 2-53 (SRS resource) |  | N/A |  | Per band | No | No |  | Agreement: •Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} oNote: Detailed signaling design is up to RAN2 | Optional with capability signalling { o{t1r1, t1r2} o{t1r1, t1r2, t1r4} o{t1r1, t1r2, t2r2, t2r4} o{t1r1, t2r2} o{t1r1, t2r2, t4r4} o{t1r1, t1r2, t2r2, t1r4, t2r4} } |
|  | [14-6] | Half-duplex UE behaviour in TDD CA | - Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA |  |  | N/A |  | [Per band combination] | No | No |  | UEs that do not indicate this capability should still be able to operate half-duplex TDD CA if network ensures same transmission direction across all the serving cells | Mandatory with capability signaling for intra-band CA band and for inter-band CA in band combination without RAN4 FG 2-5 capability |
|  | [14-7] | New RACH configuration for FR1 TDD | - new RACH configuration entries with subframe number 2 and/or 7 for RACH periodicity longer than 10 ms |  |  | N/A |  | N/A | No | No |  | Agreement: •A new UE capability is not introduced for this TEI, i.e., it is a mandatory UE feature for Rel-16. | Mandatory without capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Since Rel-15 UE may also report beamSwitchTiming value of 224 or 336, a new UE capability for Rel-16 TEI feature on Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336 is needed to differentiate UE and gNB behavior. One possible solution is to introduce following UE capability in 38.331. beamSwitchTiming–R16 SEQUENCE { scs-60kHz ENUMERATED { sym224, sym336} OPTIONAL, scs-120kHz ENUMERATED { sym224, sym336} OPTIONAL } OPTIONAL, |

  - 5G_V2X_NRSL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15. 5G_V2X_NRSL | 15-1 | Receiving NR sidelink mode 1 and 2 | 1) UE can receive PSCCH/PSSCH transmitted using NR sidelink mode 1 and 2. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot counting both PSCCH and PSSCH. 4) UE supports normal 64QAM MCS table and 256QAM MCS table. |  |  | No | \ | Per band | No | [Yes] | N.A. | [This is the basic FG for sidelink] | Optional with capability signaling. [For UE supports NR sidelink, UE must indicate this FG is supported] |
|  | 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling and configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. 2) UE supports normal 64QAM MCS table and 256QAM MCS table. |  |  | No |  | Per band Note: Random selection in the exceptional pool is supported. | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-3 | Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu and preconfiguration. 2) UE supports normal 64QAM MCS table and 256QAM MCS table. | 15-1 |  | No |  | Per band Note: Random selection in the exceptional pool is supported. | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-4 | S-SSB for NR sidelink mode 1 and 2 | 1) UE can transmit and receive S-SSB in NR sidelink mode 1 and 2 |  |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB. 2) UE can adjust its radio parameters based on CBR measurement. |  |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-6 | Short-term time-scale TDM for in-device coexistence | 1) Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using SPS in LTE sidelink mode 3 scheduled by NR Uu. |  |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-8 | UE inter-RAT sidelink control processing capability | 1) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. | 15-7 |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling X in {0.75 ms, 1 ms, [1.25 ms], [1.5 ms], 2 ms, 4 ms, 5 ms, 8 ms, 10 ms, 20 ms} |
|  | 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using LTE sidelink mode 4 configured by NR Uu and preconfiguration. |  |  | No |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling |
|  | 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 |  | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission. | Per band | No | [Yes] | N.A. | Reception of 256QAM is FFS. | Optional with capability signalling |
|  | 15-11 | [PSFCH] | [1) UE can transmit and receive PSFCH in NR sidelink mode 1 and 2.] |  |  | [Yes] |  | Per band | No | [Yes] | N.A. |  | [Optional with capability signalling] |
|  | 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE additionally supports low-spectral efficiency 64QAM MCS table. |  |  | Yes | UE supports normal 64QAM MCS table and 256QAM MCS table. | Per band | No | [Yes] | N.A. |  |  |
|  | 15-13 | PT-RS | 1) UE supports NR sidelink transmission or reception using PT-RS. | At least one of 15-1, 15-2, 15-3 |  | Yes |  | Per band | No | [Yes] | N.A. |  | Optional with capability signalling. Mandatory in FR2 |

| Company | Comments/Questions/Suggestions |
| --- | --- |

  - NR_eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_eMIMO | 16-1 | Multi-beam operation | - Support of L1-SINR based beam selection - Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - Simultaneous spatial relation update for multiple PUCCH resources - Default spatial relation for dedicated-PUCCH/SRS - Spatial relation update for AP-SRS via MAC CE - Pathloss reference RS update for PUSCH/SRS via MAC CE - SCell beam failure recovery (max # SCells configured for BFR) | TBD for #1 2-1, 2-4 for #2 2-53, 2-59 for #3-#6 8-2 for #7 2-31 for #8 |  | N/A |  | Per band for #1, #4, #5, #6 Per UE for #2, #3, #7, #8 | N | N for #1, #2, #7, #8 Y for #3 - #6 |  |  | Optional |
|  | 16-2 | Multi-TRP operation for eMBB/URLLC | Multi-DCI-based: - The maximum number of CORESET configured per “PDCCH-Config” - The maximum number of CORESET configured per TRP (i.e. same value of HigherLayerIndexPerCORESET (if configured) per “PDCCH-Config”) - The value of R for BD/CCE - Whether the UE shall rate match around configured CRS patterns which is associated with HigherLayerIndexPerCORESET (if configured) Single-DCI-based: - Whether supporting two PTRS ports - For URLLC scheme 2b, whether the UE can support CW soft combining - For URLLC scheme 3 and 4, supported maximum TBS size | TBD |  | N/A |  | Per UE | N | TBD |  |  | TBD |
|  | 16-3 | eType-II codebook | Regular eType-II - 8 parameter combinations - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - CBSR and rank restriction - UCI omission Port selection eType-II - 6 parameter combinations (combos with L=6 don’t apply) - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - CBSR and rank restriction - UCI omission | TBD |  | N/A |  | Per UE | N | N |  |  | Optional |
|  | 16-4 | Power-efficient DL transmission | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | Mandatory |
|  | 16-5 | Power-efficient UL transmission | UL full power transmission (mode [0], 1, 2) - New UL codebook set(s) - Multiple SRS resources in set with different number of ports - UE indication of TPMI(s) which delivers full power | [2-13, 2-14] for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-6 | Low PAPR DMRS | - PUSCH without transform precoding - PUSCH with transform precoding and with pi/2 BPSK modulation - PUCCH with transform precoding and with pi/2 BPSK modulation | TBD for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Vivo | - On feature 16-1, suggest following revisions to the components part - Component 1: Support of L1-SINR based beam selection - Support ZP IMR configuration - Support NZP IMR configuration - Component 2: Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - Band-combination list that supports this operation - Max number of different QCL-D RS resources in the active TCI states per band combination; - Component 3: Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - Band-combination list that supports this operation - Max number of different RS resources in the active spatial relations states per band combination; - Component 4: Simultaneous spatial relation update for multiple PUCCH resources - max number of PUCCH resource groups per BWP) - Component 7: Pathloss reference RS update for PUSCH/SRS via MAC CE - max number of configured pathloss reference RS for PUSCH/SRS/PUCCH - Component 8: SCell beam failure recovery - max # SCells configured for BFR - Support of PUCCH-BFR configuration - In addition, component 1 and 8 can be split out as separate UE features, e.g. 16-1a, 16-1b - On feature 16-2, suggest to split multi-DCI based and single-DCI based into separate UE features, i.e. 16-2a and 16-2b. In addition, suggest following revisions to the component part - For multi-DCI based (16-2a), adding following components - Increased number of PUCCH transmission within a slot; - Out of order PDSCH/PUCSH, including fully/partially time/frequency overlapped PDSCH; - Enhanced PDSCH scrambling; - For single-DCI based (16-2b), adding following components - Support of new DMRS entries; - Support of scheme 2a/2b/3/4; - Feature 16-3 can be split into two separate UE features, i.e. 16-3a Regular eType-II , 16-3b Port selection eType-II - Feature 16-4 should be optional - On feature 16-6, suggest the following additions to component 3 - Component 3: PUCCH with transform precoding and with pi/2 BPSK modulation - PUCCH format3 with pi/2 BPSK modulation - PUCCH format4 with pi/2 BPSK modulation |

  - NR_CLI_RIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17. NR_CLI_RIM | 17-1 | CLI measurement | CLI measurement (both CLI-RSSI and SRS-RSRP) |  |  | N/A |  | Per UE |  | No |  | UE operates CLI-RSSI measurement and SRS-RSRP measurement. | Optional with capability signalling |
|  | 17-2 | [Simultaneous reception of PDSCH and CLI-RSSI measurement resource] | [simultaneousRxPDSCHCLIRSSIMeasResource] | 17-1 |  | N/A |  | [Per UE] |  | [No] |  | [UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s)] | [Optional with capability signalling] |
|  | 17-3 | [Simultaneous reception of PDSCH and SRS-RSRP measurement resource] | [simultaneousRxPDSCHSRSRSRPMeasResource] | 17-1 |  | N/A |  | [Per UE] |  | [No] |  | [UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s).] | [Optional with capability signalling] |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | On 17-1, we suggest to split CLI-RSSI and SRS-RSRP as separate UE features. |

  - MR-DC/CA enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | UL power sharing for DC | UL power sharing modes supported by the UE |  |  | N/A |  | Per band combination |  |  |  | Absence means DC not supported. UE supporting DC must indicate support for at least 1 UL power sharing scheme. Indication for support of any combination of the schemes needs to be possible. {semi-static, dynamic without lookahead, dynamic with lookahead}. TBD if semi-static to be split in two modes | Optional with capability signalling |
|  | 18-2 | 1Tx operation for TDD intra-band EN-DC | 1Tx operation for TDD intra-band EN-DC |  |  | N/A |  | Per band combination | Applicable to intra.band TDD-TDD EN-DC/NE-DC only | Applicable to FR1 only |  | Extension of the R15 capability singleUL-Transmission to intra-band TDD EN-DC | Optional with capability signalling |
|  | 18-3 | TDM restriction to an FD-LTE cell in EN-DC (TDM pattern for 2Tx UE) | TDM restriction to an FD-LTE cell in EN-DC |  |  | N/A |  | Per band combination | Applicable in EN-DC/NE-DC with FD-LTE only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE: Indicates that the UE supports restricting the TD-LTE uplink transmission occasions with a TDM pattern. | Optional with capability signalling |
|  | 18-4 | SCell dormancy with L1 indication | SCell dormancy with L1 indication |  |  | N/A |  | Per band combination | No | No |  | Support L1-based mechanism to indicate dormancy/non-dormancy status for group of Scells | Optional with capability signalling |
|  | 18-5 | Cross-carrier scheduling with different SCS | Cross-carrier scheduling with different SCS |  |  | N/A |  | Per band combination | TBD | No |  | crossCarrierScheduling-OtherSCS Indicates whether the UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different. {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} Note: this capability exists in Rel-15 38.306, but the functionality was not completed in Rel-15. | Optional with capability signalling |
|  | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS |  |  | N/A |  | Per band combination | No | No |  | Indicates whether the UE supports triggering of aperiodic CSI-RS where the CSI-RS is on a carrier with one sub-carrier spacing and the triggering PDCCH is on another carrier with a different sub-carrier spacing compared to the carrier with CSI-RS. | Optional with capability signalling |
|  | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries |  |  | N/A |  | Per band combination | No | No |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group |  |  | N/A |  |  | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [Optional with capability signalling or conditioned to support for multiple PUCCH groups] |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | On 18-4 component, we suggest the following revisions - SCell dormancy with L1 indication within DRX active time |

  - UE Power Saving

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - PS_offset for the detection of DCI format 3 with CRC scrambling by PS-RNTI before DRX ON duration - DCI format 3_0 - Indication of UE wakeup or not - Indication of UE dormancy-like behavior |  |  |  |  | N/A |  | [Per UE] | No | No |  | PS_offset is determined based on UE capability | [Mandatory without capability signalling] |
|  | 19-2 | Cross Slot Scheduling | - Enhancement of DCI format 0_1 and 1_1 - DCI Indication of minimum scheduling offset |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Maximum MIMO layer per DL BWP |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |
|  | 19-4 | UE assistance information | - Preferred c-DRX configuration - Preferred aggregated bandwidth - Preferred SCell configuration - Preferred MIMO configuration - Release Request |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |
|  | 19-5 | RRM measurement relaxation | - Relaxation of intra and inter-frequency RRM measurements of neighboring cells in RRC_IDLE/Inactive |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | For 19-1, we suggest the following revisions - Component (1) is revised to the following and the note can be deleted. - PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI before DRX ON duration of long DRX - The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing - Component (2) is revised to: DCI format 2_6 - Component (4) should be a separate UE feature (e.g. 19-1a) as UE supporting WUS may not support Scell dormancy behavior. And the description of feature 19-1a should be revised to: Indication of UE Scell dormancy-like behaviour by DCI format 2_6 outside active time - Adding component (5): UE wakeup or not by configuration when DCI format 2_6 outside active time is not detected - Adding component (6): UE CSI/L1-RSRP/SRS behavior impacted by DCI format 2_6 For 19-3, we suggest the following revision - Component 1) is revised to : Maximum number of MIMO layer configuration per DL BWP |

  - NR_IAB

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20. NR_IAB | 20-1 | Inter-IAB-node discovery and measurements: SSB transmission Configuration | Support up to 4 STCs configured for an IAB node DU per cell per frequency location, including IAB-specific SSB transmission periodicities |  |  |  | Yes | N/A | Separate configuration of SSB transmissions for access UE and IAB node measurements is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions/receptions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-4 | IAB node non-TDM multiplexing capability | TDM not required between IAB-MT and IAB-DU functions |  |  |  | Yes | N/A | Only TDM-based resource configurations can be supported | [per {MT CC, DU cell} pair] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-5 | UL-Flexible-DL slot formats | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support DCI Format [4]_0 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-7 | Case 1 OTA timing alignment | Support IAB-DU DL timing alignment with parent node based on NTA+NTA,offset⋅Tc/2+Tdelta |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding 20-4, we suggest to further distinguish non-TDM multiplexing schemes by separate capabilities, i.e., capability for MT-TX/DU-TX; MT-TX/DU-RX; MT-RX/DU-TX; MT-RX/DU-RX. Which reflects the agreement made at the RAN1#99 meeting as following. Agreements: The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair) - MT-TX/DU-TX - MT-TX/DU-RX - MT-RX/DU-TX - MT-RX/DU-RX |

  - Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. Mobility Enhancement | 21-1 | Intra-frequency DAPS HO | Indicates support of intra-frequency DAPS-HO for a given [band and band combination]. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO for intra-frequency HO scenarios | [Per BC] (Note: requires RAN2 confirmation) | No | Yes | N/A |  | Optional with capability signalling |
|  | 21-2 | UE power sharing for DAPS HO | Indicates support of UL power sharing during DAPS-HO operation, i.e. semi-static or dynamic. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The UE is only able to perform semi-static power allocation for source and target cell | [Per BC] | No | Yes | N/A |  | [Mandatory with capability signalling] |
|  | 21-3 | PDCCH blind detection for MCG1 | Indicates PDCCH blind decoding capability supported in the first MCG (intended for target cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | [Per BC] | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network along with 21-4 in order to correctly define UE behaviour | Optional with capability signalling {values: 2, 3, … 16} |
|  | 21-4 | PDCCH blind detection for MCG2 | Indicates PDCCH blind decoding capability supported in the second MCG (intended for source cell in HO) when in DAPS-HO. Value range should be {[2], …, 16}. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | [Per BC] | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network along with 21-3 in order to correctly define UE behaviour | Optional with capability signalling {values: 2, 3, … 16} |

| Company | Comments/Questions/Suggestions |
| --- | --- |

  - Potential change/update on existing UE features for Rel-16 UE

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Sprint | DPS is supposed to be mandatory. We agreed to have capability signaling for Rel-15 devices due to implementation issues that some vendors had, however now we would like to see 'Mandatory with capability bit set to 1' for subfeature 8.1 in Rel-16. 8. UL TPC; 8-1; Dynamic power sharing for LTE-NR DC; When total transmission power exceeds Pcmax, UE scales NR transmission power.; EN-DC; No; N/A; Per UE; N/A; N/A; Mandatory with capability signalling set to 1. |