3GPP TSG RAN WG1 #100                                                                                    R1-2000360

e-Meeting, February 24th – March 6th, 2020

Source:	ZTE

Title:                 Remaining issues on enhancements for UL configured grant transmission

Agenda item:	7.2.5.6

Document for:  Discussion and Decision

- Introduction

In the RAN1#99 meeting, the following agreements on enhanced UL configured grant transmissions were reached in [1].

Agreements:

To align the bit width of each field of DCI format 0_2 with CS-RNTI with NDI=0 to C-RNTI:

- use Rel.15 rule with additional exceptions for the DCI format 0_2, a UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell.

  - FFS additional exceptions at least for the parameters not introduced for CG Type 2 (example: ResourceAllocationType1-granularity-ForDCIFormat0_2)

  - No additional RRC impact is expected

In this contribution, we mainly discuss the remaining issues related to above agreements, along with the text proposals for discussion.

- Discussion

To align the bit width of each field of DCI format 0_2 with CS-RNTI with NDI=0 to C-RNTI, it was agreed to reuse Rel.15 rules with additional exceptions for the DCI format 0_2. However, the additional exceptions are still FFS.

To build a type 2 CG PUSCH, a UE applies the following parameters in Rel-15.

- The parameters are configured in configuredGrantConfig.

- The parameters are indicated by activation DCI.

- The parameters that are defined in pusch-Config but neither in activation DCI nor in ConfiguredGrantConfig. These exceptions includes dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH.

In Rel-16 URLLC, two of the current Rel-15 exceptions, i.e., codebookSubset and maxRank have been changed to codebookSubset-ForDCIFormat0_2 and maxRank-ForDCIFormat0_2 for DCI format 0_2. In addition, some new parameters are introduced for DCI format 0_2. In the following, we list the parameters applicable for type 2 CG PUSCH while only specified in PUSCH-Config.

- ResourceAllocationType1-granularity-ForDCIFormat0_2

- DMRSsequenceinitialization-ForDCIFormat0_2

- PUSCHRepTypeIndicator-ForDCIFormat0_2

- NumberofbitsforRV-ForDCIFormat0_2

- frequencyHoppingOffsetLists-ForDCIFormat0_2

- PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2

Above parameters are defined in pusch-Config but neither in activation DCI format 0_2 nor in configuredGrantConfig. Thus, there parameters should be regarded as additional exceptions.

- Proposal 1: For type 2 CG PUSCH activated by DCI format 0_2 for Rel-16 URLLC, a UE follows the following parameters defined in pusch-Config: ResourceAllocationType1-granularity-ForDCIFormat0_2, DMRSsequenceinitialization-ForDCIFormat0_2, PUSCHRepTypeIndicator-ForDCIFormat0_2, NumberofbitsforRV-ForDCIFormat0_2, frequencyHoppingOffsetLists-ForDCIFormat0_2 and PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2.

Note that, frequencyHoppingOffsetLists-ForDCIFormat0_2 has already been clarified in Section 6.3.1 in TS 38.214 as below. Similarly, PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2 which is configured by pusch-TimeDomainAllocationList-ForDCIformat0_2 has been clarified in Table 6.1.2.1.1-1B in TS 38.214. Thus, these two parameters are not reflected in the text proposal at the end of this contribution.

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. For a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists-ForDCIFormat0_2 in pusch-Config.

Observation 1: Similar to Rel-15, it has been clarified in Rel-16 spec that a UE follows frequencyHoppingOffsetLists-ForDCIFormat0_2 and PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2 in pusch-Config for type 2 CG PUSCH.

In addition, there are also some new parameters introduced for DCI format 0_1 in Rel-16 URLLC. We find the parameter PUSCHRepTypeIndicator-ForDCIFormat0_1 is only specified in pusch-Config, and should be the exceptions in Rel-16 URLLC for DCI format 0_1.

- Proposal 2: For type 2 CG PUSCH activated by DCI format 0_1 for Rel-16 URLLC, a UE follows PUSCHRepTypeIndicator-ForDCIFormat0_1 in pusch-Config.

- Below we provided the corresponding text proposal to TS 38.214 on the exception parameters for type 2 CG PUSCH activating by DCI format 0_1 and 0_2 for Rel-16 URLLC[2].

--------------------------------------------Text Proposal 1 for Section 6.1 in TS 38.214 [2]-----------------------------------

- 6.1 UE procedure for transmitting the physical uplink shared channel <---------------------------Other parts are omitted -------------------------------> For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, PUSCHRepTypeIndicator-ForDCIFormat0_1, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset-ForDCIFormat0_2, maxRank-ForDCIFormat0_2, scaling of UCI-OnPUSCH, ResourceAllocationType1-granularity-ForDCIFormat0_2, DMRSsequenceinitialization-ForDCIFormat0_2, PUSCHRepTypeIndicator-ForDCIFormat0_2, NumberofbitsforRV-ForDCIFormat0_2, which are provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in Clause 6.1.4 for the PUSCH transmission corresponding to a configured grant. <---------------------------Other parts are omitted ------------------------------->

- Conclusion

According to the analysis given above, we have the following proposals:

Observation 1: Similar to Rel-15, it has been clarified in Rel-16 spec that a UE follows frequencyHoppingOffsetLists-ForDCIFormat0_2 and PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2 in pusch-Config for type 2 CG PUSCH.

- Proposal 1: For type 2 CG PUSCH activated by DCI format 0_2 for Rel-16 URLLC, a UE follows the following parameters defined in pusch-Config: ResourceAllocationType1-granularity-ForDCIFormat0_2, DMRSsequenceinitialization-ForDCIFormat0_2, PUSCHRepTypeIndicator-ForDCIFormat0_2, NumberofbitsforRV-ForDCIFormat0_2, frequencyHoppingOffsetLists-ForDCIFormat0_2 and PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2.

- Proposal 2: For type 2 CG PUSCH activated by DCI format 0_1 for Rel-16 URLLC, a UE follows PUSCHRepTypeIndicator-ForDCIFormat0_1 in pusch-Config.

- Reference

- 3GPP RAN1 #99, chairman’s notes.

- 3GPP TS 38.214-g00, Physical layer procedures for data, Release 16.