3GPP TSG RAN WG1 Meeting #100                                            R1-2000374

e-Meeting, February 24th – March 6th, 2020

Source:               ZTE

Title:                    Correction on use of LTE control channel region

Agenda item:      6.2.1.8

Document for:    Discussion and Decision

Discussion

In Rel-16 MTC enhancement, there had the following RAN1 agreement on use of LTE control channel region for DL transmission [1].

Agreement When performing rate-matching for PDSCH in the LTE control region, all resource elements shall be counted in the PDSCH mapping, except for the ones reserved for cell-specific reference signals on any antenna port.

In section 6.4.1 of current Rel-16 36.211, the agreement has been addressed as the following:

“-	If PDSCH transmission in the LTE control region is configured by higher layer parameter ce-dl-lte-control-region-config, the mapping to resource elements (k,l) not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l, starting from l=lDataStart in the first slot to the last OFDM symbol available for downlink transmission in the subframe, then from l=0 in the first slot to l=lDataStart-1 in the first slot, where lDataStart  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].”.

However, “for other purposes” is not clear in the specification. From the agreement, when performing rate-matching of PDSCH in the LTE DL control region, only cell-specific reference signals are not counted as the mapping resources. The specification should clearly state that resource elements not reserved for cell-specific reference signals are counted as mapping resources.

- Proposal 1: Adopt the changes in Text proposal #1 related to section 6.4.1 of 36.211.

Text proposal #1 in section 6.4.1 of 36.211:

# ------------------------------------------------ Start of Text proposal --------------------------------------

6.4.1	Physical downlink shared channel for BL/CE UEs

<Unchanged parts are omitted>

-	If PDSCH transmission in the LTE control region is configured by higher layer parameter ce-dl-lte-control-region-config, the mapping to resource elements (k,l) not reserved for cell-specific reference signals shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l, starting from l=lDataStart in the first slot to the last OFDM symbol available for downlink transmission in the subframe, then from l=0 in the first slot to l=lDataStart-1 in the first slot, where lDataStart  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].

<Unchanged parts are omitted>

# --------------------------------------------  End of Text proposal -------------------------------------------

Conclusions

In this contribution, we have discussed an issue related to use of LTE control channel region for DL transmission and we make the following proposals:

- Proposal 1: Adopt the changes in Text proposal #1 related to section 6.4.1 of 36.211.

Text proposal #1 in section 6.4.1 of 36.211:

# ------------------------------------------------ Start of Text proposal --------------------------------------

6.4.1	Physical downlink shared channel for BL/CE UEs

<Unchanged parts are omitted>

-	If PDSCH transmission in the LTE control region is configured by higher layer parameter ce-dl-lte-control-region-config, the mapping to resource elements (k,l) not reserved for cell-specific reference signals shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l, starting from l=lDataStart in the first slot to the last OFDM symbol available for downlink transmission in the subframe, then from l=0 in the first slot to l=lDataStart-1 in the first slot, where lDataStart  is given by clause 7.1.6.4 of 3GPP TS 36.213 [4].

<Unchanged parts are omitted>

# --------------------------------------------  End of Text proposal -------------------------------------------

- References

- 3GPP, R1-1913594, RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE, WI rapporteur (Ericsson), RAN1 #99