3GPP TSG RAN WG1 Meeting #100	R1-2000397

e-Meeting, February 24th – March 6th, 2020

Title:	Remaining issues of the 2-step RACH procedures

Source: 	ZTE, Sanechips

Agenda Item:	7.2.1.2

Document for:	Discussion and Decision

# Introduction

This document provides two TPs.

The first TP is to capture the agreement in RAN1#99 meeting which handles the LSB bits of SFN in DCI format 1_0 and change the terminology of RA-RNTI to msgB-RNTI to align the definition in 38.212 and 38.321.

The second TP is to add the introduction of CSI-RS based CFRA in section 8.2A of 38.213.

# Text Proposal in TS 38.213 for LSB bits of SFN and msgB-RNTI

In [1], the agreement about LSB bits of SFN is:

Agreements: The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by msgB-RNTI (parameters in squared brackets depend on the other proposals): - Frequency domain resource assignment – bits - Time domain resource assignment – 4 bits - VRB-to-PRB mapping – 1 bit - Modulation and coding scheme – 5 bits - TB scaling – 2 bits - [LSBs of SFN – 2 bits] - Reserved bits – [14] bits Note: The interpretation of the “LSBs of SFN” may be different for the hand over case

Moreover, for type-2 random access procedure, in response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RNTI which is temporally defined as RA-RNTI in specification, while the definition in 38.212 and 38.321 is msgB-RNTI. To align the definition among specifications, the change of the terminology of RA-RNTI to msgB-RNTI is proposed.

The following changes to TS 38.213[2] are proposed:

--------------------------------------------------------- Start of TP ---------------------------------------------------------------------

8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MSGB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB- ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MSGB-RNTI and the DCI includes the two LSB bits of SFN corresponding to the PRACH occasion used to transmit the MSGA preamble, and the UE detects a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block.If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI including the two LSB bits of SFN corresponding to the PRACH occasion used to transmit the MSGA preamble within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding MSGB-RNTI, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI but the two LSB bits of the SFN in the DCI does not match the SFN of PRACH occasion used to transmit the MSGA preamble, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

<unchanged part omitted>

--------------------------------------------------------- End of TP ----------------------------------------------------------------------

# Text Proposal in TS 38.213 for CSI-RS based type-2 CFRA

This TP is for type-2 contention free RACH in 38.213 which handles the issue of supporting CSI-RS.

The agreement in previous RAN2 meeting is:

Agreements for HO 2-step RA: RAN2 assumes that SSB and CSI-RS based 2-step CFRA can be supported. We assume that if there are RAN1 impact then CSI-RS configuration will not be supported.

In the description in 38.213[2], there are two paragraphs that are directly related to CSI-RS based RACH.

8.1 Random access preamble <---------------------------Other parts are omitted-------------------------------> For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period. <---------------------------Other parts are omitted------------------------------->

8.2 Random access response - Type-1 random access procedure <---------------------------Other parts are omitted-------------------------------> If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Subclause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0. <---------------------------Other parts are omitted------------------------------->

The section 8.1 is applied for both type-1 and type-2 random access.

For the type-2 RACH procedure, the second paragraph could be copied and pasted to the section 8.2A, and there is no other RAN1 impact.

The following changes to TS 38.213[2] are proposed:

--------------------------------------------------------- Start of TP ---------------------------------------------------------------------

8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MSGB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB- ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MSGB-RNTI, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Subclause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block.If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding MSGB-RNTI, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

<---------------------------Other parts are omitted------------------------------->

# Reference

[1] Chairman’s Notes RAN1#99 final

[2] TS38.213-g00

[2] Session_Notes_NR-U_2-stepRA_PowSav_NTN_R2-108_11_22_19_11-30