3GPP TSG RAN WG1 #100-e	         	 R1-2000416

e-meeting, February 24-28, 2020

Agenda item:		5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the RAN2 LS on consistent Uplink LBT failure detection mechanism

Document for:		Discussion and Decision

# 1	Introduction

RAN2 sent RAN1 an LS on consistent Uplink LBT failure detection mechanism in [1]. RAN2 has agreed to use LBT failure indication from L1 to MAC layer to detect consistent UL LBT failures at the UE. So far (according to RAN2’s understanding) such indication has not been specified in any L1 specifications. In the LS, RAN2 have the following action for RAN1:

To RAN1 ACTION: . RAN2 would like to respectfully request RAN1 to take the above RAN2 agreements into account and introduce uplink LBT failure indication in their specifications if needed.

In this TDoc we discuss the implications of this LS in RAN1.

# 2. Discussion

For the moment, there is no notion of UL LBT failure or indication thereof, in the RAN1 specs. On the other hand, introducing such functionality is rather straight forward.

In TS 38.213, for example, in the case of PRACH power ramping there is already similar language in place:

If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321].

For the UL LBT failure indication, TS 37.213 seems to be the most appropriate place, as channel access related procedures are captured there. A simple way to include the wording would be as follows:

TS 37.213

-------- Beginning of Text Proposal ------------

## 4.2	Uplink channel access procedures

A UE performing transmission(s) on LAA Scell(s), an eNB scheduling or configuring UL transmission(s) for a UE performing transmission(s) on LAA Scell(s), and a UE performing transmission(s) on channel(s) and a gNB scheduling or configuring UL transmission(s) for a UE performing transmissions on channel(s) shall perform the procedures described in this subclause for the UE to access the channel(s) on which the  transmission(s) are performed.

In this subclause, transmissions from a UE are considered as separate UL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in subclause 4.2.3 when applicable.

A UE performs channel access procedures in this subclause unless the higher layer parameter ChannelAccessMode-r16 is provided and ChannelAccessMode-r16 =’ semistatic’.

If a UE fails to access the channel(s) prior to an intended UL transmission, Layer 1 notifies higher layers about the channel access failure.

---------- End of Text Proposal -------------

# 3. Conclusions

In this contribution, we have discussed the RAN2 LS on consistent Uplink LBT failure detection mechanism. Based on the discussion, we make the following observations and proposals:

Proposal: Agree the Text Proposal for 37.213 in Section 2 and send a corresponding reply LS to RAN2.

# References

- R2-1916380 LS on consistent Uplink LBT failure detection mechanism, RAN2