3GPP TSG RAN WG1 #100		R1-2000456

e-Meeting, February 24th – 28th, 2020

Source:	OPPO

Title:	Text proposals for CSI enhancements for MU-MIMO support

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

Introduction

This contribution provides some TPs for CSI enhancements for MU-MIMO support.

Discussion

Text proposals for MU-CSI

For Enhanced Type II codebook, size-2LM Bitmap is used to indicate non-zeros coefficient indices and reported in UCI part 2. Only non-zero coefficients in the selected subset are defined and reported. The corresponding description in current specification TS 38.214 is as follows [1],

TS 38.214 V16.0.0 5.2.2.2.5 Enhanced Type II Codebook <omitted text> The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported …. - The KNZ-υ indicators kl,i,f(2) for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. - The KNZ-υ indicators cl,i,f for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. - The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(2)=0 are not reported and the corresponding kl,i,f(3)=0 are reported. - The remaining 2L⋅Mν⋅ν-KNZ indicators cl,i,f=0 are not reported and the corresponding kl,i,f(3)=0 are reported. ….. For coefficients with kl,i,f(3)=0, amplitude and phase are set to zero, i.e., pl,i,f(2)=0 and φl,i,f=0. <omitted text>

It is unclear to say ‘The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(2)=0 are not reported’, since remaining 2L⋅Mν⋅ν-KNZ indicators  are corresponding to zero coefficients, which means that the amplitude of the corresponding coefficients are zeros. However, kl,i,f(2)=0 does not mean zero coefficient and maps to  pl,i,f(2)=18√2 in amplitude table which is a meaningful value. We think the intention is to say UE only report  KNZ NZC coefficients corresponds to bitmap whose bits are 1, which  shares the same meaning as ‘The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported’. So we  propose to remove ‘= 0’ from the above-mentioned sentence.

Besides, ‘The remaining 2L⋅Mν⋅ν-KNZ indicators cl,i,f=0 are not reported’ is inconsistent with ‘pl,i,f(2)=0 and φl,i,f=0’, since cl,i,f=0 means φl,i,f=1.  So we  also propose to remove ‘= 0’ from this sentence.

Based on the above observations, we have the following text proposal:

TS 38.214 V16.0.0 5.2.2.2.5 Enhanced Type II Codebook <omitted text> The KNZ-υ indicators kl,i,f(2) for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. The KNZ-υ indicators cl,i,f for which i≠il*, f≠0 and kl,i,f(3)=1 are reported. The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(2)=0kl,i,f(2) are not reported and the corresponding kl,i,f(3)=0 are reported. The remaining 2L⋅Mν⋅ν-KNZ indicators cl,i,f=0 cl,i,f are not reported and the corresponding kl,i,f(3)=0 are reported. <omitted text>

Conclusion

In this contribution, we provide one TP for Type II CSI enhancement for MU-MIMO.

References

- TS 38.214, NR; Physical layer procedures for data, V16.0.0