3GPP TSG RAN WG1 #100		R1-2000457

e-Meeting, February 24th – March 6th, 2020

Source:	OPPO

Title:	Text proposals for enhancements on Multi-TRP and panel Transmission

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

Introduction

This contribution provides some TPs for multiple TRPs/panels based transmission.

Discussion

Text proposals for single DCI based M-TRP transmission

DMRS tables for single DCI based transmission

In RAN1#99 meeting, we have the following agreements on additional DMRS entry:

Agreement

DMRS entry {0, 2, 3} can be expected by the UE only when two TCI states are indicated

Agreement

For DMRS type-2, for layer combination 1+2, at least support DMRS entry {0,2,3} with 2 CDM groups without data

- {0,2,3} is used assuming SU-MIMO

For DMRS type-1, {0,2,3} is used assuming SU-MIMO

In current 38.212 [1], DMRS port {0,2,3} is introduced via some new DMRS tables (Tables 7.3.1.2.2-1A/2A/3A/4A) for Rel-16 UEs. However, the restriction (two TCI states and SU-MIMO) on the application of the new tables is not captured in specification and should be added to 38.214. Furthermore, with the restriction that DMRS port {0,2,3} can only be used with two TCI states, it is not needed to use paired DMRS tables for different DMRS configurations. Adding entry of {0,2,3} to Rel-15 table is sufficient. Hence, we have the following TPs.

// TP1 for 38.212//

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

//omitted part//

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Subclause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

Table 7.3.1.2.2-1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12 | 2 | 0,2,3 |
| 13-15 | Reserved | Reserved |

Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | 2 | 0,2,3 | 1 |  |  |  |  |

Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 2 | 0,2,3 |  |  |  |
| 25-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58 | 2 | 0,2,3 | 1 |  |  |  |  |
| 59-63 | Reserved | Reserved | Reserved |  |  |  |  |

//End for 38.212//

// TP1 for 38.214//

#### 5.1.6.2 DM-RS reception procedure A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 1_1 as described in Subclause 7.3.1.2 of [5, TS 38.212]. UE is expected to be assigned with antenna port {0,2,3} only when two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ of the scheduling DCI, and the UE does not expect potential co-scheduled UE(s) in other DM-RS ports of the corresponding CDM group. For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Subclause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords,

//End for 38.214//

Correction for TCI state number

For single DCI based M-TRP transmission, the activation command in MAC CE can map up to 8 combinations of one or two TCI states to the maximal 8 codepoints of the DCI field 'Transmission Configuration Indication'. That means up to 16 TCI states may be configured via MAC CE (some of them can be the same) if two TCI states are mapped to each of the 8 codepoints, but only 8 different TCI states can be activated according to following agreement.

Agreement

The maximum number of activated TCI states in mTRP operation is 8. The number of bits of TCI field in DCI is 3 if higher layer parameter tci-PresentInDCI is enabled. The total number of simultaneously activated TCI states is up to 8.

Hence, the description in 38.214 needs to be modified to avoid misunderstanding that only 8 TCI states can be mapped to the codepoints of the TCI field

// TP2 for 38.214//

### 5.1.5 Antenna ports quasi co-location When a UE supports two TCI states in a codepoint of the DCI field ‘Transmission Configuration Indication’ the UE may receive an activation command, as described in subclause [6.1.3.X] of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 different TCI states in the activation command.

//End for 38.214//

Text proposals for multiple DCIs based M-TRP transmission

Scheduling restriction for multi-DCI based transmission

Both single DCI based and multi-DCI based M-TRP transmissions are supported in Rel-16. However, these two transmission mechanisms are discussed separately. Currently we only agree that a UE can be scheduled with single DCI or multiple DCI based M-TRP transmission. The mixing of the two modes, e.g. multiple PDCCHs from different CORESET groups, each scheduling PDSCHs from two TRPs via two TCI states, has never been discussed in Rel-16 and should be excluded in specification. The current description in section 5.1 of 38.214 allows this type of configuration and should be clarified. When multiple PDCCHs are scheduled, only one TCI state can be indicated in each of the DCIs. The details can be found in [4].

// TP3 for 38.214//

## 5.1 UE procedure for receiving the physical downlink shared channel If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain, where at most one TCI state can be indicated by the DCI field ‘Transmission Configuration Indication’ in each of the PDCCHs. The UE may expect the reception of full/partially-overlapped PDSCHs in time only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex. For a ControlResourceSet without CORESETPoolIndex, the UE may assume that the ControlResourceSet is assigned with CORESETPoolIndex as 0. When the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed: - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of CORESETpoolindex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

//End for 38.214//

Configuration of the same BWP and SCS

In section 5.1 of 38.214 [3], multi-DCI based M-TRP transmission is introduced via multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. According to the description, when the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the UE is expected to be scheduled with the same active BWP and the same SCS. However, it means that BWP switching is not allowed even when the PDSCHs are non-overlapped in time domain, no matter whether the scheduling PDCCHs are time overlapped or not, and whether the scheduling PDCCHs are associates with the same CORESETPoolIndex or not. Even BWP switching for single TRP transmission is forbidden by current specification. To relax the restriction and ensure the same BWP for two TRPs, it is proposed to only allow BWP switching via DCI from one TRP and DCI from the other TRP should always indicate the same BWP. We have the following TP and the details can be found in [3].

// TP for 38.214//

## 5.1 UE procedure for receiving the physical downlink shared channel If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of full/partially-overlapped PDSCHs in time only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex. For a ControlResourceSet without CORESETPoolIndex, the UE may assume that the ControlResourceSet is assigned with CORESETPoolIndex as 0. When the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH. For a PDCCH associated to ControlResourceSet having value of CORESETpoolIndex equal to 1, the UE doesn’t expect the bandwidth part indicator field in a DCI of the PDCCH indicates a different BWP from the active BWP. When the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed: - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of CORESETpoolindex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

//End for 38.214//

Overlapping between HARQ-ACKs and other UL signaling

According to current 38.213 [2], if separate HARQ-ACK feedback is configured, PUCCH/PUSCH scheduled by PDCCH associated with a CORESETPoolIndex should not overlap with PUCCH/PUSCH scheduled by PDCCH associated with the other CORESETPoolIndex. For other uplink signal which is not associated with a CORESETPoolIndex, gNB should ensure that uplink signals for different TRPs are TDMed. If two uplink signals overlap in time, UE would multiplex them together based on Rel-15 mechanism.

Furthermore, if a CSI is simultaneously overlapped with two PUCCHs within a slot with HARQ-ACK respectively for eMBB and URLLC, the CSI will be multiplexed with the HARQ-ACK with the same priority. However, if a PUCCH with P/SP CSI is simultaneously overlapped with two PUCCHs within a slot with separate HARQ-ACK respectively for two TRPs (which are associated with different values of CORESETPoolIndex), as shown in Figure 1, the UE behavior on how to multiplex the CSI with the two HARQ-ACKs is not clearly defined since no Rel-15 behavior can be followed. According to current specification, it seems that UE would jointly multiplex the two HARQ-ACKs for two TRPs with the CSI together (three UCIs in one PUCCH), which is not reasonable if ACKNACKFeedbackMode = SeparateFeedback. This issue also occurs for other UL signal which is not scheduled by DCI (e.g. PUCCH with SR or type 1 configured grant PUSCH).

Figure 1: Overlapping between two HARQ-ACKs and CSI

In our contribution [4], following two options are provided for this issue, and option 2 is preferred to avoid additional RRC signaling.

- Option 1: Define an association relationship between UL signal which is not scheduled by DCI and CORESETPoolindex. A UL signal will only be multiplexed with HARQ-ACK associated with the same CORESETPoolindex.

- Option 2: UE doesn’t expect two PUCCHs within a slot with separate HARQ-ACK associated with different values of CORESETPoolIndex to be simultaneously overlapped with a UL signal.

Based on option 2, we provide a TP for 38.213 below. It should be noticed that the same issue will also occur for PUSCH, and similar solution should also be applied to PUSCH associated with different CORESETPoolIndex.

// TP for 38.213//

### 9.2.5 UE procedure for reporting multiple UCI types A UE that - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs, and does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs and a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs within the same slot to simultaneously overlap with another uplink signal. If there is one or more aperiodic CSI reports multiplexed on PUSCHs in the group of overlapping PUCCHs and PUSCHs and if symbol is before symbol that is a next uplink symbol with CP starting after after the end of the last symbol of - the last symbol of aperiodic CSI-RS resource for channel measurements, and - the last symbol of aperiodic CSI-IM used for interference measurements, and - the last symbol of aperiodic NZP CSI-RS for interference measurements, when aperiodic CSI-RS is used for channel measurement for triggered CSI report the UE is not required to update the CSI report for the triggered CSI report . is defined in [6, TS 38.214] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs scheduling the PUSCHs, the smallest SCS configuration of aperiodic CSI-RSs associated with DCI formats provided by the PDCCHs triggering the aperiodic CSI reports, and the smallest SCS configuration of the overlapping PUCCHs and PUSCHs and for , for and for .

//End for 38.213//

Joint feedback for semi-static HARQ-ACK codebook

According to the following agreement, the HARQ-ACK bits associated with a CORESETPoolIndex will be concatenated first, and then HARQ-ACK bits for different TRPs will be concatenated. In current 38.213, it is captured by “the UE generates a Type-1 HARQ-ACK codebook for a set S0∪S1 of NcellsDL=NcellsDL,0+ NcellsDL,1 serving cells.” (Set S0 and set S1 are two sets of serving cell)  However, for a serving cell including both a first CORESET and second CORESET, it is included in both set S0 and set S1, and operation of S0∪S1 will lead to a set with less than  NcellsDL=NcellsDL,0+ NcellsDL,1 cells. To correct this issue, we provide a TP as below.

Agreement

In case higher layer index per CORESET is configured, for HARQ semi-static HARQ-ACK codebook among M-TRP,

- HARQ-ACK information bits are concatenated by the increasing order of

  - PDSCH reception occasion index at first

  - and then serving cell index

  - and TRP (i.e. higher layer index configured per CORESET (if configured))

- FFS: Whether and how to specify UE behaviour in case the higher layer index per CORESET is not configured.

// TP for 38.213//

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = JointFeedback where - a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and - a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for a set S0 and a set S1 separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 respectively following the pseudo-code of determining HARQ-ACK information bits below. Then the UE concatenates the HARQ-ACK codebook generated for the set S0 and the HARQ-ACK codebook generated for the set S1 to formulate a total number of HARQ-ACK information bits for NcellsDL,0+ NcellsDL,1 serving cells.S0∪S1NcellsDL=NcellsDL,0+ NcellsDL,1 For the set of slot timing values , the UE determines a set of occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. //omitted part// Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell Set - HARQ-ACK information bit index Set to the number of serving cells configured by higher layers for the UE or NcellsDL,0 or NcellsDL,1 while Set – index of occasion for candidate PDSCH reception or SPS PDSCH release while

//End for 38.213//

Text proposals for M-TRP based URLLC enhancement

Clarification on slots used for scheme 4

For description of scheme 4 in 38.214 [3], the following two aspects should be considered for clear UE behavior:

- It is common understanding of most companies that the slots used for scheme 4 should be consecutive. However, it is not captured in specification now.

- In section 5.1.2.1, Rel-15 slot aggregation based on pdsch-AggregationFactor and corresponding UE behavior is also described to support RRC based repetition. When pdsch-AggregationFactor and scheme 4 are both configured, the UE should only perform transmission of scheme 4 based on RepNumR16 in PDSCH-TimeDomainResourceAllocation, and pdsch-AggregationFactor should be ignored. This UE behavior should be clearly defined in section 5.1.2.1 to avoid ambiguous UE behavior.

// TP for 38.214//

#### 5.1.2.1	Resource allocation in time domain

When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation,

-	If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList  which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the UE may expect to receive N PDSCH transmission occasions of the same TB in consecutive slots where N is given by RepNumR16 in PDSCH-TimeDomainResourceAllocation. The same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Subclause 5.1.2.1.

When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable CycMapping or SeqMapping in RepTCIMapping.

-	When CycMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

-	When SeqMapping is enabled, first TCI state is applied to the first and second PDSCH transmissions, and the second TCI state is applied to the third and fourth PDSCH transmissions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter RVSeqOffset and n is counted only considering PDSCH transmission occasions associated with the second TCI state.

Table 5.1.2.1-3: Applied redundancy version for the second TCI state when RVSeqOffset is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion with second TCI state |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

-	If one TCI state is indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList  which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the UE may expect to receive N PDSCH transmission occasions of the same TB in consecutive slots where N is given by RepNumR16 in PDSCH-TimeDomainResourceAllocation. The same SLIV is applied for all PDSCH transmission occasions, the first PDSCH transmission occasion follows Subclause 5.1.2.1, the same TCI state is applied to all PDSCH transmission occasions. The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted considering PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Subclause 5.1.2.1.

//End for 38.214//

Conclusion

In this contribution, we provide some TPs for enhancement on single DCI based M-TRP transmission, multi-DCI based M-TRP transmission and URLLC related enhancement.

References

- 38.212-g00, January 2020.

- 38.213-g00, January 2020.

- 38.214-g00, January 2020.

- R1-2000461，Remaining issues on Multi-TRP and panel Transmission, OPPO, RAN1#100 meeting, February 2020.