3GPP TSG RAN WG1 #100-E		R1-2000482

Athens, Greece, February 24th – 28th, 2020

Source:	OPPO

Title:	Discussion on Out-of-Order HARQ

Agenda Item:	7.2.5.4

Document for:	Discussion and Decision

Introduction

In RAN1 #99, the following conclusion was made for enhancements to scheduling/HARQ[1]:

Conclusion:

- For Rel. 16 URLLC, no support of out-of-order/overlap PDSCH/HARQ and out-of-order/overlap PUSCH operation.

Conclusion:

In Rel. 16 URLLC:

- The UE is not expected to be scheduled with two DG-PUSCH overlap in the time domain on the same carrier.

“The UE is not expected to be scheduled with two PUCCHs carrying HARQ-ACK with different priorities associated with two DG-PDSCHs scheduled on the same carrier overlapping in the time domain” was discussed by email discussion [99-NR-13] with no consensus.

In this contribution, we provide our views on the support of two PUCCHs carrying HARQ-ACK with different priorities overlap in the time domain.

Discussion

In Rel-15, more than one PUSCHs can be scheduled in one slot, and the out-of-order PUSCH scheduling is defined as the starting of the PUSCH scheduled by a later DCI is not earlier than the end of the PUSCH scheduled by an earlier DCI. On the other hand, the out-of-order HARQ is defined as the slot corresponding to a later PDSCH for ACK/NACK transmission is not before the slot corresponding to an earlier PDSCH for ACK/NACK transmission.

One example of the overlapping PUCCHs for HARQ-ACK with different priorities agreed in Rel-16 URLLC scheduling/HARQ enhancement agenda is shown in Figure 1. Actually, the two overlapping PUCCHs still satisfy the Rel-15 limitation of out-of-order HARQ. Therefore, no UE functionality of out-of-order HARQ is introduced to support the collision of two PUCCHs carrying HARQ-ACK with different priorities. On the other hand, if the HARQ-ACK bits with different priorities must be scheduled with non-overlapped PUCCH resources, additional scheduling limitation is needed, then large latency can be expected.

Observation 1: The Rel-16 overlapping PUCCHs for HARQ-ACK with different priorities satisfy the Rel-15 limitation of out-of-order HARQ.

Observation 2: No UE functionality of out-of-order HARQ is introduced to support the collision of two PUCCHs carrying HARQ-ACK with different priorities in Rel-16.

Figure 1. An example of the overlapping PUCCHs for HARQ-ACK with different priorities

The necessity and benefits of supporting multiple PUCCHs within one slot and HARQ-ACK bits with different priorities are transmitted on separated PUCCHs have been fully discussed in scheduling/HARQ enhancement agenda, and the agreements made in scheduling/HARQ enhancement never depend on the supporting of out-of-order. Therefore, there is no reason to preclude the agreements made in scheduling/HARQ enhancement.

Proposal: There is no reason to preclude the agreements of the collision handling of two PUCCHs carrying HARQ-ACK with different priorities made in scheduling/HARQ enhancement agenda.

Conclusions

In this contribution, we show our views on the support of two overlapping PUCCHs carrying HARQ-ACK with different priorities in Rel-16 with the following observations and proposal:

Observation 1: The Rel-16 overlapping PUCCHs for HARQ-ACK with different priorities satisfy the Rel-15 limitation of out-of-order HARQ.

Observation 2: No UE functionality of out-of-order HARQ is introduced to support the collision of two PUCCHs carrying HARQ-ACK with different priorities in Rel-16.

Proposal: There is no reason to preclude the agreements of the collision handling of two PUCCHs carrying HARQ-ACK with different priorities made in scheduling/HARQ enhancement agenda.

Reference

- RAN1#99 chairman notes