3GPP TSG RAN WG1 #100		R1-2000489

e-Meeting, February 24th – March 6th, 2020

Source:	OPPO

Title:	Draft text proposals on physical layer structure for NR-V2X

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

Introduction

RAN1 agreements on 5G V2X physical layer structure have been introduced to 3GPP specifications [1~4].

In this contribution, we discuss remaining details on physical layer structure of NR-V2X and give text proposals.

Physical channels

  - Slot pool

In LTE V2X, subframe pool is (pre-)configured by a bitmap, and the bitmap for subframe pool configuration is applied to a logical subframe set periodically. And the logical subframe se is derived by precluding DL subframe, special subframes, SLSS subframes, and reserved subframes from subframes within a SFN/DFN range. The number of subframes within the logical subframe set is an integer multiple of the length of the bitmap for resource pool configuration, which is achieved by precluding a certain number of reserved subframes, such that periodic resource reservation could not be interrupted cross SFN/DFN boundary.

In NR V2X, in a resource pool where reservation of a sidelink resource for an initial transmission of a TB by an SCI associated with a different TB can be enabled or disabled. If enabled, UE is allowed to reserve resources periodically, which is same as LTE V2X, in this case, the mechanism of LTE V2X can be reused to determine slot pool. If disabled, UE can only reserve resources for re-transmissions of the same TB and periodic resource reservation is not allowed, in this case, there is no need to preclude reserved slots to ensure that the number of slot within logical slots sets are multiple integer of the bitmap, in this way resource efficiency can be improved.

Proposal 1:

- Slot pool for a resource pool where reservation of a sidelink resource for an initial transmission of a TB by an SCI associated with a different TB is enabled is determined by reusing LTE mechanism;

- Slot pool for a resource pool where reservation of a sidelink resource for an initial transmission of a TB by an SCI associated with a different TB is disabled is determined by reusing LTE mechanism with an exception that no reserved slot is precluded;

- (for 38.214):

=============================================================================

8 Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain, a sidelink resource pool consists of numSubchannel contiguous sub-channels. A sub-channel consists of subchannelsize contiguous PRBs, where numSubchannel and subchannelsize are higher layer parameters.

The set of slots that may belong to a sidelink resource pool is denoted by tSL=t0SL,t1SL,…, tTmaxSL  where

-	0≤tiSL<10240×2μ,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slot,

-	NS-SSB slots in which S-SSB resource is configured,

-	Nnon-SL slots in which a symbol with index i is configured as downlink symbol or flexible symbol, as defined in clause 11.1 [6,38.213], where i=startSLsymbols,…,startSLsymbols+lengthSLsymbols-1, startSLsymbols and lengthSLsymbols are provided by higher layer;

-  if higher parameter reserveResourceDifferentTB is configured, reserved slots which are determined by the following steps:

1)	the remaining slots excluding NS-SSB and Nnon-SL slots from the set of all the slots are denoted by l0,l1,…, l10240×2μ-NS-SSB-Nnon-SL  arranged in increasing order of slot index.

2)	a slot lr0≤r<10240×2μ-NS-SSB-Nnon-SL belongs to the reserved slots if r=m∙10240×2μ-NS-SSB-Nnon-SLNreserved where m=0,…,Nreserved-1 and Nreserved=10240×2μ-NS-SSB-Nnon-SL mod Lbitmap. Here, Lbitmap the length of the bitmap is configured by higher layers.

-	the slots are arranged in increasing order of slot index.

The UE determines the set of slot assigned to a sidelink resource pool as follows:

-	A bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used.

-	A slot tkSL 0≤k<10240×2μ-NS-SSB-Nnon-SL-Nreserved belongs to the slot pool if bk' mod Lbitmap=1 where k'=k mod Lbitmap .

============================================================================

  - TBS determination

In NR Uu, the number of REs within one PRB for PDSCH TBS determination is calculated by NRE'RE=NSCRB∙Nsymbolsh-NDMRSPRB-NohPRB, where NDMRSPRB is the number of REs for DM-RS including the overhead of the DM-RS CDM groups without data, NohPRB is configured by higher layer and used to count the overhead of CSI-RS and PT-RS, etc. For PUSCH, the same formula is used, with an exception that the parameter NohPRB is used to count the overhead of PT-RS and piggybacked UCI. The reason of using a higher layer configured value NohPRB rather than the actual overhead in the formula is to unify the number of REs for initial transmission and re-transmissions of a TB, as CSI-RS, PT-RS, and piggybacked UCI not occurring on all transmissions of a TB, but the TBS determined based on the number of REs for all transmissions of a TB have to be same.

As to PSSCH in NR sidelink, one TB can be transmitted for up to 32 times. For each transmission of a TB, within the symbols allocated for PSSCH transmission, there are REs for PSCCH, 2nd stage SCI and DM-RS, following the principle of NR Uu, these REs should be precluded directly. However, other overhead such as CSI-RS and PT-RS, may not occur in all transmissions of a TB, a common value (pre-)configured per resource pool can be used to cover these cost, such as to ensure that same TBS is determined for all transmissions of a TB. Additionally, as some PRBs of starting up to 3 PSSCH symbols are used for associated PSCCH, the number of symbols for PSSCH and reference signal overhead on PRBs overlapping with associated PSCCH are different from other PRBs, hence the total number of PSSCH REs should be calculated directly (rather than based on number of REs within one PRB as NR Uu). In view of above, we have following proposal and text proposal for section 8.1.3.2 of 38.214 on transport block size determination of PSSCH.

Proposal 2: PSSCH REs for TBS determination are calculated directly by precluding REs for PSCCH, 2nd stage SCI, DM-RS, and higher layer configured overhead.

- (for 38.214):

========================================================================

8.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

-	NRE=NsymbolPSSCH×NSCRB-NohPRB×nPRB-NPSCCH-NDMRS-NSCI0-2,whereNSCRB=12 is the number of subcarriers in the frequency domain in a physical resource block, NsymbolPSSCH Nsymbslot is the number of allocated symbols for the PSSCH except AGC symbol, nPRB is the number of PRBs allocated for the PSSCH, NPSCCH Nsymbslot is the number of REs for the associated PSCCH except REs on AGC symbol, NDMRS is the number of REs for DM-RS including the overhead of the DM-RS CDM groups as indicated by SCI format 0-1, and NohPRB is the overhead per PRB configured by higher layer parameter xOverhead in higherlayersignaling.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

========================================================================

  - Initialization of the scrambling sequence generators

In [99-NR-07] it was agreed that a value (pre-)configured per resource pool is used to initialize the sequence of PSCCH DMRS, this value should be used to initialize the scrambling sequence generator.

For 2nd stage SCI and data transmitted on PSSCH, the scrambling sequence generator should be initialized with decimal representation of CRC of the associated PSCCH.

Scrambling sequence generator for sidelink master information transmitted on PSBCH should be initialized with SLSS ID.

In general, we have following proposal and text proposals for 38.211 on initialization of the scrambling sequence generators.

Proposal 3: Scrambling sequence generators for PSCCH, 2nd stage SCI and data, PSBCH are initialized by pscch-DMRS-ScramblingID, decimal representation of CRC of the associated PSCCH, and SLSS ID respectively.

- (for 38.211):

========================================================================

8.3	Physical channels

8.3.1		Physical sidelink shared channel

8.3.1.1	Scrambling

For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation.

Scrambling shall be done according to the following pseudo code

set i=0

while i<Mbitq

if b(q)i=x	// SCI placeholder bits

b(q)i=b(q)i-2

else

b(q)i=b(q)i+cq(i-M(q)) mod 2

end if

i = i + 1

end while

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0

cinit

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	M(q)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit=i=0L-1pi∙2L-1-i

where  and  are defined in clause 5.2.1 in [4, TS 38.212] equal to the decimal representation of CRC of the associated PSCCH.

<Not related omitted>

8.3.2	Physical sidelink control channel

8.3.2.1	Scrambling

The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit

where cinit∈0,1,…,65535 is given by the higher-layer parameter pscch-DMRS-ScramblingID .

<Not related omitted>

8.3.3	Physical sidelink broadcast channel

8.3.3.1	Scrambling

The block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical sidelink broadcast channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits  b0, …, b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=NIDSL

========================================================================

  - Cyclic shift for PSFCH sequence

For a resource pool, there are NCSPSFCH cyclic shift pairs configured by higher layer, these cyclic shift pairs should be evenly distributed within [0, 11]. On the other hand, the separation between paired cyclic shifts should be 6 to minimize NACK to ACK error. Based on this principle, we have following text proposals for 38.213 on how to determine the First cyclic shift for NACK and Second cyclic shift for ACK.

Proposal 4: The cyclic shift pairs of PSFCH are evenly distributed within [0, 11], and the separation between paired cyclic shifts are 6.

- (for 38.213):

========================================================================

16.3	UE procedure for reporting HARQ-ACK on sidelink

<Not related omitted>

The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211]. The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs.

A UE determines an index of a PSFCH resource for a PSFCH transmission in response to a PSSCH reception, using a sequence associated with the resource pool [4, TS 38.211], as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 0-2 [5, TS 38.212] scheduling the PSSCH reception, MID is zero or MID is the identity of the UE receiving the PSSCH as indicated by higher layers.

A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair of a PSFCH resource as in Table 16.6-1.  The first cyclic shift is determined as PID+MIDmodRPRB, CSPSFCHNtype, slotPSFCH∙Msubch, slotPSFCH∙6NCSPSFCH, the Second cyclic shift is 6 larger than the First cyclic shift.

Table 16.6-1: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | First cyclic shift | Second cyclic shift |

========================================================================

  - 2nd stage SCI formats

In RAN1#98bis [5], the following working assumption was made, this working assumption should be confirmed otherwise whether HARQ feedback is used or not cannot be indicated.

Working assumption: - For HARQ feedback in groupcast and unicast, when PSFCH resource is (pre-)configured in the resource pool, - SCI explicitly indicates whether HARQ feedback is used or not for the corresponding PSSCH transmission.

For the following outstanding issue of RAN1#99 [6], in our view, Option 1 should be used, as HARQ feedback option 1 and HARQ feedback option 2 would not definitely lead to different 2nd stage SCI sizes.

Agreements: - 2nd stage SCI format for groupcast HARQ feedback option 1 and option 2. To down-select during the week: - Option 1: The same 2nd stage SCI format is used for groupcast HARQ feedback option 1 and option 2. - SCI indicator to indicate between groupcast Option 1 and groupcast Option 2 is in the 2nd-stage SCI. - Option 2: Different 2nd stage SCI formats are used in groupcast HARQ feedback option 1 and option 2. - 1st stage SCI indicates which format is used.

Hence, following two 2nd stage SCI formats should be defined for Rel-16, as also discussion in our companion contribution [7]:

- 2nd stage SCI format for type 1 groupcast, in this case the fields of “zone ID” and “communication range” are present, this also implies that HARQ feedback is enabled and groupcast HARQ feedback option 1 is used.

- 2nd stage SCI format for other groupcast types, unicast, and broadcast, in this case, the fields of “zone ID” and “communication range” are absent;

However, as in further release additional 2nd stage SCI formats may be newly defined, at least 2 bits in SCI 0-1 should be used to indicate 2nd stage SCI formats for forward compatibility.

In general, we have following proposal and text proposal for 38.212 on 2nd stage SCI formats and contents:

Proposal 5:

- Only two 2nd stage SCI formats are defined, in one format “Zone ID” and “Communication range requirement” fields are present, and in the other format these two fields are absent;

- 2 bits in SCI 0-1 are used to indicate 2nd stage SCI format;

- Whether HARQ feedback is used or not is explicitly indicated in 2nd stage SCI, and groupcast HARQ feedback option 1 or option 2 is indicated explicitly in 2nd stage SCI ;

- (for 38.212):

========================================================================

8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	Priority – 3 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise 9 bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) bits as defined in subclause x.x.x of [6, TS 38.214], if higher parameter reserveResourceDifferentTB is configured; 0 bit otherwise.

-	DMRS pattern – [x] bits as defined in subclause x.x.x of [6, TS 38.214], if more than one DMRS patterns are configured by higher layer parameter TimePatternPsschDmrs; 0 bit otherwise.

-	2nd-stage SCI format – 2bits as defined in subclause x.x.x of [6, TS 38.214].

-	Beta_offset indicator – [2] bits as defined in subclause x.x.x of [6, TS 38.214].

-	Number of DMRS port – 1 bit as defined in subclause x.x.x of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Reserved – [2 - 4] bits as determined by higher layer parameter [XXX], with value set to zero.

<Not related omitted>

8.4.1.1	SCI format 0-2

SCI format 0-2 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-2:

-	HARQ Process ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause x.x.x of [6, TS 38.214].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause x.x.x of [6, TS 38.214].

If the 2nd-stage SCI format field in the corresponding SCI format 0-1 indicates other than type 1 groupcast as defined in clause x.x.x of [6, TS 38.214], the following fields are present:

-	HARQ feedback enabled - 1 bit.

- 	Groupcast HARQ feedback option –1 bit as defined in clause x.x.x of [5, TS 38.213].

If the 2nd-stage SCI format field in the corresponding SCI format 0-1 indicates type 1 groupcast as defined in clause x.x.x of [6, TS 38.214], the following fields are present:

-	Zone ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	Communication range requirement – [4] bits as defined in clause x.x.x of [6, TS 38.214]

========================================================================

  - Frequency domain DMRS pattern

Following working assumption was made in RAN1#97[6], as NR sidelink only support up to 2 layers, there is no need to support configuration type 2.

Working assumption: - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS. - FFS whether to support either one or both types - FFS details on multiplexing of different ports for PSSCH DMRS

Proposal 6: Only PDSCH DMRS Configuration type 1 is used for PSSCH DMRS.

- (for 38.211):

========================================================================

8.4.1.1.2	Mapping to physical resources

The sequence rm shall be mapped to the intermediate quantity ak,l(pj,μ) according to

αk,lpj,μ=wfk'r2n+k'ak,l(p0,μ)

k=4n+2k'+∆

k'=0,1

l=l

n=0,1, …

j=0,…,v-1

where wfk' and Δ are given by Tables 8.4.1.2.2-2.

<Not related omitted>

Table 8.4.1.2.2-2: Parameters for PSSCH DM-RS

|  | CDM group λ | Δ |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1 | +1 |
| 1 | 0 | 0 | +1 | -1 |

========================================================================

  - Frequency domain OCC for PSCCH DMRS

In [99-NR-07] the following agreement was made. As in one PRB there are 3 DMRS REs, 3 frequency domain OCCs should be used.

- NR PDCCH DMRS sequence is the baseline for PSCCH DMRS sequence at least with the following modification. - n_ID is determined by a (pre-)configured value per resource pool - Frequency-domain OCC is applied, one of the [2 or 3 or 4] OCCs is randomly selected by the Tx UE. - Note: there is no (pre-)configuration on the number of OCCs.

Proposal 7: 3 OCCs are used for PSCCH DMRS.

- (for 38.211):

========================================================================

8.4.1.3.2	Mapping to physical resources

The sequence rlm shall be multiplied with the amplitude scaling factor βPSCCH in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with rl0 to resource elements k,lp,μ in a slot on antenna port p=2000 according to

ak,l(p,μ)=βDMRSPSCCHej2π∅3rl3n+k'
k=nNscRB+4k'+1
k'=0,1,2
n=0,1,…

where ∅ is selected randomly by UE among {0,1,2}.

where the following conditions are fulfilled

-	they are within the resource elements constituting the PSCCH

The reference point for k is subcarrier 0 in common resource block 0.

The quantity l is the OFDM symbol number within the slot.

==========================================================================

  - PSSCH DMRS pattern for 2/3/4 PSSCH OS

In RAN1#99 it was agreed that the minimum number of symbols that can be used for sidelink within a slot is 7, considering the overhead of AGC, Gap, and PSFCH, the minimum number of symbols for PSSCH within a slot are 2. However, for now, the DMRS pattern for PSSCH with less than 5 symbols (not including the first symbol for AGC) are not decided yet, which implies that in case of less than 10 symbols are configured for sidelink on a BWP, then PSFCH resources cannot be configured, and transmissions with HARQ enabled could not be performed on the BWP consequently. To guarantee the reliability of NR V2X and completeness of specification, we have following proposal and text proposal for 38.211.

Proposal 8: Time domain DMRS pattern for PSSCH with 2 to 4 symbols should be defined.

- (for 38.211):

========================================================================

8.4.1.1.2	Mapping to physical resources

<Not related omitted>

The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.2.2-1 where ld is the duration of the scheduled resources for transmission of PSSCH and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3.

Table 8.4.1.2.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 3 | 1,2 |  |  | 1,2 |  |  |
| 4 | 1,3 |  |  | 1,3 |  |  |
| 5 | 1,4 |  |  | 1,4 |  |  |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |

==========================================================================

Conclusion

In this contribution, we discussed remaining critical details on physical layer structures, our proposals and TPs are summarized as below:

Proposal 1:

- Slot pool for a resource pool where reservation of a sidelink resource for an initial transmission of a TB by an SCI associated with a different TB is enabled is determined by reusing LTE mechanism;

- Slot pool for a resource pool where reservation of a sidelink resource for an initial transmission of a TB by an SCI associated with a different TB is disabled is determined by reusing LTE mechanism with an exception that no reserved slot is precluded;

Proposal 2: PSSCH REs for TBS determination are calculated directly by precluding REs for PSCCH, 2nd stage SCI, DM-RS, and higher layer configured overhead.

Proposal 3: Scrambling sequence generators for PSCCH, 2nd stage SCI and data, PSBCH are initialized by pscch-DMRS-ScramblingID, decimal representation of CRC of the associated PSCCH, and SLSS ID respectively.

Proposal 4: The cyclic shift pairs of PSFCH are evenly distributed within [0, 11], and the separation between paired cyclic shifts are 6.

Proposal 5:

- Only two 2nd stage SCI formats are defined, in one format “Zone ID” and “Communication range requirement” fields are present, and in the other format these two fields are absent;

- 2 bits in SCI 0-1 are used to indicate 2nd stage SCI format;

- Whether HARQ feedback is used or not is explicitly indicated in 2nd stage SCI, and groupcast HARQ feedback option 1 or option 2 is indicated explicitly in 2nd stage SCI ;

Proposal 6: Only PDSCH DMRS Configuration type 1 is used for PSSCH DMRS.

Proposal 7: 3 OCCs are used for PSCCH DMRS.

Proposal 8: Time domain DMRS pattern for PSSCH with 2 to 4 symbols should be defined.

Text proposal for 38.211

========================================================================

8.3	Physical channels

8.3.1		Physical sidelink shared channel

8.3.1.1	Scrambling

For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation.

Scrambling shall be done according to the following pseudo code

set i=0

while i<Mbitq

if b(q)i=x	// SCI placeholder bits

b(q)i=b(q)i-2

else

b(q)i=b(q)i+cq(i-M(q)) mod 2

end if

i = i + 1

end while

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0

cinit

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	M(q)=Mbit,SCI2q

-	The scrambling sequence generator shall be initialized with

cinit=i=0L-1pi∙2L-1-i

where  and  are defined in clause 5.2.1 in [4, TS 38.212] equal to the decimal representation of CRC of the associated PSCCH.

<Not related omitted>

8.3.2	Physical sidelink control channel

8.3.2.1	Scrambling

The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit

where cinit∈0,1,…,65535 is given by the higher-layer parameter pscch-DMRS-ScramblingID .

<Not related omitted>

8.3.3	Physical sidelink broadcast channel

8.3.3.1	Scrambling

The block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical sidelink broadcast channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits  b0, …, b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=NIDSL

<Not related omitted>

8.4.1.1.2	Mapping to physical resources

The sequence rm shall be mapped to the intermediate quantity ak,l(pj,μ) according to

αk,lpj,μ=wfk'r2n+k'ak,l(p0,μ)

k=4n+2k'+∆

k'=0,1

l=l

n=0,1, …

j=0,…,v-1

where wfk' and Δ are given by Tables 8.4.1.2.2-2.

<Not related omitted>

The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.2.2-1 where ld is the duration of the scheduled resources for transmission of PSSCH and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3.

Table 8.4.1.2.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 3 | 1,2 |  |  | 1,2 |  |  |
| 4 | 1,3 |  |  | 1,3 |  |  |
| 5 | 1,4 |  |  | 1,4 |  |  |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |

Table 8.4.1.2.2-2: Parameters for PSSCH DM-RS

|  | CDM group λ | Δ |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1 | +1 |
| 1 | 0 | 0 | +1 | -1 |

<Not related omitted>

8.4.1.3.2	Mapping to physical resources

The sequence rlm shall be multiplied with the amplitude scaling factor βPSCCH in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with rl0 to resource elements k,lp,μ in a slot on antenna port p=2000 according to

ak,l(p,μ)=βDMRSPSCCHej2π∅3rl3n+k'
k=nNscRB+4k'+1
k'=0,1,2
n=0,1,…

where ∅ is selected randomly by UE among {0,1,2}.

where the following conditions are fulfilled

-	they are within the resource elements constituting the PSCCH

The reference point for k is subcarrier 0 in common resource block 0.

The quantity l is the OFDM symbol number within the slot.

========================================================================

Text Proposal for 38.212:

========================================================================

8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	Priority – 3 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise 9 bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) bits as defined in subclause x.x.x of [6, TS 38.214], if higher parameter reserveResourceDifferentTB is configured; 0 bit otherwise.

-	DMRS pattern – [x] bits as defined in subclause x.x.x of [6, TS 38.214], if more than one DMRS patterns are configured by higher layer parameter TimePatternPsschDmrs; 0 bit otherwise.

-	2nd-stage SCI format – 2 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Beta_offset indicator – [2] bits as defined in subclause x.x.x of [6, TS 38.214].

-	Number of DMRS port – 1 bit as defined in subclause x.x.x of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Reserved – [2 - 4] bits as determined by higher layer parameter [XXX], with value set to zero.

<Not related omitted>

8.4.1.1	SCI format 0-2

SCI format 0-2 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-2:

-	HARQ Process ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	New data indicator – 1 bit as defined in clause x.x.x of [6, TS 38.214].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause x.x.x of [6, TS 38.214].

If the 2nd-stage SCI format field in the corresponding SCI format 0-1 indicates other than type 1 groupcast as defined in clause x.x.x of [6, TS 38.214], the following fields are present:

-	HARQ feedback enabled - 1 bit.

- 	Groupcast HARQ feedback option –1 bit as defined in clause x.x.x of [5, TS 38.213].

If the 2nd-stage SCI format field in the corresponding SCI format 0-1 indicates type 1 groupcast as defined in clause x.x.x of [6, TS 38.214], the following fields are present:

-	Zone ID – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	Communication range requirement – [4] bits as defined in clause x.x.x of [6, TS 38.214]

========================================================================

Text Proposal for 38.213:

========================================================================

16.3	UE procedure for reporting HARQ-ACK on sidelink

<Not related omitted>

The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211]. The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs.

A UE determines an index of a PSFCH resource for a PSFCH transmission in response to a PSSCH reception, using a sequence associated with the resource pool [4, TS 38.211], as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 0-2 [5, TS 38.212] scheduling the PSSCH reception, MID is zero or MID is the identity of the UE receiving the PSSCH as indicated by higher layers.

A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair of a PSFCH resource as in Table 16.6-1.  The first cyclic shift is determined as PID+MIDmodRPRB, CSPSFCHNtype, slotPSFCH∙Msubch, slotPSFCH∙6NCSPSFCH, the Second cyclic shift is 6 larger than the First cyclic shift.

Table 16.6-1: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | First cyclic shift | Second cyclic shift |

========================================================================

Text Proposal for 38.214:

=============================================================================

8 Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain, a sidelink resource pool consists of numSubchannel contiguous sub-channels. A sub-channel consists of subchannelsize contiguous PRBs, where numSubchannel and subchannelsize are higher layer parameters.

The set of slots that may belong to a sidelink resource pool is denoted by tSL=t0SL,t1SL,…, tTmaxSL  where

-	0≤tiSL<10240×2μ,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slot,

-	NS-SSB slots in which S-SSB resource is configured,

-	Nnon-SL slots in which a symbol with index i is configured as downlink symbol or flexible symbol, as defined in clause 11.1 [6,38.213], where i=startSLsymbols,…,startSLsymbols+lengthSLsymbols-1, startSLsymbols and lengthSLsymbols are provided by higher layer;

-  if higher parameter reserveResourceDifferentTB is configured, reserved slots which are determined by the following steps:

1)	the remaining slots excluding NS-SSB and Nnon-SL slots from the set of all the slots are denoted by l0,l1,…, l10240×2μ-NS-SSB-Nnon-SL  arranged in increasing order of slot index.

2)	a slot lr0≤r<10240×2μ-NS-SSB-Nnon-SL belongs to the reserved slots if r=m∙10240×2μ-NS-SSB-Nnon-SLNreserved where m=0,…,Nreserved-1 and Nreserved=10240×2μ-NS-SSB-Nnon-SL mod Lbitmap. Here, Lbitmap the length of the bitmap is configured by higher layers.

-	the slots are arranged in increasing order of slot index.

The UE determines the set of slot assigned to a sidelink resource pool as follows:

-	A bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used.

-	A slot tkSL 0≤k<10240×2μ-NS-SSB-Nnon-SL-Nreserved belongs to the slot pool if bk' mod Lbitmap=1 where k'=k mod Lbitmap .

<Not related omitted>

8.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

-	NRE=NsymbolPSSCH×NSCRB-NohPRB×nPRB-NPSCCH-NDMRS-NSCI0-2,whereNSCRB=12 is the number of subcarriers in the frequency domain in a physical resource block, NsymbolPSSCH Nsymbslot is the number of allocated symbols for the PSSCH except AGC symbol, nPRB is the number of PRBs allocated for the PSSCH, NPSCCH Nsymbslot is the number of REs for the associated PSCCH except REs on AGC symbol, NDMRS is the number of REs for DM-RS including the overhead of the DM-RS CDM groups as indicated by SCI format 0-1, and NohPRB is the overhead per PRB configured by higher layer parameter xOverhead in higherlayersignaling.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

========================================================================

References

- 3GPP TS 38.211 V16.0.0 (2019-12), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation(Release 16)

- 3GPP TS 38.212 V16.0.0 (2019-12),  3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Multiplexing and channel coding (Release 16)

- 3GPP TS 38.213 V16.0.0 (2019-12),  3rd Generation Partnership Project; Technical Specification Group Radio Access Network;  NR; Physical layer procedures for control (Release 16)

- 3GPP TS 38.214 V16.0.0 (2019-12),  3rd Generation Partnership Project; Technical Specification Group Radio Access Network;  NR; Physical layer procedures for data (Release 16)

- 3GPP RAN1 #98bis, Chairman’s Notes, Oct. 2019.

- 3GPP RAN1 #99, Chairman’s Notes, Nov. 2019.

- R1-2000492, Remaining issues of physical layer procedure for NR-V2X, OPPO

- 3GPP RAN1 #97, Chairman’s Notes, May 2019.