3GPP TSG RAN WG1 meeting #100                                        R1-2000510

e-Meeting, February 24th – March 6th, 2020

Title:	[Draft] Reply LS on secondary DRX group

Response to:	R1-2000017/ R2-1916597

Release:	Rel-16

Work Item:	TEI16

Source:	ZTE [3GPP RAN WG1]

To:	3GPP RAN WG2

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-1916597 on secondary DRX group.

- RAN1 understands that the introduction of secondary DRX configuration for FR2 is to reduce UE power consumption, especially for FR2 when multiple cells are configured. However, in RAN1’s view, the secondary DRX group would impact the operations of PDCCH scrambled by PS-RNTI, i.e.,DCI format 2_6. The potential impacts are as follows.

  - Scenario 1:Two groups with different DRX cycles

- According to the working assumption in RAN1, the wake-up indication is only applied to the long DRX cycle. If the secondary DRX group is introduced, it is unclear whether UE needs to detect PDCCH for DCI format 2_6 when the next cycle for the first DRX group is short DRX cycle, while for the second DRX group, the next cycle is long DRX cycle and the  monitoring occasion (MO) of DCI format 2_6 is configured in the second DRX group, as it is shown in Figure 1.

- Figure 1 Two groups with different DRX cycles

  - Scenario 2: Two groups with misaligned Active Time

- According to RAN1 agreements, UE does not detect DCI format 2_6 during Active Time. UE’s behavior needs to be clarified in RAN1, when the second DRX group that is configured with MO of DCI format 2_6 is outside Active Time, while the first DRX group is in the state of Active Time, as it is shown in Figure 2.

- Furthermore, according to RAN1 agreements, if there is no valid monitoring occasions for DCI format 2_6 prior to the next DRX cycle, UE should start drx-onDurationTimer. It should be clarified in RAN1 whether UE should start the drx-onDurationTimer for the first DRX group or not, if UE detects DCI format 2_6 in Figure 2 and the DCI indicates not-wake-up.

- Figure 2 Two groups with misaligned Active Time

2. Actions:

To RAN WG2

RAN1 respectfully requests RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #100b		20 – 24 Apr 2019,  Busan, Korea

TSG RAN WG1 Meeting #101	       25 – 29 May 2020, Athens, Greece