3GPP TSG RAN WG1 Meeting #100-e	R1-2000565

e-Meeting, February 24th-March 6th, 2020

Agenda Item:	7.2.4.2.1

Source:	FUTUREWEI

Title:	Remaining details on mode-1 resource allocation

Document for:	Discussion and decision

# Introduction

At RAN1#99, RAN1 completed the standardization work for NR V2X. Some details were left for the CR phase, with a list of open issues identified at RAN#86 [1].

In [1], eight issues were identified for mode 1 resource allocation, and are shown in Table 1. In this contribution, we discuss each of these issues.

Table 1. Open issues for mode 1 resource allocation.

| Mode 1 Resource Allocation Mode 1 Resource Allocation | D1 | Details of SL DCI format size alignment with existing NR Uu DCI | No |
| --- | --- | --- | --- |
|  | D2 | Details on DCI contents for dynamic grant and Type-2 configured grant | No |
|  | D3 | Multiplexing multiple SL HARQ-ACKs in a PUCCH resource | Depends on the outcome of RAN1 discussion |
|  | D4 | Remaining details of UE behaviour in reporting SL HARQ-ACK | No |
|  | D5 | HARQ process ID determination for configured grants | Depends on the outcome of RAN1 discussion |
|  | D6 | Timing determination for configured grant type 1 in cross RAT scheduling and same RAT scheduling | No |

# Discussion

## Details of SL DCI format size alignment with existing NR Uu DCI

At RAN1#99, the following was agreed:

- Existing DCI size budget is maintained when the UE is configured with SL

- (working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

There are two DCIs that need to be configured for mode-1 resource allocation:

- DCI for dynamic allocation

- DCI for configured grant type-2

In line with the decisions made for LTE-V and Rel-15 NR, we suggest to have a single DCI format for these two cases.

The DCI size can be matched to either DCI format 0_0 or 0_1. Based on the DCI content already agreed, it appears that the DCI for mode-1 can be relatively large. Thus, DCI 3_0 size should be matched to the one of DCI 0_1. If needed, zero-padding bits can be added to match the two sizes.

Proposal 1:

- DCI format 3_0 is the same size as DCI format 0_1

- Zero-padding bits are added to DCI 3_0 to match the size of DCI 0_1, if needed

## Details of DCI contents

The following content of DCI format 3_0 is captured in TS38.212:

-	Time gap – [x] bits determined by higher layer parameter timeGapFirstSidelinkTransmission, as defined in subclause x.x.x of [6, TS 38.214]

-	HARQ process ID – [x] bits as defined in subclause x.x.x of [6, TS 38.214]

-	New data indicator – 1 bit as defined in subclause x.x.x of [6, TS 38.214]

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in subclause x.x.x of [6, TS 38.214]

-	SCI format 0_1 fields according to subclause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator – 3 bits as defined in subclause x.x.x of [6, TS 38.214].

-	PUCCH resource indicator – 3 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise [x] bits as defined in subclause x.x.x of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

The following field should be added:

- Power control bit: In LTE DCI format 5, there is one bit to indicate if the UE has to follow the gNB-to-UE open loop power control rule or not. The rationale is that if there is no cellular transmission, there is no need to protect the gNB from sidelink interference. Having this bit is advantageous for NR sidelink as well since the same applies as for LTE: if the gNB is not expecting any UL transmission, there is no need to constrain the power on the SL.

In addition, the number of bits needs to be decided for the time gap and the HARQ process ID. For the HARQ process ID, we propose 3 bits, as explained in our companion contribution [2]. For the time gap, there is no need for much resolution, and 2 bits is enough

Proposal 2: For DCI format 3_0:

- Time gap is encoded with 3 bits

- HARQ process ID is encoded with 3 bits

- The following field is added to DCI format 3_0: Power control bit

## Multiplexing of SL-HARQ in a PUCCH resource

One question that arises is if multiplexing of SL-HARQ is allowed in a PUCCH resource. The following was agreed during a follow-up email discussion after RAN1#98b:

- NR supports reporting of multiple SL HARQ-ACKs in a single PUCCH resource.

  - The Rel-15 procedures for multiplexing DL HARQ-ACKs are reutilized.

  - Reports carry SL HARQ-ACKs for dynamic grants and/or configured grants.

    - A UE does not expected to be indicated to transmit SL HARQ-ACK information for more than one SL configured grant in a same PUCCH.

    - Note: A UE can be provided with multiple SL CGs with different (non-overlapping) slots for the corresponding PUCCH transmissions for SL HARQ-ACK reporting.

This problem was already studied for Rel-15 NR and multiplexing rules were agreed. RAN1 already agreed to reuse the existing rules for multiplexing SL HARQ feedback, therefore we do not see the need to specify anything additional rule.

## Remaining details of UE behavior in reporting SL HARQ-ACK

There are cases where the HARQ feedback content is not clear for groupcast options 1 and 2. For option 1, if the UE does not receive any NAK on the PSFCH, it reports an ACK to the gNB. If the UE receives at least one NAK, it reports NAK. However, with the priority rules agreed at RAN1#98b, a UE can prioritize the PSFCH transmission over PSFCH reception. If the UE reporting HARQ feedback could not listen to a PSFCH because of its own transmission, the safest solution should be assumed, and the UE should report a NAK to the gNB. For mode 1, a similar solution can occur. In such a case, a NAK should be reported to the gNB as well.

Proposal 3:

- If a UE did not receive a PSFCH from other UEs, the UE automatically reports a NAK to the gNB

## HARQ process ID determination for configured grants

One small remaining issue the HARQ process ID determination for configured grants. Two options are possible:

- The HARQ process ID is configured by the gNB. This solution is simple but presents limited flexibility since the UE will always use the same ID for CGs.

- The HARQ process ID is linked to the resources of the grant. This solution is the one used for Rel-15 NR.

Both solutions can work. We have a slight preference for the second option since it is consistent with what was previously standardized.

Proposal 4: the HARQ process ID determination is based on the physical resources of the grant and reuses the Rel-15 process ID determination rules.

## Timing determination for configured grant type-1

For dynamic grant and CG type-2, an agreement was reached for timing:

- For dynamic grant and configured grant type-2, the slot of the first sidelink transmission is the in the first SL slot of the corresponding resource pool that starts not earlier than (working assumption for the formula) TDL-TTA2×TC+m×Tslot  where TDL is starting time of the slot carrying the corresponding DCI, TTA is the timing advance value and m is the slot offset (based on the SL numerology) between DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslot is the SL slot duration.

  - FFS the case of CG type 1

For dynamic grant and CG type-2, the working assumption should  be confirmed since there is no issue with the formula. For CG type-1, the timing needs to be figured out.

For UL CG 1, the timing is specified in Section 5.8.2 of TS 38.321:

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

For SL CG 1, the same formula can be used, thus no additional work is needed. While no decision needs to be taken by RAN1, RAN1 should inform RAN2 that the UL timing procedure is reused.

Proposal 5:

- Confirm the WA of RAN#99 for timing for dynamic grant and configured grant type-2

- For configured grant type-1, RAN1 assumes that the slot of the first sidelink transmission is determined using the same procedure as for UL, as defined in Section 5.8.2 of TS38.321

- Inform RAN2 of this assumption

# Conclusion

The remaining details for mode-1 resource allocation  were discussed. We propose the following:

Proposal 1:

- DCI format 3_0 is the same size as DCI format 0_1

- Zero-padding bits are added to DCI 3_0 to match the size of DCI 0_1, if needed

Proposal 2: For DCI format 3_0:

- Time gap is encoded with 3 bits

- HARQ process ID is encoded with 3 bits

- The following field is added to DCI format 3_0: Power control bit

Proposal 3:

- If a UE did not receive a PSFCH from other UEs, the UE automatically reports a NAK to the gNB

Proposal 4: the HARQ process ID determination is based on the physical resources of the grant and reuses the Rel-15 process ID determination rules.

Proposal 5:

- Confirm the WA of RAN#99 for timing for dynamic grant and configured grant type-2

- For configured grant type-1, RAN1 assumes that the slot of the first sidelink transmission is determined using the same procedure as for UL, as defined in Section 5.8.2 of TS38.321

- Inform RAN2 of this assumption

# References

[1] RP-193198, Task list for 5G V2X in RAN1#100, LGE, RAN#86, Sitges, Spain, Dec 9-12, 2019

[2] R1-2000564, Remaining details on mode-1 resource allocation, Futurewei, RAN1#100-e, Feb.24- Mar. 6th 2020