3GPP TSG RAN WG1 Meeting #100-e	R1-2000569

e-Meeting, February 24th-March 6th, 2020

Agenda Item:	7.2.4.4

Source:	FUTUREWEI

Title:	Remaining details on in-device coexistence between LTE and NR sidelinks

Document for:	Discussion and decision

# Introduction

In-device coexistence has progressed well, with the major decisions about how to handle coexistence already reached. In particular, at RAN1#99, the following was reached:

- When NR multiple transmissions (if supported) are overlapped with LTE SL TX/RX and if these NR multiple transmissions have different priorities (which are known in advance to the UE), the highest priority value of NR multiple transmissions is used for comparing that of LTE SL TX/RX and then SL operation with a higher relative priority is performed.

- In-device coexistence conflicts for network-controlled modes are addressed in the same way as for UE-autonomous modes

  - No addition spec is expected

In this contribution, we discuss the last remaining details of in-device coexistence. In particular, the case of the PSSCH carrying channel state information only has not been addressed yet.

# Discussion

We now examine the priority handling of the following cases: PSSCH carrying CSI and synchronization signals.

## PSSCH carrying only CSI

Per the WID, CSI information is carried in the PSSCH. CSI information is important information:

- It indicates if a resource sensed as empty is actually not appropriate since the receiver is experiencing high interference on that particular resource

- It is helpful to perform link adaptation and multi-antenna transmission

CSI information can increase reliability, reduce latency, and improve spectral efficiency. Thus, a PSSCH carrying only CSI should have a priority level in order to avoid or limit preemption by LTE traffic. If CSI is transmitted in a periodic manner, the UE can indicate and reserve resources for future transmissions of the PSSCH with only CSI, thus receiving UEs can know the priority of PSCCHs carrying only CSI in advance.

Proposal 1: A PSCCH carrying only CSI has a pre-configured priority level

## Synchronization signals

There are two issues with synchronization signals transmission:

- Issue 1: a synchronization signal transmission on one RAT may be scheduled at the same time as a data transmission on the other RAT

- Issue 2: a synchronization signal on one RAT may be scheduled at the same time as a synchronization signal on the other RAT

Issue 1 was addressed at RAN1#98b, with the assignment on a priority for synchronization signals/channels. Issue 2 still needs to be addressed.

For issue 2, we note that there are many cases where the synchronization on the LTE sidelink and NR sidelink would be the same. For instance, both sidelinks could be synchronized with GNSS, and would lose synchronization at the same time. Or, by following the priority rules, the NR sidelink might have been synchronized to an eNB, thus having the same timing as the LTE sidelink. In such a case, it is possible to have the synchronization signals configuration on the LTE and on the NR side in such a way that synchronization signals never collide.

Proposal 2:

- For intra-device coexistence, no correction is necessary for synchronization signals:

  - When the LTE and NR sidelinks use the same timing, the synchronization signal configuration is such that NR synchronization signals do not collide with LTE synchronization signals

# Conclusions

In this contribution, we discussed in-device coexistence. We propose the following:

Proposal 1: A PSCCH carrying only CSI has a pre-configured priority level

Proposal 2:

- For intra-device coexistence, no correction is necessary for synchronization signals:

  - When the LTE and NR sidelinks use the same timing, the synchronization signal configuration is such that NR synchronization signals do not collide with LTE synchronization signals

In order to support proposal 1, we have the following text proposal for TS38.214:

--------------------------------BEGIN TEXT PROPOSAL--------------------------------------

### 8.5.3	CSI reporting

CSI reporting is aperiodic and is described in [10, TS 38.321]. A PSSCH carrying CSI reporting has a pre-configured priority level.

--------------------------------END TEXT PROPOSAL------------------------------------------