3GPP TSG RAN WG1 Meeting #100-e	R1-200601

February 24th – March 6th, 2020.

# Title:	Reply to RAN2 LS on default codebook parameters

# Response to:	LS (R2-1916592) on default codebook parameters from RAN2

# Release:	Rel-16

# Work Item:	NR_eMIMO-Core

Source:	RAN1 (Samsung)

To:	RAN2

Cc:

Contact Person:

Name:

Tel. Number:

E-mail Address:

# Attachments:	-

1. Overall Description:

RAN1 would like to provide replies on the following questions from RAN2 in R2-1916592:

RAN2 requests RAN1 to provide the default values for the following parameters for type I single panel codebook (type1 singlePanel) which the UE is mandated to report: - supportedCSI-RS-ResourceList; - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set.

The description of the IE CodebookParameters in TS 38.331 [1] is copied below.

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

…

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

The rationale behind choosing the default values for the above parameters for Type I single panel codebook should be based on a configuration that is simple and has low UE complexity. Based on the rationale, the default values are shown as yellow highlighted above, and are tabulated in Table 1.

Table 1: default values

| Parameters |  | Value |
| --- | --- | --- |
| supportedCSI-RS-ResourceList | SEQUENCE SIZE OF SupportedCSI-RS-Resource | 1 |
|  | maxNumberTxPortsPerResource | p2 |
|  | maxNumberResourcesPerBand | 1 |
|  | totalNumberTxPortsPerBand | 2 |
| Modes |  | mode1 |
| maxNumberCSI-RS-PerResourceSet |  | 1 |

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG RAN1 Meeting #100	-Bis	20th – 24th April 2020		Busan, KR.

TSG RAN1 Meeting #101		25th – 29th May 2020		Athens, GR.

# References

- 3GPP RAN2, TS 38.331 V15.8.0