3GPP TSG RAN WG1 #100			R1-2000630

e-Meeting, February 24th – March 6th, 2020

Agenda Item:		7.2.5.6

Source:		Samsung

Title:		Remaining issues for configured grant

Document for:		Discussion and decision

Introduction

In last RAN1 meeting, Rel-16 eURLLC WID was finished to improve reliability and latency with a variety of new features such as new DCI format, sub-slot configuration, new PUSCH repetition, UL cancellation indication and so on. However, there are some remaining issues to be resolved to provide intended features without any ambiguities.

In this contribution, it will discuss an open issue for UL type 2 configured grant.

Discussion

In TS 38.213 v16.0.0, frequency domain resource assignment (FDRA) field of DCI format is used for releasing DL SPS or UL grant type 2 as a validation shown in following tables. DCI format 1_x is used for releasing DL SPS, while DCI format 0_x is used for releasing UL grant type 2.

< TS 38.213 v16.0.0> 10.2 PDCCH validation for DL SPS and UL grant Type 2 ------------------------------------------------------- Unchanged Texts Omitted---------------------------------------------------- Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. HARQ process number; set to all '0's/0_1/0_2; set to all '0's. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1. Table 10.2-4: Special fields for multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1.

Since DCI format 0_1 and DCI format 0_2 can also support FDRA type 0 (non-contiguous resource allocation) and 1 (contiguous resource allocation), it should be all ‘0’s for FDRA type 0 and all ‘1’ for FDRA type 1 that is the same way as DCI format 1_x be. This is because all ‘0’ values for FDRA type 0 is actually unused bit field for resource allocation.

Proposal 1: Endorse the following text proposal for 38.213

< TS 38.213 v16.0.0> 10.2 PDCCH validation for DL SPS and UL grant Type 2 ------------------------------------------------------- Unchanged Texts Omitted---------------------------------------------------- Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. HARQ process number; set to all '0's/0_1/0_2; set to all '0's. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1. Table 10.2-4: Special fields for multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1.

Conclusions

This contribution discussed about a remaining issue for configured grant. Following is the summary of this contribution.

Proposal 1: Endorse the following text proposal for 38.213

< TS 38.213 v16.0.0> 10.2 PDCCH validation for DL SPS and UL grant Type 2 ------------------------------------------------------- Unchanged Texts Omitted---------------------------------------------------- Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. HARQ process number; set to all '0's/0_1/0_2; set to all '0's. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1. Table 10.2-4: Special fields for multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation DCI format 0_0/0_1/0_2; DCI format 1_0/1_1/1_2. Redundancy version; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1; set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1.