3GPP TSG RAN WG1 #100	   	 	  	                 R1-2000678

e-Meeting, February 24th – March 6th, 2020

Agenda item: 7.2.6.1.

Source: LG Electronics

Title: Text proposals on CSI enhancement for MU-MIMO support

Document for: Discussion and Decision

- Introduction

In this contribution, we discuss the Rel-16 maintenance issue on enhanced Type II CSI codebook and suggest the corresponding text proposal.

- Text proposal on FD basis indication for N3

In RAN1#97 and RAN1#98, the following agreement were made in [1] and [2] as:

Agreement@RAN1#97

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

  - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

- For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2

  - FFS: If alt 3-4 is supported, size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2

- The following FD basis subset selection scheme is supported:

  - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

  - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where  is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Agreement@RAN1#98

On the remaining details on UCI parameters

- Fix α=2

- The range of values for is unrestricted, i.e.

Based on above agreements, FD basis subset selection for the value of N3 is described in TS 38.214 as follows. Especially in case of N3>19, it operates two-step window-based FD basis subset selection with Minitial and the FD basis indication is parameterized with  i1,6,l per layer.

<TS 38.214> ##### 5.2.2.2.5 Enhanced Type II Codebook Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where Minitial∈-2Mυ+1,-2Mυ+2,…,0 n3,l=n3,l(0),…,n3,l(Mυ-1) n3,lf∈0,1,…,N3-1 which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l l=1,…, υ, where i1,5∈0,1,…,2Mυ-1. i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2Mυ-1Mυ-1-1N3>19..

The elements of n3,l are found from i1,6,l using algorithm for FD basis subset selection adopting 𝐶(𝑥,𝑦), which is described in 5.2.2.2.5.

The problem in the current specification is that the upper-bound of range of 𝑥∗ is fixed to N3, thus it can lead the negative value of nl(f)whenN3>19. As shown above, on the calculation of i1,6,l, x value in Cx,y can be different according to N3 value, i.e. x=N3-1 for N3≤19 and x=2Mυ-1 for N3>19. To correct this issue, the range of 𝑥∗ will be adjusted depending on N3. Based on the above discussion, we suggest the following text proposal below.

Proposal: Adopt the following TPs to 5.2.2.2.5 of TS 38.214.

#### 5.2.2.2 Precoding matrix indicator (PMI) ##### 5.2.2.2.5 Enhanced Type II Codebook --------------- Unchanged parts omitted ------------- For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using Cx,y as defined in 5.2.2.2.3 and the algorithm: s0=0 if N3≤19 T = N3 else T = 2Mυ end if for f=1,…,Mυ-1 Find the largest x*∈Mυ-1-f,…,N3T-1-f in Table 5.2.2.2.5-4 such that i1,6,l -sf-1≥Cx*,Mυ-f ef=Cx*,Mυ-f sf=sf-1+ef if N3≤19 n3,l(f)=N3-1-x* else nl(f)=2Mυ-1-x* if nl(f)≤Minitial+2Mυ-1 n3,l(f)=nl(f) else n3,l(f)=nl(f)+(N3-2Mυ) end if end if --------------- Unchanged parts omitted -------------

References

[1] 3GPP RAN1, RAN1#97, Chairman Notes

[1] 3GPP RAN1, RAN1#98, Chairman Notes