3GPP TSG RAN WG1 #100-e	R1-2000765

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.4.2.1

Source: 	NEC

Title:	Remaining issues on resource allocation Mode 1

Document for:	Discussion/Decision

- Introduction

RAN plenary had discussion the task RAN1 needs to focus on in RAN1#100 for 5G V2X. RP-193198 captures the discussion outcome. For sidelink resource allocation mode 1, the remaining issues are:

| D1 | Details of SL DCI format size alignment with existing NR Uu DCI |
| --- | --- |
| D2 | Details on DCI contents for dynamic grant and Type-2 configured grant |
| D3 | Multiplexing multiple SL HARQ-ACKs in a PUCCH resource |
| D4 | Remaining details of UE behaviour in reporting SL HARQ-ACK |
| D5 | HARQ process ID determination for configured grants |
| D6 | Timing determination for configured grant type 1 in cross RAT scheduling and same RAT scheduling |

In this contribution, we share our views on D4: Remaining details of UE behavior in reporting SL HARQ-ACK.

- Discussion

For D4, in last RAN1#99 meeting, we have got following working assumption:

Working assumption: - The timing of the PUCCH used for conveying SL HARQ is indicated in DCI or RRC (only for transmissions without a DCI) in terms of PSFCH-to-PUCCH physical slots, where the slot duration is defined based on the PUCCH SCS. - Note: it is not intended to define any new sync requirements for gNBs

As shown in Fig.1, the interval between SL DCI and scheduled PSCCH/PSSCH (t0), and the interval between PSCCH/PSSCH and associated PSFCH (t1) are fixed or (pre-) configured. It will be more efficiency if the timing of PUCCH for SL HARQ feedback is indicated in terms of the interval with respect to slot of PSFCH for the SL transmission, i.e. t2 shown in Figure 1. So, we propose to confirm this working assumption.

Fig. 1 Timeline for sidelink scheduling and HARQ ACK/NACK feedback from transmitter UE

Proposal 1: Confirm the following working assumption:

- The timing of the PUCCH used for conveying SL HARQ is indicated in DCI or RRC (only for transmissions without a DCI) in terms of PSFCH-to-PUCCH physical slots, where the slot duration is defined based on the PUCCH SCS.

Note: it is not intended to define any new sync requirements for gNBs

- Conclusion

In this contribution, we discussed a remaining issues on physical layer resource allocation mode 1 and have following proposal:

Proposal 1: Confirm the following working assumption:

- The timing of the PUCCH used for conveying SL HARQ is indicated in DCI or RRC (only for transmissions without a DCI) in terms of PSFCH-to-PUCCH physical slots, where the slot duration is defined based on the PUCCH SCS.

Note: it is not intended to define any new sync requirements for gNBs

- Reference

[1] RP-193198, Task list for 5G V2X in RAN1#100, 3GPP TSG RAN #86, December 9th – 12th, 2019

[2] RAN1 Chairman’s Notes, 3GPP TSG RAN 1#99, November 18th – 22nd, 2019